NEC Pinwriter printer driver

version 1.0, 4/27/90

for Autodesk Device Interface (ADI) version 4.0

INTRODUCTION

This document explains the installation and use of PINADI.COM

version 1.0, an Autodesk Device Interface (ADI) driver designed

to run with NEC Pinwriter printers in conjunction with AutoCAD

and AutoSketch on DOS systems.

LIMITED WARRANTY DISCLAIMER

NECTECH does not warrant that the functions contained in the

Licensed Program will meet your requirements or that the

operation of the licensed Program will be uninterupted or error

free.  THE LICENSED PROGRAM IS PROVIDED TO YOU "AS IS" WITHOUT

WARRANTEE OF ANY KIND, EITHER EXPRESSED OR IMPLIED WARRANTIES OF

MERCHANTABILITY AND FITNESS FOR A PARTICULAR PURPOSE.  THE ENTIRE

RISK AS TO THE QUALITY AND PERFORMANCE OF THE LICENSED PROGRAM

PROVE DEFECTIVE YOU ASSUME THE ENTIRE COST OF ALL NECESSARY

SERVICING, REPAIR OR CORRECTION.  Some states do not allow the

exclusion of implied warranties, so the above exclusion may not

apply to you.

LIMITATION OF REMEDIES

IN NO EVENT WILL NECTECH BE LIABLE TO YOU FOR ANY DAMAGES,

INCLUDING ANY LOST PROFITS, LOST SAVINGS, INDIRECT, SPECIAL, LOST

DATA, INCIDENTAL OR CONSEQUENTIAL DAMAGES, OR OF ANY CLAIM BY ANY

OTHER PARTY.  Some states do not allow the limitation or

exclusion of liability for incidental or consequential damages so

the above limitation or exclusion may not apply to you.

HARDWARE REQUIREMENTS

The driver is suitable for use with 24-pin NEC Pinwriter

printers.  Some printer models can't use all of the options

provided by the driver.  For example, a narrow carriage printer

can't handle wide plots and some models can't print in color.

Please refer to your printer user's manual for details.

A parallel card and cable is required.  Serial output is not

supported because the time required to plot would be unacceptably

long.

COPYING THE DRIVER SOFTWARE

The PINADI printer ADI (ver 1.0) driver for AutoCAD and

AutoSketch is contained in the file PINADI.COM which is 6k bytes

long.  You will probably want to install the driver in the same

directory where you store AutoCAD or AutoSketch.  Here's how to

copy the driver software onto your hard disk.  (The example

assumes an AutoCAD directory is on drive C and is named ACAD).

1. Change to the directory where you store AutoCAD:

C:> CD ACAD [Return]

2. Insert the diskette containing the file PINADI.COM in drive A.

3. Copy the driver files to your hard disk

C:> COPY A:PINADI.COM  [Return]

INSTALLING THE DRIVER

The printer ADI driver must be installed (loaded into memory)

prior to entering AutoCAD.  It is a "terminate and stay resident"

(TSR) software program which can use up to 64K of RAM depending

on how it is configured.  Once loaded, it remains in memory until

you turn off or reboot your computer.

To install the driver:

C:> PINADI [Return]

The driver will then be installed with the following defaults:

monochrome (black and white) printing

form feed after every plot

input from AutoCAD or AutoSketch (rather than from a file)

output directly to the printer

printer connected to parallel port 1 (LPT1)

180 x 180 dots per inch print density

messages displayed on monitor (AutoSketch is off)

interrupt 7B(hex)

8" paper width

To change this setup, you can choose to include one or more

option codes on the command line.  If you use more than one code,

separate each by a space. The options and codes are:

code 	option

---- 	-----

C  		Color (default is monochrome output)

F 		disable Formfeed after plot

H or ? 	        display this list of options on monitor

Ifilename	Input from a file where "filename" is the path and name

of the file

Ofilename	Output to a file where "filename" is the path and name

of the file

Px  		Parallel port x where x = 2 or 3 (default is 1, LPT1)

Rx    	        Resolution specified by x from the following

table (default is x=4):

x:   1    2    3    4    5    6

horizontal dots per inch:   60   90  120  180  360  360

vertical dots per inch:  180  180  180  180  180  360

S		disable display messages for AutoSketch

U  		Uninstall driver from computer's memory

Vxx 		use interrupt Vector number xx hex (default is 7B)

W  		Wide paper, 13.6-inch print width (default is 8 inches)

For example, to install the driver to do color printer plots (if

your PINWRITER is color capable and you have the color ribbon

installed):

C:> PINADI C  [Return]

To install the driver to NOT send a form feed after each plot:

C:> PINADI F [Return]

(default is a form feed after every plot)

To display a list of these options:

C:> PINADI H or ? [Return]

You can also install the driver to write plot files directly to

your disk drive for later printing (this will create an output

file named 'filename' on your default drive for later printing).

This is useful for printing multiple copies of a plot.  Using

this option as a TSR with AutoCAD, you will be able to change the

output file name within AutoCAD before creating each plot.  The

ouput file created can be directly sent to your printer using the

DOS copy or print commands.  See the section titled "OUTPUTTING

FILES FOR DIRECT PRINTING FROM DISK" for more details:

C:> PINADI Ofilename [Return]

The driver can use an AutoCAD binary printer plot file

as input directly from your disk drive.  This option does not

install the driver as a TSR.  See the section titled "USE AS A

POSTPROCESSOR" for more details:

C:> PINADI Ifilename [Return]

Note:  These filenames must conform to DOS standards.

To install the driver to use a different parallel port:

C:> PINADI Px [Return]

(here x is the new port assignment 1, 2 or 3;

default is x=1 for LPT1)

To set the printer resolution to 360 horzontal dots per inch by

180 vertical dots per inch:

C:> PINADI R5 [Return]

To install the driver to work with AutoSketch (if you use

AutoSketch and don't use this option, messages will be written on

the screen over your drawing):

C:> PINADI S [Return]

(default writes messages to the screen)

To remove the installed TSR driver from your computer's memory:

C:> PINADI U

This uninstall option will overide other options.  To avoid

unpredictable results, remove this and other TSR programs in the

strict reverse order that they were installed.

To install the driver for a different interrupt vector (default

is 7B hex):

C:> PINADI Vxx [Return]

(where xx is the new hexadecimal interrupt number)

To install the driver for printing on wide paper (this will

create a plot up to 13.6" wide):

C:> PINADI W [Return]

(default is 8")

To install the driver using a combination of the above options:

C:> PINADI Oplotdata.001 c v78 w

This will do the following:

1. 	Create a file 'plotdata.001' of your drawing on your

default drive for plotting at a later time.

2.	Make a color print of the plot.

3. 	The interrupt vector will be 78 (in hex).

4. 	Allow wide plot up to 13.6 inches.

MEMORY REQUIREMENTS FOR DIFFERENT CONFIGURATIONS

RAM Memory required by installed driver

resolution			Black & White		Color

-------------------		-------------		------------

x	X  by  Y dpi		narrow	wide		narrow	wide

-	------------		------	----		------	----

1	60 x 180	 	4k	5k		10k	12k

2	90 x 180		5k	6k		12k	15K

3 	120 x 180		6k	8k		13k	17k

4	180 x 180        	7k	10k		16k	22k

5	360 x 180		11k	17k		24k	36k

6 	360 x 360 	        20k	32k		42k	64k

CONFIGURING AUTOCAD FOR THE PINWRITER DRIVER

It is necessary to reconfigure AutoCAD for an installed ADI

printer plotter driver.  You will only need to do this before you

want to use the ADI driver for the first time, or when you change

the paper width or the horizontal or the vertical plot

resolutions of your PINADI driver.  Note the configuration of

PINADI when you install it.  AutoCAD's printer plotter

configuration densities must be equal to the PINADI resolution

for your plots to be correct.

To configure AutoCAD, select the configuration option at

AutoCAD's main menu.  Next, select the printer-plotter

configuration option.  Then, select the ADI printer-plotter.  You

may have to tell AutoCAD where to find the generic ADI printer-

plotter driver file that comes with AutoCAD. It is in a AutoCAD

driver file called PPGEN.DRV.  Next, answer the questions to

configure an ADI printer-plotter. Be sure that you:

1.	  Select output format as an Installed ADI driver <2>.

2.	  Set the maximum horizontal (X) plot size in inches.

These values are 7.99" for the standard size or 13.59"

for the wide carriage printers.  The vertical (Y) axis

length should normally be set to your paper size.

However, the length can be set longer to allow printing

past the page break.

3.	  Set the density equal to the installed resolution

(dots/inch) of the PINADI driver.  If AutoCAD's density

(dots/inch) is greater, your drawing will be expanded

along that axis and may be clipped.  If the driver's

resolution is greater, your drawing will be compressed

along that axis.

4.	  The vertical or y-axis has only 2 densities, 180 dots

per inch and 360 dpi. For most printer plotting, 180

dpi should be fine. The 360 dpi vertical resolution is

achieved by causing the printer to make two passes of

its printhead.

For further information, see: Chapter 8, the printer

configuration section of the Installation & Performance Guide to

AutoCAD; Chapter 13 of your main Reference Manual, for AutoCAD

details; and your PINWRITER Users guide for printer details.

AutoCAD will now be configured to do a printer plot using the

installed PINADI driver.

CREATING PLOTS WITH THE PINWRITER PRINTER DRIVER

To send plots to the PINWRITER from AutoCAD's drawing editor type

the AutoCAD command 'prplot' at the command line.  Alternately,

from AutoCAD's Main Menu, select the Printer-Plot option <4>.

Make sure your printer is connected, turned on and on line, and

hit return when ready.  Answer the questions and confirm your

choices.  Then your plot will be produced on printer.

AutoCAD will repeatedly loop through the vectors that make up the

drawing.  Note that printer plots can take a long time to

produce, so don't worry if nothing happens for a while.  They can

take a very long time depending on the complexity of the drawing,

especially if you chose color or if the plot density is high.

To abort the plot, type CTRL-C.  If you get an "Abort,Retry ?"

message, check that your printer in on-line, and then enter R for

retry.

OUTPUTTING PRINTER FILES FOR DIRECT PRINTING FROM DISK

You may wish to create a file that may be printed multiple times

directly from disk.  Once a print file is created, you can then

print using the DOS copy command without running AutoCAD.

CAUTION: This option should be used with care.  It is possible to

create very large files, larger than 8 Megabytes, if all the

settings are at their maximum.  If it is used with some thought

it will produce good results.

Here is an example:

Say you want to produce a 360 x 180 dpi color plot of the

columbia.dwg on 13.59" wide paper.

Install the PINADI printer driver by typing:

C:> PINADI R5 Ocolumbia.nec C W [Return]

Configure AutoCAD as a ADI printer at 360 x 180 dpi, and a

horizontal width of 13.59".  Run AutoCAD and printer plot the

columbia.dwg file.  The file columbia.nec will be created on the

default disk for later printing from the command line.

You can do this by typing the line:

C:> COPY COLUMBIA.NEC PRN /B [Return]

In the above example, if you plotted the <E>xtents it would

create a file of 2.5 Megabytes and would take over an hour to be

produced on a IBM system 30.  But if you created a file of the

<D>isplay at 180 x 180 dpi for 7.99" wide paper by typing the

following:

C:> PINADI C Ocolumbia.nec

the file columbia.nec would be only .25 Megabytes.  The creation

time for this file is only 5 minutes on an AT clone running at 10

Mhz.

USE AS A POSTPROCESSOR

The ADI driver PINADI can also work as a postprocessor, reading

AutoCAD ADI binary files from your disk and sending them directly

to the printer.  Printer plot to an ADI binary printer file from

AutoCAD or AutoSketch 2.0.  This is done by configuring AutoCAD

or AutoSketch for binary-file output rather than for the

installed interrupt driver.  Then, when the Autodesk product is

no longer running, you may use PINADI to plot from the binary

file to your Pinwriter printer.

Here's an example:

You want to produce a 360x360 dpi color plot of columbia.dwg on

13.59" wide paper.  First you run AutoCAD's configuration program

to select the ADI Printer, plot to binary file, 360x360 dots per

inch, and 13.6" horizontal paper width.  Use the PRPLOT command

in AutoCAD.  This produces a binary printer plot file called

"columbia.prp".  Next, exit AutoCAD and plot the drawing by

invoking:

PINADI Icolumbia.prp C R6 W

This will cause PINADI to make a plot of the contents of the file

"columbia.prp" in color at 360 x 360 dpi on wide paper.  When the

plot is done, PINADI will vanish from memory (it does not install

itself as it would if you wanted to plot directly from AutoCAD).

If you added Ocolumbia.nec to the above command line, the data

wouldn't be sent to the printer.  A file named columbia.nec would

be created.  See the above section above titled "OUTPUTTING

PRINTER FILES FOR DIRECT PRINTING FROM DISK" for details.

AutoSketch 2.0 supports this feature.  Earlier versions did not

create binary plot files.

USE WITH AUTOSKETCH

The PINADI driver may be used with AutoSketch.  Just remember to

use the S code on the control line.  Failure to do this will

cause driver messages to printed on the AutoSketch screen,

overwriting the drawing displayed there.

Before you install this driver for AutoSketch, make sure that

your printer is turned on and is on-line.  If it is not, the

driver might not be installed and an error message will appear

telling you that it wasn't installed.

Before every plot from AutoSketch using this ADI driver, you must

make sure that the printer is turned on, connected and on line.

Failure to do this will return you to the drawing screen with out

any error messages.  If the printer goes off-line, runs out of

paper or becomes disconnected at any time during the plot, no

more data will be sent to the printer and you will be returned to

the drawing screen again without any error messages.  Therefore

if your drawing is not being plotted at all or stops plotting and

you are returned to the drawing editor, make sure the printer has

paper, is turned on, connected and is on line.

AutoSketch must be reconfigured so that it knows you will be

using an ADI driver to do printer plots.  To initiate

AutoSketch's reconfiguration type:

C:>SKETCH /R [Return]

Configure AutoSketch's plotter as a Autodesk Device Interface

Printer <3>. AutoSketch will then prompt you to give it more

information regarding this ADI driver.  See the previous section

titled 'CONFIGURING AUTOCAD FOR THE PINWRITER PRINTER DRIVER' and

the ADI Plotter section in Appendix B in your AutoSketch Users

Guide.

AutoSketch prompts you to indicate how your plotter is connected

to your computer, choose:  2. Parallel port, system printer, or

hardware handshake serial port.

Then follow the instructions in Configuring AutoSketch in Chapter

2 of the AutoSketch Users Guide.  Remember this ADI driver can

only be used as a parallel device.

Note that when you open the plot area dialogue box, the values

for x and y may be incorrect.  Set them now, to the size of the

paper you have in your printer.  Rotating the plot may also be

helpful.

For color printing, be sure to set PEN INFO for each drawing.

Otherwise all of the colors will be printed as color 1 which is

red.

COLOR ASSIGNMENTS

For color printing, the PINWRITER uses a four-color ribbon.  The

PINWRITER driver will print in order the ribbon colors yellow

(Y), magenta (M), cyan (C), and black (B).  This will produce the

following AutoCAD colors:

Number	AutoCAD color	Printed color	Ribbon color(s)

------	-------------	-------------	---------------

0			no print

1	red		orange		Y M

2	yellow		yellow		Y

3	green		green		Y   C

4	cyan		cyan		    C

5	blue		violet		C M

6	magenta		magenta		  M

7	white		black		      B

8			brown		  M   B

9			black		Y M C

10			dark violet	  M C B

11			yellow black	Y     B

12			olive		Y   C B

13			light brown	Y M   B

14			grey		C     B

15			deep black	Y M C B

Color numbers 16 through 255 cycle through the same colors as 0

through 15.

TIPS AND HINTS

Here are some tips for using the PINADI driver:

1. If you will use the driver with every session of AutoCAD,

you may want to place the PINADI command in your AUTOEXEC.BAT

file with the correct options set, because you must install the

Printer driver after every reset of your system.  Or put the

PINADI command in a batch file which you use to invoke AutoCAD or

AutoSketch.  Also put the command PINADI U at the end of this

batch file to remove the driver from memory when it is no longer

needed.

2. The Printer must be connected, turned on, and on line

before you plot a file using the driver.

3. Do not turn the Printer on and off in the middle of a

plotting session.  An error message will be displayed if you try

to use the driver while the Printer is turned off.

4. If you are using the tractor feed, make sure you've

advanced the paper far enough to have the paper bail contact the

platen.  Be sure the top cover is in place and all rollers are

guiding the paper.  If not, the paper may not advance uniformly,

creating discontinuities in the plot.

5. In color mode, for optimum color clarity and ribbon life,

avoid using resolution 360x360 and colors 9 through 15.  These

tend to cause contamination of the lighter ribbon bands by darker

ink that is already on the paper.  This can cause later printing

of light colors to be marked by spots of darker color.

6. If you are using this ADI driver with AutoSketch in a

color mode make sure that you configure EACH AutoSketch drawing's

Pen Info for the different colors.  If you don't, your entire

drawing will plot in the default color red.

7. You will find that high density and color plots take much

longer to complete than low density and monochrome plots.

9. Note that the different density options use different

amounts of memory in your computer.  High density plots need a

lot of memory.  They also take a long time to complete.  Color

adds greatly to the memory and time requirements.  Memory used up

by drivers is subtracted from the memory your drawings need when

you run AutoSketch.  AutoCAD doesn't have that problem, but there

still is limited memory available for use by drivers.  The PINADI

driver uses special data compression techniques to minimize

memory use. In color mode, if memory requirments get to great,

the driver conserves memory by automatically printing narrower

bands for each pass of the print head.

10. It is good practice to save your work before begining a

printer plot.  Sometime when aborting a plot, especially if the

printer has not been set on-line properly, DOS will kick you out

of AutoCAD and back to the DOS command line.  If this happens,

unsaved work may be lost.

AutoCAD, AutoSketch and ADI are registered trademarks of

Autodesk, Inc.