MR2800-W Modem

Quick Start User's Guide

Contents

1	Introduction

2	Preparing The Computer For MR2800-W Installation

3	Installing The Modem Hardware

4	Installing The Modem Software on Windows 9x and Windows 2000

5	Installing The Modem Software on Windows NT 4.0

6	Verifying Correct Modem Installation

7	Troubleshooting

8	Un-installing and Upgrading the MR2800-W Modem

9	Changing The Operating System

10 	IDE Device Compatibility

11	Contact Information

Appendix A	MR2800-W Main Features

1	Introduction

MR2800-W modem can be used on Intel reference motherboard platforms using Intel's core logic chipsets supporting AC-Link 2.1. MR2800-W modem is a feature-rich, V.90 modem at an attractive price. It supports all fax and data fall back modes, and different driver installs include telephone answering machine(TAM) operation and full speakerphone functionality. This document details installation, troubleshooting, and usage of the MR2800-W software modem.

The MR2800-W modem provides high-speed communications between your personal computer and a remote location, such as an Internet Service Provider (ISP), so you can:

- Receive data at up to 56 Kbps in V.90 or K56Flex modes

- Get automatic fallback to V.34 (33.6 Kbps) rates under bad line conditions and on non-V.90/K56Flex headends

- Send faxes from your personal computer at rates up to 14.4 Kbps

- Use your PC as a telephone answering machine (TAM)

The MR2800-W modem runs on Windows 95/98, in a Windows 95/98 DOS box, Windows NT 4.0, and Windows 2000.

Note: The modem cannot be used in pure DOS or on any other operating system. Note that Windows 9x, NT 4.0, and 2000 require different modem drivers. The driver for one OS will    not work on the other OS.Make sure you have the correct drivers before installing

Computer OEMs that bundle the MR2800-W modem can benefit from sizable cost reductions, hardware reduction, and lower power consumption. End users benefit from quick, easy and affordable software upgrades, which help them keep current with the latest communications technology.

Important: There are three different SM56 modem types, and each supports a different feature-set, as follows:

Product Name					Modem Type			Main Feature-set Supported

MR2800-W Modem				Data/Fax only			All data and fax modes

No telephone answering machine (TAM)

No speakerphone

No caller ID

No distinctive ring

______________________________________________________________________________________

You can check which modem you have by selecting Start->Settings->Control Panel, then double click on the Modems icon. The modem name will appear in the Modem Properties window.

2	Preparing the Computer For MR2800-W Installation

To ensure problem-free installation of the MR2800-W modem, ensure that a COM port is available for it to install on. The MR2800-W install program interrogates lower COM ports (below COM 5) and installs the modem on the first    available and unused port. Although the MR2800-W functions perfectly on any available COM port, some communications applications may not be able to use the modem if it gets installed above COM 4. Refer to the Troubleshooting section in this document for information on how to force the modem onto a COM port    below COM 5 after install is complete.

You can check which COM ports are on your computer as follows:

1. Open the Windows 95/98 Control Panel.

2. Double click the System icon.

3. Choose the Device Manager tab.

4. Expand the Ports (COM and LPT) branch to see which COM ports are already installed.

3	Installing The Modem Hardware

1. Power down the computer.

2. Locate the AC-Link connector on the motherboard, and carefully insert the MR2800-W modem board.

3. Connect the Wall jack on the modem board to an analog phone jack using an RJ-11 phone cable. If your modem board also has a telephone handset jack, make sure to not plug the telephone line into that jack. The jacks will be labeled with a little icon and/or text indicatin which is which.

4. Power ON the computer.

Important: When using the MR2800-W modem, or any modem for that matter, you should always connect the modem line input to a direct analog telephone line. This will maximize modem performance by avoiding any problems introduced by internal PBX systems, etc.

4	Installing Modem Software on Windows 9x and Windows 2000

(Note: The Windows Install Wizard may vary slightly with different versions of the OS and different service packs)

On starting Windows for the first time after installing the MR2800-W modem, the Plug and Play (PnP) sub-system detects the new hardware and begins an install wizard for the modem. The MR2800-W modem is detected as a new PCI Device. The wizard fist looks for the modem information (INF) file, which is included on Motorola distribution disk or MR2800-W CD-ROM. This file contains information about the modem, including device type, device driver information, as well as the AT command/response sets that it supports.

Select Driver from Disk Provided by Hardware Manufacturer (or Have Disk depending on the OS version) and browse to the MR2800-W modem INF file (called SM56ACL.INF) on the MR2800-W disk/CD-ROM. Windows will find the information on the disk. On Windows 95 and older versions of Windows 98 you may get an error message Cannot Locate File...... If you do, simply repoint the install wizard to the MR2800-W disk/CD-ROM and it will find the file the second time around.

Windows copies the necessary MR2800-W files to the computer. After modem installation is complete, Windows reports the MR2800-W modem in the Control Panel and Device Manager.

For D/F/TAM and D/F/TAM/Speakerphone modems, Windows finds a second device after the modem has been installed. Windows notifies you that it has found a Serial Wave Device for the modem and prompts for a Wave Device driver. Re-point to the MR2800-W disk/CD-ROM and click OK. It finds the INF file and installs the appropriate drivers.

Note:

The MR2800-W also installs a helper utility on the Windows System Tray (right side of the Task Bar). This application includes real time modem status (dialing, negotiating, connect rate, etc.), in addition to user selection for COM port, country and language, and enable/disable Windows 9x DOS box support. There is also an audio test included, which allows you to verify if your sound subsystem supports Direct X 3.0 (or later) -- needed to hear call progress (dialing and negotiation).

5	Installing SM56 Software on Windows NT 4.0

Important: Make sure you have the NT 4.0 modem drivers. They are different than those for Windows 9x or Windows 2000.

Windows NT 4.0 does not fully support plug and play (PnP). Therefore, after installing the modem board in the computer and booting, the operating system will not automatically recognize that new hardware is present. You must manually install the modem by running sm56set.exe from the distribution media. This install program will automatically add a new COM port, install the modem, then assign it to that new COM port. When installation is complete you will be prompted to Restart the PC. You must do this before using the modem.

Important: There is no Upgrade application for Windows NT 4.0.  To upgrade to a new version of the modem, you must first uninstall the previous version (Run uninstall from Add/Remove Programs in the Control Panel), then do a new installation as descibed in the previous paragraph.

6	Verifying Correct MR2800-W Modem Installation

Verifying Modem Software Operation on Windows 9x and Windows 2000

1. Open the Windows Control Panel and double click the Modems icon.

2. Select the Diagnostics tab, and single click the MR2800-W Modem to select it.

3. Click on the More Info button.

Within a few seconds a window will appear with the modem responses to various ATI commands issued to it. If this happens the modem software is properly installed and functioning correctly.

Verifying Modem Software Operation on Windows NT 4.0

There is no More Info button under the Modems icon on Windows NT 4.0. Instead, you need to open some terminal application, such as Hyperterminal, and issues the ATI commands manually.

Verifying Proper Hardware Operation

To verify correct MR2800-W modem card hardware operation, you can use the following local analog loopback (LAL) test procedure, as follows:

1. Important: Remove the telephone line cable from the modem card.

2. Open Hyperterminal.

3. Optionally, select a connection name and icon.

4. In the Connect Using window, select the MR2800-W...... modem.    Click OK.

5. Enter a number in the Phone Number box...any number since we will not be dialing it. Click OK.

6.On the next screen select Cancel.

7. Enter AT and hit <Enter>.

The response OK should appear.

8. Enter ATS46 = 23 and hit <Enter>.

9. Enter AT&T1 and hit <Enter>. Wait a few seconds.

10. Type some letters at the keyboard. If the hardware is functioning correctly, the letters you type appear on the HyperTerminal display.

11. To exit this test mode, type +++ in quick succession (note no carriage return) to escape to command mode, then type ATH <Enter> to hang up.

7	Troubleshooting

If there is a problem when using the MR2800-W modem, and your communications application does not explain the problem, check the following list of symptoms and tips.

- My Communications Software Cannot Use the Modem at COM 5

Some communications applications, such as older versions of AOL, do not communicate with a COM port higher than COM4. Sometimes on Windows 9x platforms the MR2800-W install program will not be able to map the modem to a COM port number below COM5. If the modem installs on COM 5 or higher you can manually force the modem onto a different COM port using the MR2800-W Modem helper application in the Windows System tray (just to the right of the Task Bar).

- Windows 95/98 DOS box applications do not work with the modem

By default DOS box support is disabled after installing the MR2800-W modem. You must enable it via the MR2800-W Modem helper application in the Windows System tray (just to the right of the Windows Task Bar). Make sure the DOS box check box is checked. If not, click it to check it, exit the MR2800-W Modem helper application and restart the computer. You must restart the computer to complete DOS box enabling.

If the Enable DOS box check box is already checked (i.e. enabled), and DOS box applications still do not work, try:

1.      Open the Control Panel.

2.      Double click the System icon.

3.        Expand the Modem branch.

4.      Double click the MR2800-W modem.

5.      Select the Modem tab.    Make a note of the COM port number.

6.      Select the Resources tab. Make a note of the IRQ number listed.

7.      For DOS support to operate correctly, the SM56 modem cannot occupy the standard IRQ for the COM port number.

8.      Uncheck the Use Automatic Resources check box.

Double click on the Interrupt Request label and change the IRQ to a different number that is not in conflict with another device. (If there is no free IRQ:    free one, or change other device IRQ levels to free a non-standard IRQ).

The standard I/O addresses and IRQs for COM Ports 1-4 are as follows:

COM Port		IO Address		IRQ

1			3F8			4

2			2F8			3

3			3E8			4

4			2E8			3

- There is no dial tone

1.      Ensure that the telephone cable is securely connected at both ends.

2.      Ensure the telephone cable is plugged into the correct jack on the modem -- some modems provide a second jack for handset support.

2.      Unplug the telephone line cable from the computer, and connect it directly to a telephone from the wall outlet. Check for a dial tone. If there is none, the problem is in the telephone line or system. Call the service provider.

- When Attempt To Dial You Get An Error Message

When you try to dial a connection an error message box pops up, indicating that your are not connected to a proper analog phone line. This is the MR2800-W digital line guard feature which protects against hardware damage if the modem is plugged into a high current phone outlet -- such as a digital phone jack. The MR2800-W automatically detects the over-current and goes back on-hook before hardware damage occur. Check the phone outlet and make sure that it is a real analog phone line. Try another analog phone jack.

- I Cannot Hear Call Progress

1.      Make sure your speakers are connected to the speaker output jack on your sound system.

2.	Call progress reporting on the MR2800-W requires that Direct X 3.0 or later be installed on your PC, and that your sound system supports Direct X (3.0 and later) playback. To test, go into the MR2800-W Modem helper Desktop Tray application and select the Call Progress tab. Then click the Test button. If you do not hear anything then your sound system does not support Direct X 3.0. Contact your sound card vendor for driver updates or more information.

- The modem cannot complete a connection to another modem

1.      Ensure that your modem is dialing the correct number. Ensure that you've specified the correct area code, if one is required.

2.    Determine whether the remote modem is correctly configured to communicate with yours.

- The modem does not answer incoming calls

1.      Ensure that the automatic answer parameter is set to one of the enabled options, using the ATS0 command (ATS0=1 to answer after one ring, ATS0=2 to answer after two rings, and so on).

2.      Ensure that no other devices, such as fax or answering machines, are answering calls before the modem does.

.

- The modem disconnects while transmission is in progress

1.      Ensure that the telephone cable is securely connected at both ends.

2.      Ensure that call-waiting is disabled. In most areas, the command *70 or    #70 disables call-waiting. Check with your telephone company for the correct key sequence.    (With call-waiting, the incoming call's click sound may be disrupting your call.)

3.      IDE activity (hard drive and CD-ROM devices) can cause an AC-Link modem to drop connections. Refer to the IDE Device Compatibility section for more details.

- Data is not transmitted or received for unusually long periods of time

1.      Re-dial the call.    (The telephone line connection may be poor.)

2.      Try another ISP number, the server could be busy.

- You cannot enter tone selections successfully when calling tone-driven applications

When dialing a remote system that requires you to enter selections using the telephone keys, such as a voice-mail depot or bank-account information provider, you can lengthen the duration of the tones your modem sends, so that the remote system can detect them better.    To adjust DTMF tone length, use the AT+VTDn command, where n specifies the tone duration. Check the MR2800-W online Help (via the MR2800-W Modem helper application on the Windows Desktop Tray) for a list of supported AT Commands.

- The modem does not respond to AT commands

1.      Ensure that your communications software is configured to use the same COM port as the modem's COM port.

2.      Reset modem parameters to default options by entering AT&F; then re-enter custom options.

- The modem responds to commands, but they do not appear on the screen

Make sure that the local display echo option is enabled by entering ATE1.

- You've installed a new peripheral device; now the modem does not work

1.      In the Windows desktop tray, select Start.

2.      Select Help. The Windows Help Topics window appears.

3.      Select the Contents tab.

4.      Select If You Have a Hardware Conflict.

5.      A series of troubleshooting actions appears.    Follow the appropriate sequence.

- The modem connects; then meaningless characters appear

1.      Open the Control Panel. Double click the Modems icon.

2.      Click on the COM port that the MR2800-W......Modem is installed on

3.      Click on Properties.

4.      Select Connection.

5.      Click on Advanced.

6.      Check the Use Error Control-Required to Connect box.

- The modem cannot connect; the Error Control option is selected

The modem may be connecting at a rate higher than appropriate for the line conditions.

1.      Use the AT%B command to limit the MR2800-W maximum connection rate. (For a list of AT commands, refer to the on-line User's Guide.)

2.      Lower the rate, using AT commands, until the problem is corrected. You can add AT commands to do this; refer to the next section .

- How Do I    Add AT Commands Before the Modem Dials My ISP?

1.      Open the Control Panel.

2.      Double-click the Modems icon.

3.      Click on once to select the MR2800-W Modem.

4.      Click on Properties.

5.      Select Connection.

6.      Click on Advanced.

7.      In the Extra Settings box, chain AT commands as needed. You do not need to enter the AT prefix, but it will not hurt if you do.

8	Un-installing and Upgrading the MR2800-W Modem

MR2800-W Modem Un-Install Procedure

To un-install the modem:

1. Open the Windows Control Panel

2. Double click the Add/Remove Programs icon.

3. Select MR2800-W...Modem Un-install and click Add/Remove.

4. Shut down the computer and remove the modem board from the computer.

MR2800-W Modem Upgrade Procedure

Important: There is no Upgrade application for Windows NT 4.0.  To upgrade to a new version of the modem, you must first uninstall the previous version (Run uninstall from Add/Remove Programs in the Control Panel), then perform a new Windows NT 4.0 installation as descibed earlier.

If you wish to upgrade the MR2800-W software from an earlier Build on Windows 9x and Windows 2000:

1. Obtain the latest driver set from your direct modem supplier.

2. Run the upgrade utility -- sm56set.exe, and follow the prompts

9	Changing the Operating System

The MR2800-W modem drivers are the same for Windows 95 and Windows 98. If the modem is installed on Windows 95 and you upgrade to Windows 98, the MR2800-W will continue to function as before. No new drivers are required.

However, if you change from Windows 95/98 to Windows NT 4.0 or Windows 2000 you must get a new set of drivers for that operating system. Contact your MR2800-W modem supplier for driver status and updates.

10	IDE Device Compatibility

Higher interrupt priority devices and high volume hard drive or CD-ROM activity on AC-Link PC's can pre-empt modem operation and potentially cause retrains or even disconnects. Examples include playing a game while downloading a file from The Net, or sometimes, even playing a graphic/disk intensive online game such as Quake. To avoid this you must configure your IDE devices to use DMA mode, in the BIOS (if selectable) and in Windows.

Changing the BIOS:

1. Restart your computer

2. Enter the BIOS configuration system menus.

3. Locate the hard drive configuration screens.

4. Make sure that PIO 4 and DMA2 are selected -- note that some BIOS's may not have these selections.

Changing in Windows 9x:

1. Boot into Windows.

2. Go into the Device Manager.

3. Select Disk Drives, and double click the hard drive tag.

4. Select the Settings tab and click to enable DMA.

5. repeat for your CD-ROM drive.

11	Contact Information

If you have a problem with the MR2800-W modem, ensure that the problem and its solution are not shown in the Troubleshooting section. If you cannot resolve it through this list first contact your direct MR2800-W modem or PC supplier.

Modem related information should include as much detail as possible to allow Intel and Motorola engineers to qualify and reproduce (if necessary) the problems, including:

Information about    your modem:

- MR2800-W version number (find this by recording the modem's response to the ATI3 command)

Information about    your setup:

- The telephone number you are calling from.

- The telephone number you are calling to.

- If performing a lab test, a detailed description of the equipment used.

- The remote modem information.

- Direct external analog telephone line or through a PBX -- use direct analog lines where possible.

Information about    the problem:

- The actions and steps that you performed.

- A description of what you saw; be specific.

- A description of what you expected to see.

- If possible, a description of what you saw using a non-AC-Link modem under the same conditions.

Appendix A:    MR2800-W Main Features

- Compatibility with Windows 95/98 and Win 9x DOS Box communication applications.

- Full Windows Plug and Play support.

- Powerful modem installation, un-installation, and upgrade engines.

- Data and fax mode operation.

- Support for various data modulation modes:

- V.90 connection rates if the headend is a true V.90 location. V.90 downstream rates    to 56Kbps. Upstream rates to 33.6Kbps (V.34).

- Fallback to K56flex® mode if the headend is K56flex, not V.90. K56flex® downstream rates    to 56Kbps. Upstream rates to 33.6Kbps(V.34).

- Connection at V.34 rates (33.6 Kbps) if the headend is not V.90 or K56flex®.

- V.32bis, V.32, V.22bis, V.23, V.22/B212, V.21, Bell 103.

- Error correction - V.42, LAPM, MNP2-4.

- Data compression - V.42bis, MNP5.

- Fax modes supported - V.17, V.27ter, V.29.

- Supports distinctive ring on all models except Data/Fax only modems

- Supports US Caller ID on all models except Data/Fax only modems.

- Supports Telephone Answering Machine (TAM) on all models except Data/Fax only modems.

- Tone and pulse dialing.

- Call progress audio channeling through the PC sound system (using Direct X 3.0 and later).

- Adaptive rate re-negotiation (up and down) during a connection to compensate for line dynamics.

- Includes Local Analog Loopback (LAL) for quick hardware troubleshooting.

- Includes a very useful MR2800-W Modem helper application to allow country and language selections, report real time modem status, and provides sound system test utility for Direct X 3.0 (and later).

- Online User's Guide in standard Windows HLP format. Includes modem usage and troubleshooting details, as well as a complete listing of supported AT commands.

__________

Readme 08/19/99