The Braille Sense 4.0 update is now released.

E-Mail: support@gwmicro.com

Phone: 260-489-3671

Released: 01/25/2007

We would like to thank those who have helped us in the beta testing of the Braille Sense 4.0update.  It is our belief that the public betas have greatly enhanced the Performance, Reliability, and Stability of the Braille Sense.

Improvements from 3.5 to 4.0

- The all new Database manager.  Now you can create your own Database to meet your own personal needs.

- MSN Messenger support.  Now you can send and receive text messages with others who are using MSN Messenger

- Support for Web Braille books.  Now you can download and read books from Web Braille.

- A compressed mode for reading books that are not formatted for 32 cells.  Now when using Compressed Document mode, you can fully utilize all of your Braille Display when reading books such as those from Web Braille.  This mode can be turned on when reading in the word Processor by pressing SPACE-DOTS-1-2-4-6

- Support for HTML e-mail messages.

- Added the ability to open and read Microsoft Word documents.

- In the Word Processor in addition to the TXT, BRL, and HBL document types that were already supported  we have added the ability to save a document as BRF, DOC, and RTF.

- We have fixed lockups that were happening with the Braille Sense DAISY player when reading certain DAISY files.

- Improved functionality for the Bluetooth manager.

- Added a Check Spelling Automatically option in the Options menu.  When you turn this option on if there is a misspelled word the Braille Sense will sound a tone alerting you that a word has been misspelled. This feature will work in programs such as the Word Processor, e-mail, Msn Messenger, etc.

- Added a Default Document option in the Options menu on the Braille Sense.  Now you can choose to have the Word Processor default to BRL, TXT, or HBL.  The default is now set to BRL.

- Added a Backup Restore Settings item under Utilities.  Now you can easily backup and restore your Settings as well as other items on your Braille Sense.

- The manual has been updated on the Braille Sense.  You will find the new 4.0 manual under help . To Access the manual press F1 to go in to the Braille Sense Program menu.  Press H (DOTS-1-2-5) then use SPACE-DOT-4 to arrow to the section you want to read. Press ENTER on the section that you want to read.

The manual in ASCII text and Microsoft Word documents can be downloaded by going to our FTP site.

ftp.gwmicro.com/braille_sense

Where you will find, braille_sense_manual_for_software_version_4_0_word_files.zip

As well as the file,  braille_sense_manual_for_version_4_0_text_files.zip.

How to Update Your Braille Sense

There are 2 ways that you can update your Braille Sense, the online upgrade, or downloading the update from our web page and using the Upgrade Offline option.

Please note that when upgrading the Braille Sense you must be connected to AC power.

Using the Online Upgrade Option

If you have setup your Internet connection you can update your Braille Sense to 4.0 by doing the following.

- Press F1 to bring up the list of programs on the Braille Sense.

- Type U (DOTS-1-3-6) to bring up the utilities option.

- Type U again to bring up the Upgrade Braille Sense option, and press ENTER.

- The update should begin downloading.

- As the Upgrade continues progress messages will be announced. The progress will be shown on the Braille Sense Braille display.

- Do not disconnect the Braille Sense from AC power during the upgrade process.

- When the update is complete you will hear a message that says “system update success, system will be restarted.” Once you hear the Braille Sense startup music you will be placed at the File manager. You can check your version by doing SPACE-V (Ddots-1-2-3-6.)  Your version should read, “Braille Sense S/W Ver. 4.0, build 20070124.”

- Press SPACE-E DOTS-1-5) or SPACE-Z DOTS-1-3-5-6) to exit back to the list of programs on your Braille Sense.

Now you can begin enjoying the new features the Braille Sense 4.0 has to offer.

Downloading the Braille Sense 4.0 Upgrade from Our FTP site, and Using the Upgrade Offline option

The easiest way to upgrade the Braille Sense is by using the Online Upgrade feature.

However there may be times when you want to use the Upgrade Offline option to update a Braille Sense

- Go to ftp.gwmicro.com/braille_sense/ braille_sense_eng_v40_20070124_local.zip

- When prompted choose “save” and choose the folder where you want to save the file.

- When the download is complete browse to the folder where you downloaded the zip file.

- Open the zip file.  Press CONTROL-A to select the contents of the zip file and then press CONTROL-C to copy the contents of the zip file to the clipboard.

- Insert your USB stick or Memory card and browse to the drive letter of the USB stick or Memory card.

- Open up the drive and press CONTROL-V to copy the contents of the zip file to the root of your USB stick or compact Flash card.

- Connect the Braille Sense to AC power and turn on the Braille Sense.

- Insert the Compact Flash card or USB stick in to the Braille Sense

- Press F1 to bring up the list of programs on the Braille Sense.

- Type U (DOTS-1-3-6) to bring up the utilities option.

- Type U again to bring up the Upgrade Braille Sense option.

- Press SPACE-DOT-4 to arrow to Upgrade Offline.

The upgrade should begin installing.

As the Upgrade continues progress messages will be announced. The progress will be shown on the Braille Sense Braille display.

Do not disconnect the Braille Sense from AC power during the upgrade process.

- When the update is complete you will hear a message that says “system update success, system will be restarted.” Once you hear the Braille Sense startup music you will be placed at the File manager. You can check your version by doing SPACE-V (Ddots-1-2-3-6.)  Your version should read, “Braille Sense S/W Ver. 4.0, build 20070124.”

- Press SPACE-E DOTS-1-5) or SPACE-Z DOTS-1-3-5-6) to exit back to the list of programs on your Braille Sense.

Now you can begin enjoying the new features the Braille Sense 4.0 has to offer.

It is because of the feedback from you the Braille Sense user that these new features, enhancements, and improvements, have been made in the Braille Sense 4.0 upgrade.

Please let us know if you have any questions, comments, or suggestions and we will be glad to pass them on to the developers.