BRAILLE SENSE

Manual:

(For Software Version 4.0)

SECTION 7 E-Mail

TABLE OF CONTENTS

7. Email………………………………………………………………..…...2

7.1 Executing E-mail and Service Registration	2

7.1.1 Launching the E-Mail Program	2

7.1.2 Registering E-Mail Service	3

7.2 Sending and Receiving E-Mail	4

7.2.1 Receiving E-mail	4

7.2.2 Reading the Received E-mail Messages	5

7.2.3 Sending E-Mail Messages	8

7.2.4 Local Folder	14

7.3 Functions of the InBox	15

7.3.1 Deleting E-mail in Inbox	15

7.3.2 Reply And Reply All to a Received E-Mail Message	17

7.3.3 Forwarding a Received E-Mail Message	18

7.3.4 Saving a Received E-Mail Message	19

7.3.5 Sending E-Mail Saved in the Storage Box	20

7.3.6 Printing E-Mail Messages	21

7.4 Edit Menu	21

7.4.1 Select All	22

7.4.2 Start Selection	22

7.4.3 Copy	22

7.4.4 Paste	23

7.4.5 Cut	23

7.4.6 Delete	23

7.4.7 Move to Unread Message	24

7.4.8 Find	24

7.4.9 Find Again	25

7.4.10 Copy to Folder	25

7.4.11 Move to Folder	27

7.5 Settings in the Tool Menu	28

7.5.1 Service Manager	29

7.5.2 Set Path	33

7.5.3 Set Options	36

7.5.4 Set Spam	36

7.6. Hot Keys for E-Mail	41

7.6.1 Hot Keys That Are Used in the Inbox	41

7.6.2 Hot Keys for Sending E-Mail Including Reply, Forward, and Save in the Mail Storage Box	42

7.6.3 Hot Keys for Reading or Writing an E-mail Message	42

# 7. E-mail

The e-mail program is used to communicate with others using the Internet.  In order to use the e-mail program, you must have your own e-mail account, which has been provided to you by your Internet service provider.

The e-mail account that you have should support POP3 (Post Office Protocol 3) and SMTP (Simple Mail Transfer Protocol).  You should check with your Internet service provider to make sure that your provider supports POP3 and SMTP.

Before you use the e-mail program, you will need to setup the Braille Sense to use the Internet, which is in the “utility” menu (see chapter 11).

When checking the “inbox” in the email program, the menus are different from the menus that are used when you are writing an email.  You should read this chapter carefully, before you use the e-mail program on the Braille Sense.

## 7.1 Executing E-mail and Service Registration

### 7.1.1 Launching the E-Mail Program

To launch the e-mail program on the Braille Sense press “f1” to bring up the “program” menu of the Braille Sense, then press “space-4” repeatedly until “e-mail” is displayed, and then press “enter.”  Or, just press “e (dots 1-5)” on the “program” menu in order to execute the e-mail program.

When you launch the e-mail program for the first time, the Braille Sense will display “create service information at service manager in tool menu.”  Then, the “inbox” will be opened.  The “inbox” is the main window of the e-mail program.  When you run the e-mail program for the first time, this “inbox” is empty and the Braille Sense displays “no list 0/0.”

If there are any e-mail messages that the Braille Sense has already received, “making mail list,” “exist mail count xx” will be announced in voice and on the LCD display; however, “xx% processed” will be displayed on the Braille display.  Once progress has reached100%, the “inbox” will be opened and it will show the received e-mail in the list.

### 7.1.2 Registering E-Mail Service

In order to receive e-mail, your e-mail service information must be registered in the Braille Sense.  If you have not yet registered your e-mail service information in the Braille Sense, please register your e-mail service information as follows:

- Call up the e-mail menu by pressing “space-m (dots 1-3-4)” or “F2.”

- Move to the “tool” menu by pressing “space-4.”

- Press “enter” when “tool” is displayed.  Then, “service manager” will be displayed.

- Press “enter” on “service manager,” then “service name: no list 0/0” will be displayed.  You can also move to this menu by pressing “enter-m (dots 1-3-4).”

- Move to “register” by pressing “tab (space-4-5).”

- Press “enter” when “register” is displayed.

- Then, the dialog box will be opened and “service name?” will be displayed.  Type in the service name with the Braille keys.  (Ex: Braille Sense)

- Press “space-4” to move to “logon id?.”  Then, type in your e-mail ID using computer Braille.  (Ex: Sense or sense@braillesense.com)

- Press “space-4” to move to “password?.”  Type in your password using computer Braille.  When you type in your password, the Braille Sense will display the password you typed in as “*.”  (Ex: ******)

- Press “space-4” to move to “e-mail address?.”  Type in your e-mail address using computer Braille.  (Ex: sense@braillesense.com)

- Press “space-4” to move to “pop3 server?.”  Type in the POP3 server address using computer Braille (Ex: mail.braillesense.com).  Check with your Internet service provider or network administrator to find out what you should use as the POP3 server name.

- Press “space-4” to move to “smtp server?” and type in the SMTP server address (Ex: smtp.braillesense.com).  Check with your Internet service provider or network administrator to find out what you should use as the SMTP server name.

- Press “tab (space-4-5)” to move to “confirm,” and then press “enter” in order to save the changes.  Or, just press “enter.”

- Then, the Braille Sense will exit to “service name list” and show the registered service name (Ex: service name: Braille Sense 1/1).

- If you want to register another service, go to step 5 again, and follow the steps explained above.

- When you finish registering the services, press “space-z (dots 1-3-5-6)” to exit to the “inbox.”

In order to move from a setting item (Ex: ID, password) to the previous setting item, press “space-1.”  And, in order to move to the “confirm” button or the “cancel” button, press “tab (space-4-5)” or “shift-tab (space-1-2).”

If you made a typo in an item such as service name, ID, or password, you can correct the typo as follows;

1)  In order to delete the letter where the cursor is located, press “space-d (dots 1-4-5).”

2)  In order to delete the letter in front of the cursor position, press “backspace.”

3)  In order to delete the current line, press “backspace-1-4.”  Then, the Braille Sense will ask “delete line? yes.”  If you press “enter,” the selected line will be deleted.  If you do not want to delete the current line, press “space” to change from “yes” to “no” and press “enter.”

When you need to type using computer Braille to type in your ID, password, and address, please use the following computer Braille:

1)  Dot (.””): dots 4-6

2)  At sign (“@”): Press “space-u” and release it, then press dot 4.

3)  Under bar (“_”): dots 4-5-6

4)  Dash (“-”): dots 3-6

You will find other menus like “information,” “modify,” “delete,” and “close” in the “service manager.”  These menus will be explained in section 7.5.1.

## 7.2 Sending and Receiving E-Mail

### 7.2.1 Receiving E-mail

In order to receive e-mail, you must register your email service information as explained in section 7.1.2.  The Braille Sense is capable of receiving plain text email and html email messages

You can receive e-mail messages by doing the following.

1)  Call up the e-mail menu by pressing “space-m (dots 1-3-4)” or “F2.”

2)  Move to “message” by pressing “space-4.”

3)  Press “enter” on the “message” menu.  Then, “reply” will be displayed.  You can move to this menu directly by pressing “backspace-m (dots 1-3-4).”

4)  Move to “receive new mail” by pressing “space-4.”

5)  Press “enter” on “receive new mail.”  Then, a dialog box will appear where you can select the service list and “service name?” is displayed.  You can move to this dialog box directly by pressing “enter-n (dots 1-3-4-5).”  This dialog box consists of “service list,” a “confirm” button, and a “cancel” button.

6)  When “service name? (registered service name x/x)” is displayed, press “space-1” or “space-4” to move to the desired service.

7)  Press “enter” when the desired service is displayed, or press “tab (space-4-5)” to move to the “confirm” button, and press “enter.”

8)  Then the Braille Sense will display “xx mail to receive, start receiving;” “xx/xx mail received;” “complete.”  “Received xx mail,” “success xx,” and “fail xx” will be displayed sequentially. When the e-mail messages are downloaded from the server, the e-mail list is displayed in the “inbox” (Ex: title: (title of the email received) xx/xx).

In step 8 above, if the e-mail size is more than the Braille Sense can handle, it announces “source data exceeded the buffer size.  Skip current mail,” and then the Braille Sense will begin downloading the next e-mail message.

If you want to cancel the download, press “tab (space-4-5)” at step 7 above to move to the “cancel” button, and press “enter,” or you can press “space-e (dots 1-5).”

### 7.2.2 Reading the Received E-mail Messages

If you downloaded e-mail from your server as explained in section 7.2.1, the e-mail messages will be listed in the “inbox.”  Each e-mail in the “inbox” has the following items.

1)  Title (Subject)

2)  Date

3)  From (Sender)

4)  Message

5)  Attach (this will be displayed only for an e-mail that has an attached file)

You can move from one item to the next item or previous item by pressing “tab (space-4-5)” or “shift-tab (space-1-2),” respectively.

#### 7.2.2.1 Title Control

“Title” shows the title of the e-mail; “title: (title of the e-mail) xx/yy.”  After the title of the email, you will find “xx/yy,” which shows the number (xx) of the current e-mail out of the total number (yy) of received e-mail messages.  For example, “11/21” means that the e-mail is the 11th e-mail out of a total of 21 e-mail messages.

If there is an attached file to the e-mail, “a title: (title of the e-mail) xx/yy” will be displayed.  The symbol “a (dot 1)” means that the e-mail has an attached file.  It is displayed as “a (dot 1)” on the Braille display, and it is announced as “1 attachment,” which indicates the number of the attached files in voice.

The Braille Sense also indicates which e-mail messages have been read, and which e-mail messages have been unread.  E-mail messages that have not yet been read have the “-” sign placed in front of the title.  For example, let’s suppose there is an email with an attachment that has been unread.  It would appear as “-a title: (title of the e-mail) xx/yy.”

You can move from one e-mail message to the next e-mail message when this title is displayed.  You can move by using the following keys;

Move to the next e-mail:  Space-4

Move to the previous e-mail:  Space-1

Move to the first e-mail in the Inbox:  Space-1-2-3 or space-1-3

Move to the last e-mail in the Inbox:  Space-4-5-6 or space-4-6

#### 7.2.2.2 Date Control

“Date” shows the date and time when the e-mail was received; “date: (date), (time) xx/yy.”  The date is displayed in the following order:  day, month, date, and year.  The time is displayed in the following order:  hour, minute, and second with the 24 hour system.  And, “xx/yy” shows the current number of the email out of the total number of e-mail messages received.  You can also move from one e-mail message to the next e-mail message when the date is displayed.  The move keys are explained in section 7.2.2.1.

The Braille Sense also indicates which e-mail messages have been read, and which e-mail messages have been unread.  E-mail messages that have not yet been read will have the “-” sign placed in front of the date.  For example, let’s suppose there is an email that has been unread.  It would appear as “- date: (date), (time) xx/yy.”

#### 7.2.2.3 Sender Control

“From” shows the sender information; “from: (sender’s e-mail address) xx/yy.”

The Braille Sense also indicates which e-mail messages have been read, and which e-mail messages have been unread.  E-mail messages that have not yet been read will have the “-” sign placed in front of the sender.  For example, let’s suppose there is an email that has been unread.  It would appear as “- from: (sender’s e-mail address) xx/yy”

You can save the e-mail id of the sender using the following sequence:

1)  Press “enter-I (dots 2-4)” when “from: (sender’s e-mail address) xx/yy” is displayed.

2)  Then, the “add address” dialog box will be opened, and “name? (sender’s e-mail address)” will be displayed.

3)  Type in the sender’s name with your Braille keyboard, or press “tab (space-4-5)” to move to “confirm,” and press “enter.”

When you type in the name, you can type it in directly without pressing the cursor key to designate the input position on the Braille cell, if you want to retype the name.  But, if you designate a specified position by pressing a corresponding cursor key, you can modify the displayed name.

If the address that was typed in is saved, and the dialog box is closed, the Braille Sense will show you the “from” item again.

By saving the e-mail ID using the above method, the name and its e-mail id will be saved in the Braille Sense.  The saved e-mail ID can be used to send your message.  This will be explained in section 7.2.3.3.

If you want to cancel saving the e-mail ID, press “tab (space-4-5)” to move to the “cancel” button, and press “enter,” or just press “space-e (dots 1-5).”

#### 7.2.2.4 Message Control

“Message” shows the message body of the selected e-mail.  You can move to a message by pressing “tab (space-4-5).”  Or, you can read a message by pressing “enter” when “title”, “date,” or “from” is displayed.   When you read the message, all of the control keys are the same as in the word processor.

#### 7.2.2.5 Attachment Control

You can find and download the attached file at ‘Attach’. It will appear only for the e-mail that has an attached file.

‘Attach’ is displayed in the form of ‘attach: (attached file name) xx/yy’. The ‘yy’ is the total number of the attached files to the e-mail and ‘xx’ is the ‘xxth’ attached file.

The following is the procedure to download attachments;

1)  Move to the desired file name in “attach” by pressing “space-4” or “space-1.”

2)  Press “space” on the file name that you want to open.  Then “*” will be displayed in front of the file name.

3)  Press “enter” to download that file. Then “download path: /flashdisk/download/exchange? No” will be displayed. Press “enter” in here.

By doing so, the selected file will be saved in the “download” folder in “flashdisk.”

If you have more than one attachment that you want to download, you can press “enter-a (dot 1).”  If you want to download only the attached file that is displayed on the Braille display, just press “enter” without pressing “space.”

If you want to change the download path, please refer to section 7.5.2.1 (Set Download Path).

### 7.2.3 Sending E-Mail Messages

In order to send an e-mail message, you should register your e-mail address as explained in section 7.1.2 (Registering e-mail service).  Writing an e-mail message and sending procedure is as follows;

1)  Call up the e-mail menu by pressing “space-m (dots 1-3-4)” or “F2.”  The “file” menu will be displayed.

2)  Press “enter” on the “file” menu.  Then “write new mail” will be displayed.

3)  Press “enter” on “write new mail.”

4)  Then “recipient?” will be displayed on the Braille display.  Pressing “enter-w (dots 2-4-5-6)” will take you directly to this stage.

5)  Type in the e-mail address of the recipient when “recipient?” is displayed.  Please type in the e-mail address using computer Braille (Ex: sense@braillesense.com).

6)  Next, press “tab (space-4-5)” to move to “carbon copy recipient?.”

7)  Type in the e-mail address of the other recipient using computer Braille.  If you do not need to carbon copy a recipient, you can skip to the next step.

8)  Press “tab (space-4-)” to move to “hidden recipient?.”

9)  Type in the e-mail address of another recipient using computer Braille.  This e-mail address will not be seen by the recipient or the carbon copy recipient.  If there is no “hidden recipient,” you can skip to next step.

10)  Press “tab (space-4-5)” to move to “subject?.”

11)  Type in the title of the e-mail message at “subject?.”  You can input the subject in ASCII, Grade 1, or Grade 2.  You can change the mode by pressing “space-g (dots 1-2-4-5).

12)  When you finish typing in the subject, press “tab (space-4-5)” to move to “message?.”

13)  You can write an e-mail message.  The input method and the editing method are the same as the input and editing method used in the word processor.  You can also write the message in ASCII, Grade 1, or Grade 2.

14)  When you finish writing the e-mail message, press “space-m (dots 1-3-4)” to call up the “e-mail” menu.  Then the “file” menu will be displayed again.

15)  Press “enter.”

16)  Press “space-4” to move to “send.”

17)  Press “enter” when “send” is displayed on the Braille display.  Then, the “service list” dialog box will be opened.  If you press “enter-s (dots 2-3-4)” at step 15, the “service list” dialog box will be displayed at once.  The “service list” consists of “service list,” “confirm,” and “cancel.”

18)  “Service name? (registered service name x/y)” will be displayed on the Braille display.  You can select the service name by pressing “space-1” or “space-4.”

19)  Press “enter” on the desired service name.  Or, move to the “confirm” button by pressing “tab (space-4-5),” and then press “enter.”

20)  Then, “under sending” will be displayed, and the e-mail will be sent to the recipients.  When the Braille Sense finishes sending e-mail, “completed send email” will be displayed.  If it fails to send the e-mail, the Braille Sense will display “failed send message.”  If this occurs, the e-mail will be saved in the “storage box.”

If the Braille Sense completed or failed to send the e-mail, it will return you to the “inbox.”

At “recipient?,” “carbon copy recipient?,” and “hidden recipient?,” you can input more than one e-mail address.  You can also type in e-mail addresses that have been saved in the Braille Sense by searching the address list.  This method will be explained in section 7.2.3.2 (Sending e-mail to Multiple Recipients) and section 7.2.3.3 (Searching Address List).

If you want to attach a file, execute “file attachment,” which will be explained in section 7.2.3.1 (File Attachment), between steps 13 and 14 above.

If you want to cancel writing and sending an e-mail during the execution of the above steps, you can exit by pressing “space-e (dots 1-5)” or “space-z (dots 1-3-5-6)” before step 14.  Then the Braille Sense will return you to the “inbox.”

#### 7.2.3.1 Sending a File Attachment

You can attach a file that you want to send with an e-mail while you are writing an e-mail or inputting the recipient, hidden recipient, or subject.

The following is the procedure to attach a file.

- Call up the menu by pressing “space-m (dots 1-3-4)” or “F2.”  Then, the “file” menu will be displayed.

- Press “enter” on the “file” menu.  Then, “write new mail” will be displayed.

- Move to “attach file” by pressing “space-4.”

- Press “enter” on “attach file.”  Then, “file list” will be opened.

You can get to this step directly from step 1 by pressing “enter-f (dots 1-2-4).”  The default folder for the “file list” is the “upload” folder, which is a sub folder of “flashdisk.”  If there are no folders in the “upload” folder of the Braille Sense due to any reason, “no list” will be displayed.  In this case, you have to make a folder in the “file manager,” or move to the desired folder with the move keys that are explained below.

The move keys in the file list are as follows;

Move to the upper level folder:  Backspace

Move to previous item:  Space-1

Move to next item:  Space-4

Move to the beginning of the list:  Space-1-2-3

Move to the end of the list:  Space-4-5-6

- Move to the desired file, and press “enter.”  If you want to attach more than one file, move to the desired file, and press “space” repeatedly.  Then “*” will be displayed in front of the file names.  Press “enter” if you want to attach the selected files.

- If the selected files are attached, the Braille Sense will announce “xx attached,” and then move to the stage before you executed step 1 in this procedure.  “xx” is the number of the attached files.

If there are any files attached to an e-mail, the “attach” item will be displayed after the “message” item like the following:  “attach: (attached file name) xx/yy (file size).”  “yy” is the total number of attached files, and “xx” means the “xxth” file out of “yy” files.

You can move and check the attached files by pressing “space-1” or “space-4.”  If there is an undesired file in the attached file list, you can delete it from the list by pressing “space-d (dots 1-4-5).”  If you press “space-d (dots 1-4-5),” “delete (file name) file? Yes” will be displayed.  If you press “enter,” this file will be deleted from the attached file list.  If you press “space” to switch from “yes” to “no,” and then press “enter,” your selection will be cancelled, and the file will remain on the attached file list.

#### 7.2.3.2 Sending E-Mail to Multiple Recipients

You can type in more than one recipient, carbon copy recipient, or hidden recipient.  If you type in more than one e-mail address in those items, the same e-mail will be sent to all of them at the same time.

When you input more than one recipient, you should insert a ‘;’ or ‘,’ between the e-mail addresses using the following Braille dots.

Semi-colon (";"):  Dots 5-6

Comma (","):  Dot 6

For example, “sense@braillesense.com;hims@braillesense.com.”

#### 7.2.3.3 Searching the Address List

You can type in the recipient at “recipient,” “carbon copy recipient,” and “hidden recipient” from the address list without typing in the e-mail address directly.

The address list has the e-mail addresses that were typed in to the “address manager” and were typed in at the “from control” that was explained in section 7.2.2.3 (Sender control).

The following are the steps to search for e-mail addresses:

1)  Press “space-m (dots 1-3-4)” or “F2” to call up the “e-mail” menu at “recipient,” “carbon copy recipient,” or “hidden recipient.”  Then, “file” will be displayed.

2)  Press “enter” on the “file” menu.

3)  Press “space-4” to move to “lookup address.”

4)  Press “enter” on “lookup address.”  Then “address list key word” will be displayed.  Pressing “enter-l (dots 1-2-3)” will take you to this step directly.

5)  Type in the last name of the desired recipient at “address list key word,” and then press “enter.”  Or, press “tab (space-4-5)” to move to “confirm,” and then press “enter.”

6)  Then, “xx addresses are found” will be displayed, and the first item of the found addresses is also displayed.  Here, “xx” is the number of the found addresses that have the same name in the address list (Ex: HIMS hims@brailleSense.com 1/xx).

7)  Press “space-1” or “space-4” to move to the desired address, and press “enter” when the desired address is displayed.  You can select more than one address by pressing “space.”  When you have chosen the addresses you want, press “enter.”

8)  Then, the selected addresses are inserted in “recipient,” “carbon copy recipient” or “hidden recipient.”

In step 5 above, if you press “enter” without typing in the desired name, all of the addresses in your address list will be displayed.

In step 5, if there is no matching address that you want to find, then “not found item” will be announced and “address list key word” will be displayed again.

And, in step 7 above, if there is a matching name, but no e-mail address, “no mail address” will be displayed, and the item cannot be selected.

#### 7.2.3.4 Saving E-Mail Messages as Text, and to the Storage Box

If you want to send an e-mail message later, you can save it as text in the storage box.  This “save as text” function will save the email as text, and place it in the storage box.

The following describes how to save an email as text:

1)  Press “space-m (dots 1-3-4)” or “F2” after you write the e-mail message.  Then, the “file” menu will displayed.

2)  Press “enter.”

3)  Press “space-4” to move to “save as text.”

4)  Press “enter.”  Then, the dialog box for save will be opened, and “file name? noname.txt” will be displayed.  You can jump to this stage by pressing “space-s (dots 2-3-4).”

5)  Press “enter” on “file name? noname.txt,” or press “tab (space-4-5)” to move to the “confirm” button.  Then, the message will be saved as “noname.txt.”  If you want to name the e-mail message, type in a file name.  The detailed procedure is the same as “save as” in the word processor.  You can also change the path for saving it.  Please refer to section 4.1.4 for the explanation of “save as” in the word processor in this user manual.

6)  After the Braille Sense saves the e-mail message, it will return to the status it was at before step 1 above.

By following the explanation above, the e-mail message will be saved as “noname.txt” in the “flashdisk/email/outbox.”  If you want to find this saved message, you can retrieve and send it by using “send mail in storage box,” which will be explained later.

The “title” of that e-mail will be the original title that you already typed in when you typed it in as the “subject” before, instead of “noname.”

If you want to save the file in another folder, and not in “flashdisk/email/outbox,” you will not find the e-mail message when you execute “send mail in storage box.”

Regarding how to send email that is in the storage box, please refer to section 7.3.5 (Sending E-mail Saved in the Storage Box).

If you did not type anything in the “message,” and executed “save as storage box,” then “body to save does not found” will be announced, and you will be returned to the status you were in before executing “save as storage box.”

While you are typing in a “message,” if you press “space-e (dots 1-5)” to cancel writing an e-mail, the Braille Sense will display “current writing mail save? Yes.”  If you press “enter,” then the Braille Sense will execute from step 5 above.  If you want to return to the “inbox” without saving the email, press “space” to change “yes” to “no,” and then press “enter.”

If you press “space-z (dots 1-3-5-6)” instead of “space-e (dots 1-5)” while you are writing an e-mail message, the Braille Sense will return to the “inbox” without asking whether it will save the message or not.

- 7.2.4 Local Folder

The “local folder” function is located under the “file” menu.  This function lets you navigate between different email folders.  It also lets you cut or copy messages between email folders.

To use the “local folder” function, use the following steps:

- Open the menu by pressing “space-m (dots 1-3-4)” or “F2,” and then press “enter” to open the “file” menu.  Press “space-4” or “space-1” to move to “local folder.”  Then, press “enter” on “local folder.”  While you are in the “file” menu, the shortcut key is “d (dots 1-4-5),” and the hotkey is “enter-d (dots 1-4-5).”

- The “local folder” dialog box consists of 3 controls:  “folder name” list, a “confirm” button, and a “cancel” button.  You can move between these controls by pressing “tab (“space-4-5”)” or “shift-tab (“space-1-2”).”

- In the “folder name” list, select the email folder to go to by pressing “space-4” or “space-1.”

- Then, move to the “confirm” button by pressing “tab (“space-4-5”).”  Press “enter” to open the selected email folder.  Or, you can press “tab (“space-4-5”)” once more to move to the “cancel” button.  If you want to cancel, press “enter” on cancel.  Pressing “space-z (dots 1-3-5-6)” or “space-e (dots 1-5)” anywhere in the dialog box will cancel and close the “local folder” dialog box.

To move or copy e-mail messages to a different email folder, you can select e-mail messages by pressing “space.”  Press “enter-x (dots 1-3-4-6)” to cut, or press “enter-c (dots 1-4)” to copy the email.  Go to the email folder where you want to paste the e-mail messages using “local folder.”  Then, press “enter-v (dots 1-2-3-6)” to paste.  For more information on how to copy e-mail messages to another email folder or move e-mail messages to another email folder, see sections 7.4.10 and 7.4.11.

## 7.3 Functions of the InBox

### 7.3.1 Deleting E-mail in Inbox

You can delete email that you have received from the “inbox.”  You can delete an e-mail message that you have selected, or, more than one e-mail by selecting each of them individually.

#### 7.3.1.1 Deleting One E-Mail Message

You can delete an e-mail message in the “inbox” by following the steps below;

1)  Press “space-1” or “space-4” to move to the “title” of the e-mail that you want to delete.

2)  Press “space-d (dots 1-4-5).”  Then “(title of the selected e-mail) files delete? Yes” will be displayed.

3)  Press “enter” if you want to delete it.

If you want to cancel it, at step 2 above, press “space” to change “yes” to “no,” and then press “enter.”  Or, you can press “space-e (dots 1-5)” or space-z (dots 1-3-5-6).”

#### 7.3.1.2 Deleting Multiple E-mail Messages

If you want to delete more than one e-mail message, delete the e-mail messages using the following steps:

1)  Press “space-1” or “space-4” to move to the e-mail “title” that you want to delete.  You can select it in “date control,” too.

2)  Press “space” on the e-mail that you want to delete.  Then “*” will be displayed in front of the selected e-mail title, and the label (“title:”) will disappear.  So, it will then be displayed as “* (e-mail title) xx/yy.”

3)  Move to the other e-mail that you want to delete by pressing “space-4” or “space-1.”  Then, press “space.”

4)  You can select more e-mail messages that you want to delete.  Once you finish selecting all of the e-mail messages you want to delete, press “space-d (dots 1-4-5).”

5)  Then “xx mail(s) deleted? Yes” will be displayed.  If you press “enter,” the selected e-mail messages will be deleted from the Braille Sense.

If you want to cancel the deletion when the “xx mail(s) deleted? Yes” is displayed, you can toggle “yes” to “no” by pressing “space,” and then press “enter.”  Or, you can cancel it by pressing “space-e (dots 1-5)” or “space-z (dots 1-3-5-6)” at step 2 above.

#### 7.3.1.3 Deleting E-Mail by Making a Selection

If you want to delete multiple e-mail messages by designating a selection, you can delete them as follows:

1)  Press “enter-a (dot 1)” on the “title” or “date” control.  Then, all of the e-mail messages in the “inbox” will be selected.  Then, “*” will be displayed in front of all of the e-mail messages.

2)  Press “space-d (dots 1-4-5)”

3)  “xx files deleted? Yes” will be displayed.  If you press “enter” on “yes,” all of the selected e-mail messages will be deleted.

If you want to delete the successive e-mail messages, you can delete them using the following steps:

1)  Press “enter-b (dots 1-2)” on the first e-mail where you want to start the selection.

2)  Press “space-4” to move to the last e-mail that you want to delete.  Then, press “space-d (dots 1-4-5).”

3)  “xx files delete? Yes” will be displayed.  Press “enter” if you want to delete them.

You can Press “enter-b (dots 1-2)” which will make the selection, just like the “start selection” function in the word processor.  In front of the e-mail titles selected, “*” will be attached.

If you have pressed “enter-a (dot 1)” or “enter-b (dots 1-2),” but you want to cancel it, press “space” when “xx files delete? Yes” is displayed to toggle “yes” to “no,” and then press “enter.”

Or, you can cancel it by pressing “space-e (dots 1-5)” or “space-z (dots 1-3-5-6).”

### 7.3.2 Reply And Reply All to a Received E-Mail Message

When you reply to an email message that was received, you can edit or add a message to the original message or add another recipient.  For example, you can reply to an e-mail message that you previously received in your “inbox.”  Let’s demonstrate with an example.  Suppose you received an email that had the following information:

Sender’s e-mail address:  hims@brailleSense.com

Title of the e-mail:  Welcome to HIMS

You could reply to this e-mail by completing the following steps:

1)  Press “space-m (dots 1-3-4)” or “F2” to call up the menu when you are in “title,” “date,” or “message” control

2)  Press “space-4” to move to the “message” menu.

3)  Press “enter.”  Then, “reply” will be displayed.

4)  Press “enter” on “reply.”  Then, “message?” will be displayed on the Braille display.  You can find the original message body of the received email; “message? –-- source message –--“ followed by the received email message.  You can get to this stage by pressing “enter-r (dots 1-2-3-5).”

5) Press “shift-tab (space-1-2)” to move to “subject?,” hidden recipient”,” “carbon copy recipient,” and “recipient?”  At “subject?,” “subject? Re: Welcome to HIMS” will be displayed.  And, at “recipient?,” “recipient? hims@braillesense.com” will be displayed.

6)  Press “tab (space-4-5)” to move to “message?.”  Type in the message that you want to add or edit.  Type in the “message” that you want to add or edit.  You can type in your message as in step 14 in section 7.2.3 (Sending e-mail).

After you press “enter-r (dots 1-2-3-5),” you can write an e-mail message with the same method that was explained in section 7.2.3 (Sending E-mail) in this user manual.  You can attach a file, and add or delete recipients as well as write an e-mail message.

If you want to add another e-mail recipient, you can input it as described in step 5 above.  Press the “cursor routing key” that is located at the position where you want to add the recipient, and type in the new e-mail address.  Use computer Braille when typing in this information.

You can also save the e-mail message that you are writing as a reply.  The method is the same as that explained in section 7.2.3.4.  It will be saved in the storage box.

You can reply to an email by using "reply all," as well as "reply."  Pressing “reply” will send a message to only the original sender.  Pressing “reply all” will send an email message back to the original sender as well as all other recipients to the original email.  However, “reply all” will not send an email to you, even though you were a recipient of the original email.

To activate the “reply all” function, open the menu by pressing “space-m (dots 1-3-4) or “F2.”  Then, press “space-1” or “space-4” to move to “message.”  Then, press “enter” on “message.”  Use “space-4” to move to “reply all,” and press “enter.”  You can activate the “reply all” function without calling up the menu by pressing “backspace-r (dots 1-2-3-5).”  When you activate the “reply all” function, you will see all email addresses of the original sender and the original recipients in the "recipient?" edit box.  Then, you can use the same steps to send a message as you would when using the “reply” function.

### 7.3.3 Forwarding a Received E-Mail Message

“Forwarding” means that you will send an e-mail that you received to another person.  You can also add or edit the message and subject (title).  You can also attach a file to this e-mail.

Let’s explain how to use “forward” with an example.

Let’s suppose there is an e-mail message that you received as follows:

Sender’s e-mail address:  hims@braillesense.com

Title of the e-mail:  Welcome to HIMS

You can forward this e-mail to the others using the following method:

1)  Press “space-m (dots 1-3-4)” or “F2” to call up the menu when title, date, or message item is displayed.

2)  Press “space-4” to move to the “message” menu.

3)  Press “enter.”  Then, “reply” will be displayed.

4)  Press “space-4” to move to “forward.”

5)  Press “enter.”  Then “recipient?” will be displayed.  You can also jump to this stage by pressing “enter-f (dots 1-2-4).”  Input the e-mail address of the new recipient using computer Braille.

6)  Press “tab (space-4-5)” to move to “carbon copy recipient?,” “hidden recipient?,” and “subject?.”  At “subject,” the title of the e-mail will be displayed like “subject? Fw: Welcome to HIMS.”

7)  Press “tab (“space-4-5”)” to move to “message?.”  You can find the original e-mail message with the following message; “message? ----- forward message -----.”

8)  Type in the message.  The method is the same as explained on section 7.2.3 (Sending e-mail).

From step 5 of this procedure, the method to write e-mail is the same as explained in section 7.3.2.  So, you can type in additional recipients and attach files to the email that you are forwarding.

You can also save the e-mail that you are writing in the storage box.  How to save a message in the storage box is explained in section 7.2.3.4.

### 7.3.4 Saving a Received E-Mail Message

You may need to save an e-mail as a document when you are reading an e-mail message.  You can do this by completing the following steps:

1)  Press “space-m (dots 1-3-4)” or “F2” to call up the menu.  Then, the “file” menu will be displayed.

2)  Press “enter.”

3)  Press “space-4” to move to “save as text.”

4)  Press “enter” on “save as text.”  Then, the dialog box will be opened, and the Braille Sense will show “file name? noname.txt.”  You can move to this stage directly by pressing “space-s (dots 2-3-4).”

5)  Press “enter” when “file name? noname.txt” is displayed, or press “tab (space-4-5)” to move to the “confirm” button, and press “enter.”  You can rename the file as explained in section 4.1.4 regarding the “save as” function in the word processor.

6)  You will move back to step 1 (as described above) automatically if the e-mail is saved.

The text file will be saved in “flashdisk/work” with the file name “noname.txt.”  You cannot change the file format.  You can read the saved text file in the word processor.

In order to cancel “save as text,” press “tab (space-4-5)” at step 5 one more time to move to “cancel,” and then press “enter.”  Or, just press “space-e (dots 1-5)” or “space-z (dots 1-3-5-6).”

### 7.3.5 Sending E-Mail Saved in the Storage Box

### There are two kinds of e-mail messages in the “storage box.”

1)  The e-mail message that the user saved, as explained in section 7.2.3.4 (Saving E-mail as Text, and to storage box).

2)  The e-mail that was not sent due to a network error or another reason.

The e-mail saved in the “storage box” can be sent again using the steps below:

1)  Press “space-m (dots 1-3-4)” or “F2” in the “inbox” to call up the “e-mail” menu.

2)  Press “space-4” to move to the “message” menu.

3)  Press “enter.”  Then, “reply” will be displayed.

4)  Press “space-4” to move to “send mail in storage box.”

5)  Press “enter.”  Then, the e-mail titles will be displayed that are saved in the “storage box.”  You can jump to this stage by pressing “enter-s (dots 2-3-4).”

6)  Press “space-4” or “space-1” to move to the e-mail that you want to send again.  Then, you can send it with the same method as explained in step 14 in section 7.2.3 (Sending E-mail).

The e-mail in the storage box consists of “title,” “recipient,” “carbon copy recipient,” “hidden recipient,” “subject,” “message,” and “attach” items.  “Attach” will be displayed if a file is attached to this e-mail.  You can move from one item to the next by pressing “tab (space-4-5)” and “shift-tab (space-1-2).”

You can move from one e-mail to the next e-mail by pressing “space-4” or “space-1.”

The e-mail message saved in the “storage box” can also be modified.  In step 5 above, press “tab (space-4-5)” when “title” is displayed to move to the desired item, such as “recipient” or “message,” etc.  Then, you can modify the email message, but the “title” cannot be modified.

In order to exit from “send mail in storage box,” press “space-e (dots 1-5)” or “space-z (dots 1-3-5-6).”  Then, the Braille Sense will return to “inbox.”

### 7.3.6 Printing E-Mail Messages

You may want to print an email that you received in using either an embosser or an ink printer.

In order to print to an ink printer, do the following:

1)  Connect the Braille Sense to an ink printer with the printer cable that was provided by the manufacturer of the printer.

The printing port must be the same port that was set in the “print setting” in the “option menu.”

2)  Press “space-m (dots 1-3-4)” or “F2” to call up the menu.  Then, the “file” menu will be displayed.

3)  Press “enter.”

4)  Press “space-4” to move to “print e-mail.”

5)  Press “enter.”  Then, the dialog box will be opened, and “Ink print” is displayed.  If you press “space-p (dots 1-2-3-4)” you will be taken directly to this stage.

6)  Press “enter” on “Ink print.”  Or, press “tab (space-4-5)” to move to the “confirm” button, and then press “enter.”  Then, the selected e-mail will be printed through an ink printer.

If you want to print through a Braille printer (embosser), press “space” at step 5 above to change “ink print” to “Braille embosser,” and then press “enter.”

If you want to cancel printing, press “tab (space-4-5)” until the “cancel” button is displayed, and then press “enter.”  Or, you can press “space-e (dots 1-5)” or “space-z (dots 1-3-5-6).”

## 7.4 Edit Menu

The “edit” menu contains the following items:  “select all,” “start selection,” “copy,” “paste,” “cut,” “delete,” “move to unread message,” “find,” “find again,” “copy to folder,” and “move to folder.”   To access the “edit” menu, press “space-m (dots 1-3-4)” or “F2.”  Then, use “space-4” to move to the “edit” menu, and press “enter” on “edit.”

### 7.4.1 Select All

The “select all” function will allow you to create a selection that will automatically select everything that can be selected where you are in the email program.  This function will allow you to select text, as well as e-mail messages.  To activate this function, press “space-m (dots 1-3-4)” or “F2.”  Then, you can use “space-4” to go to the “edit” menu, and press “enter.”  Or, you can press “enter-a (dot 1)” to activate this function without calling up the menu.

### 7.4.2 Start Selection

The “start selection” function allows you to select a certain portion of text that you would like to edit.  When you use the “start selection” function, the cursor will appear as blinking “dots 1-2-3-4-5-6-7-8.”  The “start selection” function allows you to make changes to only the text that is located within the selection, and the changes will not affect text outside of the selection.  The “start selection” function is especially useful for copying, cutting, and pasting.  You can also use the “start selection” function to create a selection of e-mail messages.  You can make a selection in a document if you place the cursor on the location where you want to start the selection of text.  If you want to use this function, press “space-m (dots 1-3-4)” or “F2” to open the “edit” menu, and then press “enter” on “start selection,” or you can press “b (“dots1-2”)” in the “edit” menu to get to the function.  While editing a document, pressing “enter-b (dots 1-2)” will activate this function.  The end point of the selection is where you place the cursor after setting the starting point.  For example, if you have typed the word, “mother” in a document, and you want to select the word mother, you should place the cursor before the letter “m,” and then place the cursor directly after the letter “r.”  Then, you will have selected the entire word.

### 7.4.3 Copy

The “copy” function is only available when you have an email open.  After making a selection in an email, press “space-m (dots 1-3-4)” or “F2” to open the menu, and move to “edit” by pressing “space-4.”  Press “enter” to open the “edit” menu.  Press “space-4” to move to “copy” and then press “enter,” or press “c (dots 1-4)” in the “edit” menu.  While editing a document, pressing “enter-c (dots 1-4)” will activate this function.

If you copy the selection, it will be copied to the clipboard.  The clipboard is the virtual space that is used for storing copied data temporarily.  However, if you decide to copy another selection, the data in the clipboard that you copied before will be erased, and the selection that you have just copied will replace the selection that was on the clipboard.

### 7.4.4 Paste

The “paste” function is only available when you have an email open.  To paste data on the clipboard, press “space-m (dots 1-3-4)” or “F2,” and then press “enter” on “edit” to enter into the “edit” menu.  Move to “paste” by pressing “space-4” repeatedly and then press “enter” on “paste,” or press “v (dots 1-2-3-6)” in the “edit” menu (which is the short cut) to paste the selection.  While editing an email, press “enter-v (dots 1-2-3-6)” to activate this function.

### 7.4.5 Cut

The “cut” function is only available when you have an email open.  To cut a selection, press “space-m (dots 1-3-4)” or “F2,” and press “enter” on “edit” to enter into the “edit” menu.  Move to “cut” by pressing “space-4” repeatedly, and then press “enter” on “cut,” or press “x (dots 1-3-4-6)” in the “edit” menu (which is the short cut) to cut your selection.  While editing a document, pressing “enter-x (dots 1-3-4-6)” will activate the “cut” function.

The difference between “cut” and “copy” is that if you execute the “cut” function, the data of the selection will not be moved to the clipboard and the selection in the document will be erased.  However, the “copy” function will copy the selection to the clipboard, and the text in the selection will not be erased from the document.

### 7.4.6 Delete

To delete a selection or email, press “space-m (dots 1-3-4)” or “F2,” and then press “enter” on “edit” to enter into the “edit” menu.  Move to “delete” by pressing “space-4” repeatedly, and then press “enter” on “delete,” or press “d (dots 1-4-5)” (as a short cut) in the “edit” menu to delete the selection.  When you are editing an email, you can press “space-d (dots 1-4-5)” to activate this function.

### 7.4.7 Move to Unread Message

The “move to unread message” option allows you to move to the next message that is unread.  If you use this function on the last unread message in a list, you will be sent to the first unread message in the list.  To activate this function while you are in the “title,” “date,” or “from” control, open the menu by pressing “space-m (dots 1-3-4)” or “F2.”  Then, move to the “edit” menu by pressing “space-4” or “space-1.”  Then, press “enter” on “edit.”  Or, you can press “u (dots 1-3-6)” while you are in the “edit” menu.  To use this function without calling up the menu, press “enter-u (dots 1-3-6).”

### 7.4.8 Find

The “find” command is used to find text in the current mailbox.  Once you enter text to find, the “find” function will look for text that matches in “title,” “date,” and/or “from.”  Find only works in “title,” “date,” or “from” control.

The “find” dialog box consists of a “find string?” edit box for text to search, a “region” radio button, a “direction” radio button, a “confirm” button, and a “cancel” button.  In the “find” dialog box, you can move between the five controls by pressing “tab (“space-4-5”)” or “shift-tab (“space-1-2”).”  The shortcut key is “f (dots 1-2-4).”  You can access this function without calling up the menu by pressing “space-f (dots 1-2-4).”

To activate the “find” function, use the following steps:

- Open the menu by pressing “space-m (dots 1-3-4)” or “F2,” and move to the “edit” menu by pressing “space-4.”  Then open the “edit” menu by pressing “enter” on “edit.”

- Press “space-4” or “space-1” to navigate to “find,” and then press “enter,” or press the shortcut key, “f (dots 1-2-4).”  Than find dialog box appear.   When the find dialog box appears, the focus is on the “find string” edit box where the user can type the text that they want to find.

- After typing in the text, press “tab (“space-4-5”)” to move to the next control, “region” radio button.  Here you can select between “all,” “title,” “date,” or “from.”  The “all” control means “title,” “date,” and “from” controls together.  You can select an item by pressing “space-4” or “space-1.”  By selecting a specific control, you can search the control for the text that you want to find.

- Pressing “tab (“space-4-5”)” moves to the “direction” radio button, which has two choices, “next” or “previous.”  Pressing “space-4” or “space-1” lets you select the searching direction as “next” or “previous.”

- Press “tab (“space-4-5”)” to move you to the “confirm” button.  Then, press “enter” to execute the function.  If you want to cancel, press “tab (“space-4-5”)” to move to the “cancel” button.  Then, press “enter.”  Or, you can press “space-z (dots 1-3-5-6)” or “space-e (dots 1-5)” anywhere in this dialog box to cancel.

### 7.4.9 Find Again

The “find again” command finds the next text according to what you found using “find” dialog box, and it will go to the previous text if you set the direction to “previous” when you used the “find” function before.   The “find again” function only works in the “title,” “date,” or “from” control.

To activate the “find again” function, use the following steps:

- Open the menu by pressing “space-m (dots 1-3-4) or “F2.”

- Move to the “edit” menu by pressing “space-4,” and press “enter” on “edit.”  Then, navigate to “find again” by pressing “space-4” or “space-1.”  Then, press “enter” on “find again.”  Under the “edit” menu, the shortcut key is “g (dots 1-2-4-5).”  To activate this function without calling up the menu, press “enter-g (dots 1-2-4).”

### 7.4.10 Copy to Folder

The “copy to folder” command copies e-mail messages from one email folder to another email folder.  It does not allow you to copy e-mail messages to a folder that you would find in the “file manager.”  For example, let’s suppose you have the email folder, “inbox,” and a folder you created called “important.”  The “copy to folder” function would allow you to copy an email from the “inbox” to the “important” email folder.  Refer to section 7.2.4 for an explanation on how to navigate between email folders.

The dialog box consists of 6 controls: “folder list,” “create folder” button, “delete folder” button, “rename folder” button, a “confirm” button, and a “cancel” button.  You can move between the controls by pressing “tab (“space-4-5”)” or “shift-tab (“space-1-2”).”  If you want to cancel while you are in these dialog boxes, press “space-e (dots 1-5)” or “space-z (dots 1-3-5-6).”

Use the following steps to copy an email to a folder:

- Select the mail you want to copy by pressing “space,” or create a selection of e-mail messages by pressing “enter-b (dots 1-2).”

- Go to the menu by pressing “space-m (dots 1-3-4)” or “F2,” and press “space-4” or “space-1” to move to the “edit” menu.  Then, press “enter” to get in to the “edit” menu.  Move to “copy to folder,” and then press “enter,” which will open a dialog box.  The shortcut key for “copy to folder” is “p (dots 1-2-3-4)” while you are in the “edit” menu.  To activate the “copy to folder” function without calling up the menu, press “backspace-c (dots 1-4).”

- Select an email folder to copy e-mail messages to in the “folder” list.  Then, press “enter,” or move to the “confirm” button, and press “enter.”  This will copy e-mail messages to the selected email folder in the “folder list.”  However, the focus returns to the email folder where you copied the e-mail messages from.

In the dialog box, pressing “enter” on the “create folder” button opens a new dialog box.  This dialog box consists of 3 controls:  “folder name,” a “confirm” button, and a “cancel” button.  Here you can type in a name for the new email folder in the “folder name” control.  Then, press “enter” to create a new email folder, or move to the “confirm” button by pressing “tab (“space-4-5”),” and press “enter” to create a new email folder.

Pressing “enter” on the “delete folder” button deletes the selected email folder in the “folder list” control.  Before the email folder is deleted, the Braille Sense asks for confirmation.  You can toggle between “yes” and “no” by pressing “space.”  Then, press “enter” to delete the email folder.

The “inbox” and “outbox” are protected from deletion, so attempting to delete them will not work.  If you attempt to delete the “inbox” or the “outbox,” you will hear the warning sound to notify you of the error.  In the “folder list” in the “copy to folder” dialog box, the hot key for “delete folder” is “space-d (dots 1-4-5).

Like the “create folder” button, pressing the “rename folder” button opens a dialog box which has the same controls as the “create folder” dialog box.  However, the “folder name” control shows the name of the folder that was selected from the “folder list.”  Here you can type in the name that you want to rename the folder with.  After editing the folder name control, press “enter,” or move to the “confirm” button, and press “enter.”

### 7.4.11 Move to Folder

The “move to folder” function moves e-mail messages from one email folder to another email folder.  It does not allow you to move e-mail messages to a folder that you would find in the “file manager.”  For example, let’s suppose you have the email folder, “inbox,” and a folder you created called “important.”  The “move to folder” function would allow you to move an email from the “inbox” to the “important” email folder.  Refer to section 7.2.4 for an explanation on how to navigate between email folders.

The dialog box consists of 6 controls: “folder list,” “create folder” button, “delete folder” button, “rename folder” button, a “confirm” button, and a “cancel” button.  You can move between the controls by pressing “tab (“space-4-5”)” or “shift-tab (“space-1-2”).”  If you want to cancel while you are in these dialog boxes, press “space-e (dots 1-5)” or “space-z (dots 1-3-5-6).”

Use the following steps to move an email to a folder:

- Select the mail you want to copy by pressing

“space,” or create a selection of e-mail messages by pressing “enter-b (dots 1-2).”

- Go to the menu by pressing “space-m (dots 1-3-4)” or “F2,” and press “space-4” or “space-1” to move to the “edit” menu.  Then, press “enter” to get in to the “edit” menu.  Move to “copy to folder,” and then press “enter,” which will open a dialog box.  The shortcut key for “copy to folder” is “p (dots 1-2-3-4)” while you are in the “edit” menu.  To activate the “copy to folder” function without calling up the menu, press “backspace-c (dots 1-4).”

- Select an email folder to copy e-mail messages to in the “folder” list.  Then, press “enter,” or move to the “confirm” button, and press “enter.”  This will copy e-mail messages to the selected email folder in the “folder list.”  However, the focus returns to the email folder where you copied the e-mail messages from.

In the dialog box, pressing “enter” on the “create folder” button opens a new dialog box.  This dialog box consists of 3 controls:  “folder name,” a “confirm” button, and a “cancel” button.  You can type in a name for the new email folder in the “folder name” control.  Then, press “enter” to create a new email folder, or move to the “confirm” button by pressing “tab (“space-4-5”),” and press “enter” to create a new email folder.

Pressing “enter” on the “delete folder” button deletes the selected email folder in the “folder list” control.  Before the email folder is deleted, the Braille Sense asks for confirmation.  You can toggle between “yes” and “no” by pressing “space.”  Then, press “enter” to delete the email folder.

The “inbox” and “outbox” are protected from deletion, so attempting to delete them will not work.  If you attempt to delete the “inbox” or the “outbox,” you will hear the warning sound to notify you of the error.  In the “folder list” in the “copy to folder” dialog box, the hot key for “delete folder” is “space-d (dots 1-4-5).”

Like the “create folder” button, pressing the “rename folder” button opens a dialog box which has the same controls as the “create folder” dialog box.  However, the “folder name” control shows the name of the folder that was selected from the “folder list.”  Here you should type in the name that you want to rename the folder with.  After editing the folder name control, press “enter,” or move to the “confirm” button, and press “enter.”

## 7.5 Settings in the Tool Menu

You can change the download path or set the e-mail service using the “tool” menu.

The “tool” menu has the following four sub menus:

1)  Service manager

2)  Set path

3)  Set options

4)  Set spam

Each sub menu will be explained in detail in this section.

### 7.5.1 Service Manager

“Service manager” is used to register, delete, or modify an e-mail service.

The following are the steps required to execute this sub menu.

1)  Press “space-m (dot 1-3-4)” or “F2” to call up the “e-mail” menu.

2)  Press “space-4” to move to the “tool” menu.

3)  Press “enter.”  Then, the “service manager” will be displayed.

4)  Press “enter.”  Then, “service name: (registered service name) 1/yy” will be displayed.  You can move to this stage directly by pressing “enter-m (dots 1-3-4).”  Then, the Braille Sense will display the first item of the “service manager” that is in “service list.”

The “service manager” contains the following 6 items.

1)  Service list

2)  Information

3)  Register

4)  Modify

5)  Delete

6)  Close

You can move among these items by pressing “tab (space-4-5)” or “shift-tab (space-1-2).”  You can execute each item by pressing “enter” on the desired item that is displayed.

If you want to exit from the “service manager,” move to the “close” button, and then press “enter.”  Or, press “space-e (dots 1-5)” or “space-z (dots 1-3-5-6).”

The following is a detailed explanation of each item.  “Register” was already explained in section 7.1.2.  Please refer to that section for information about “register.”

#### 7.5.1.1 Signature and Advanced

If you activate the "service manager," you will see "service name: your registered service name 1/xx" in the list.  While registering your account, you can use "signature" to add a signature to all e-mail messages that you write, and "advanced" setting to changed advanced options.

- Just as in Section 7.1.2, "registering e-mail service," when you are registering the "smtp address" in step 12.  Then, you can press “tab (“space-4-5”)” to go to the "advanced" button.

- Press “enter” on the button.

- You will see "pop3 port number? 110" by default.  Here you can type in another pop3 port number in the computer edit box.

- Press “space-4” to go to "smtp port number? 25."  This is also a computer edit box.  Here you can type in another smtp port number.

- Press “space-4” to go to "smtp ID."  Here, you can add another smtp ID if your Internet service provider requires you to use a different smtp ID.  You should use ASCII to type this in.

- Press “space-4” to go to "smtp password."  You can type in your password that corresponds to the above ID.  It should be typed in using ASCII.

- Press “space-4” to go to "using default smtp? no."  If you want to use the above new username and password that were set in steps 5 and 6, you should press “space” to choose "yes."  Otherwise, leave it at "no."

- Press “tab (“space-4-5”)” to go to "using signature? no."  If you want to use a signature, press “space” choose "yes," or if you do not want a signature, leave it at "no."  Step 9 will show you how to setup a signature if you want to use one.

- Press “tab (“space-4-5”)” to go to the "signature" button.  Press “enter,” and you will see the “signature” multi-edit box.  You can type your signature in here that you want to use.  Press “tab (“space-4-5”),” and press “enter” on "confirm."  This will save the signature, and you will be returned to the "signature" button.  You can cancel by pressing “enter” on "cancel" by pressing “tab (“space-4-5”)” after typing-in a signature.

- Press “tab (“space-4-5”), and press “enter” on "confirm."

#### 7.5.1.2 Checking Registered Service List

The first item of the “service manager” is the “service list.”  It is displayed as “service name: (registered service name) 1/yy.”

The following is the method to check the registered service information:

1)  Press “space-4” or “space-1” to move to the desired service in the service list.

2)  If the desired service is displayed, press “enter” or “tab (space-4-5)” to move to the “information” button, and then press “enter.”

3)  Then, the information dialog box will be opened, and “service name: (Registered service name)” will be displayed (Ex: service name:  Braille Sense).

4)  Press “space-4” or “tab (“space-4-5”)” and make sure the information is correct for each item.  Pressing “enter” on the “close” button will return you to the service list.

Make sure the information is correct for the selected service.

The move keys are listed below to move among the items in the dialog box:

Move to the previous item:  Space-1

Move to the next item:  Space-4

Move to the first item:  Space-1-3 or space-1-2-3

Move to the last item:  Space-4-6 or space-4-5-6

Move to the previous control:  Space-1-2

Move to the next control:  Space-4-5

If the information displayed exceeds 32 Braille cells, the remaining parts can be checked by pressing the “right scroll button.”

#### 7.5.1.3 Modify

If the “service manager” is executed, “service name: (Registered service name) 1/yy” will be displayed in the “service list.”  The user can modify the desired information by using the following steps:

1)  Press “space-4” or “space-1” to move to the desired service in the service list.

2)  When the desired service is displayed, press “tab (space-4-5)” to move to the “modify” button, and then press “enter.”  Or, just press “m (dots 1-3-4).”

3)  Then, the same message will appear that was explained in step 7 in section 7.1.2 (Registering E-mail Service).  But, in this case, it is a “modification,” so the Braille Sense will show you the current information.  For example, “service name? Braille Sense.”

4)  Please follow step 7 through step 13 in section 7.1.2.

5)  When the modification is complete, it will return to the “service list.”

You can move among the items (service name, ID, password…) by pressing “space-1” or “space-4.”  You can move to the “confirm” button or the “cancel” button by pressing “tab (space-4-5)” or “shift-tab (space-1-2).”

On all of the items, the values that were typed in already will be displayed.  If you type in new information, the new information will replace the old information.  If you want to modify part of an item, press the “cursor routing key” at the desired position and type in the new information.

If you made a typo in an item (service name, ID, password…), you can modify it using the following process:

1)  You can delete the current letter by pressing “space-d (dots 1-4-5).”

2)  Press “backspace” in order to delete the previous letter.

3)  In order to delete the current line, press “backspace-1-4.”  Then, “delete line? yes” will be displayed.  Press “enter” in order to delete the line.  Otherwise, press “space” to change from “yes” to “no,” and then press “enter.”

#### 7.5.1.4 Deleting the Registered Service

If the “service manager” is launched, the Braille Sense will show the “service list” in the form of “service name: (Registered service name) 1/yy.”

You can delete the service that you want to delete using the following process:

1)  Use “space-4” or “space-1” to move to the desired service in the “service list.”

2)  When the desired service is displayed, press “space-d (dots 1-4-5)” or “d (dots 1-4-5).”  Or, press “tab (space-4-5)” to move to the “delete” button, and then press “enter.”

3)  Then, “delete (Registered service name) service? Yes” will be displayed.

6)  Press “enter” in order to delete it.

7)  The Braille Sense will delete it, show “completed deletion,” and return to the “service list.”

If you want to cancel the deletion, press “space” when “delete (Registered service name) service? Yes” is displayed in order to change “yes” to “no,” and then press “enter.”  Or, just press “space-e (dots 1-5)” or “space-z (dots 1-3-5-6).”

### 7.5.2 Set Path

“Set path” sets the default download path where file attachments and email are stored.  You can set the download path using the following method:

1)  Press “space-m (dots 1-3-4)” or “F2” to call up the “e-mail” menu in the “Inbox.”

2)  Press “space-4” to move to the “tool” menu.

3)  Press “enter.”  Then, “service manager” will be opened.

4)  Press “space-4” to move to “set path.”

5)  Press “enter” on “set path.”  Then “download path: /flashdisk/download” will be displayed.  You can move to this stage by pressing “enter-p (dots 1-2-3-4).”

“Set path” contains the following three items:

1)  Download path

2)  Upload path

3)  Email download disk

You can move among these items by pressing “space-4” and “space-1.”  Each item consists of “path,” “modify,” and “close.”  At “path,” the default path will be displayed.  You can move among this information by pressing “tab (space-4-5)” or “shift-tab (space-1-2).”

“Set path” will be closed if you press “space-e (dots 1-5)” or “space-z (dots 1-3-5-6).”  Or, you can close it by pressing enter on the “close” button.

#### 7.5.2.1 Set Download Path

The default path for downloaded attached files is “/flashdisk/download.”

You can set the download path using the following method:

1)  Execute steps 1 through 5, as explained in section 7.4.2 (Set path).

2)  Press “enter” on “download path: /flashdisk/download,” or press “m (dots 1-3-4).”  Or, press “tab (space-4-5)” to move to the “modify” button, and then press “enter.”

3)  Then, “no list” will be displayed when the “/flashdisk/download” folder is empty.  But, if the folder has a lower level folder, “(Folder name) 1/yy” will be displayed.  When “set download path” is executed, the Braille Sense will only show the folder.  So, if there is no folder in the “download” folder, it will show “no list.”

4)  Move to the desired folder by pressing move keys below:

Move to the upper level folder:  Backspace

Move to the previous item:  Space-1

Move to the next item:  Space-4

5)  When you encounter the desired folder, press “space” to select it, and then press “enter.”  If a folder is selected, “*” will be displayed in front of the selected folder.

If you press “enter” to set the path, the Braille Sense will show the changed path.  If you changed the path to the “work” folder that is one of the lower level folders located in “flashdisk,” “download path: /flashdisk/work” would be displayed.

When you download an attached file, if you press “enter” while “download path: /flashdisk/download exchange? yes” is displayed, this creates the same situation as step 3 above.  Please follow the above remaining steps.  In this case, if you select a folder, and press “enter,” the attached file will be downloaded into the selected folder and return to the “inbox.”

#### 7.5.2.2 Set Upload Path

“Upload path” is the path where the files to be attached are saved.  The default path is “/flashdisk/upload.”  You can change this path to a different path using the following method:

1)  Execute steps 1 – 5, as explained in section 7.5.2.

2)  Press “space-4” to move to “upload: /flashdisk/upload.”

3)  Press “enter” or “m (dots 1-3-4)” on “upload path: /flashdisk/upload.”  Or, press “tab (space-4-5)” to move to the “modify” button and then press “enter.”

4)  Then, “no list” will be displayed when the “/flashdisk/upload folder is empty.  But, if the folder has a lower level folder, “(Folder name) 1/yy” will be displayed.  When “set upload path” is executed, the Braille Sense will only show the folder.  So, if there are no folders in the “upload” folder, it will show “no list.”

5)  Move to the desired folder by pressing the following keys:

Move to the upper level folder:  Backspace

Move to previous item:  Space-1

Move to next item:  Space-4

6) Press “space” to select the desired folder, and press “enter.”  “*” will be displayed in front of the selected folder.

If you finish setting the path by pressing “enter,” the Braille Sense will show the changed path.  If you changed the path from the “upload” folder to the “music” folder in the “flashdisk,” it would display “download path: /flashdisk/music.”

#### 7.5.2.3 Set E-Mail Download Disk

The e-mail messages that are downloaded from your network are saved in the “email” folder, which is a lower level folder of “flashdisk.”  If the size of an email you are receiving is so large that it cannot be downloaded and stored in the “flashdisk,” you can change to a different disk using the following method:

1)  Connect the external storage disc to the Braille Sense.

2)  Execute steps 1 – 5, as explained in section 7.5.2.

3)  Press “space-4” to move to “email download disk: /flashdisk.”

4)  Press “enter” or “m (dots 1-3-4)” on “email download disk: /flashdisk.”  Or, press “tab (space-4-5)” to move to the “modify” button, and then press “enter.”

5)  Then, the “flashdisk” will be displayed.  Press “space-4” or “space-1” to move to the disk list.  If you have connected a USB memory stick or CF memory, “smd" (stick memory) or “cfd" (cf memory) will be displayed during step one above, respectively.

6)  Press “enter” on the desired disk name.

It will show the disk name if the disc setting is completed by pressing “enter.”  If you changed it to “CF memory,” it will show “email download disk: /cfd.”  If it is changed, the “email” folder will be created automatically in the selected disc, and the e-mail messages will be downloaded in this disk.

### 7.5.3 Set Options

When you download e-mail from your e-mail server, you can set the Braille Sense to keep e-mail messages on the server or not.  In the “set options,” you can select it.

The default value is set to keep your e-mail on the mail server.

You can change this setting by using the following method:

1)  Press “space-m (dots 1-3-4)” or “F2” to call up the “e-mail” menu in the “inbox.”

2)  Move to the “tool” menu by pressing “space-4.”

3)  Press “enter.”  Then, “service manager” will be displayed.

4)  Press “space-4” to move to “set options.”

5)  Press “enter.”  Then, “save mail copy in server? Yes” will be displayed.  Press “enter-o (dots 1-3-5)” in order to jump to this stage.

6)  Press “space” to change “yes” to “no,” and press “enter” if you do not want a copy of the email to remain on the e-mail server.

By using the setting above, the new e-mail message will not be on the e-mail server if you downloaded the e-mail to the Braille Sense.  If an e-mail is not downloaded into the Braille Sense successfully, that e-mail will remain on the e-mail server.  If you want the copy to remain on the e-mail server, you can change the setting from “no” to “yes” by pressing “enter” at step 5 above.

You can cancel “set option” while you are changing a setting by pressing “space-e (dots 1-5)” or “space-z (dots 1-3-5-6).”

### 7.5.4 Set Spam

When the Braille Sense downloads e-mail from the e-mail server, there may be a lot of e-mail messages that are spam.  In this case, you can avoid downloading these spam e-mail messages by registering specified words in the title or the specified e-mail address.

The spam e-mail messages that are set at “set spam” will not be downloaded into the Braille Sense, and will remain on the e-mail server.

Use the following steps to set up spam e-mail:

1)  Press “space-m (dots 1-3-4)” or “F2” to call up the menu while you are in the “inbox.”

2)  Press “space-4” to move to the “tool” menu.

3)  Press “enter.”  Then, “service manager” will be opened.

4)  Press “space-4” to move to “set spam.”

5)  Press “enter.”  Then, “type: subject 1/3” will be displayed.  You can also jump to this stage by pressing “enter-e (dots 1-5).”

Now, you are in the “set spam” menu.

“Set spam” contains the following 6 items;

1)  Type

2)  List

3)  Add

4)  Modify

5)  Delete

6)  Close

You can move among the items by pressing “tab (space-4-5)” or “shift-tab (space-1-2).”

In order to exit from “set spam,” move to the “close” button and press “enter,” or press “space-e (dots 1-5)” or “space-z (dots 1-3-5-6).”

“Type” consists of 3 sub items; “subject,” “ID,” and “host.”  You can move among these sub items by pressing “space-1” or “space-4.”

The “list” will be displayed in a different form according to the sub item of the “type.”

If you have selected “subject,” the list will be displayed as “subject: (Registered word) 1/xx.”  If you selected “ID,” it will be displayed as “id: (Registered id) 1/xx.”  It will be displayed as “no list 0/0” if there is no registered word or ID.

Now we will explain how to register, modify, and delete the spam settings.

#### 7.5.4.1 Add As Spam E-Mail

First, we will explain how to set the e-mail program to sort out spam by setting any specified words in the e-mail title.

1)  Execute steps 1 through 5, as explained in section 7.5.4.

2)  Press “a (dot 1)” when “type: subject 1/3” is displayed.  Or, press “tab (space-4-5)” to move to the “add” button, and then press “enter.”

3)  Then, the “add dialog box” will be opened and “subject to add?” will be displayed.

4)  Type in the words, and press “enter.”  Or, press “tab (“space-4-5”)” to move to the “confirm” button, and then press “enter” after you type in the words.

5)  Then, “add spam data completed” will be announced and you will be returned to the “list.” Then, “subject (registered word) 1/1” will be displayed.

If you want to add more words, repeat step 2, and add the words.  If there is more than one registered word, you can check it by pressing “space-1” or “space-4.”

By adding the word as explained above, if there are any e-mail messages that include the words you entered in the title of the message, the email will not be downloaded.

You can also avoid receiving e-mail messages from a specific sender or host by registering it.

The following are the steps to register a sender’s ID:

1)  Execute steps 1 to 5 in section 7.5.4.

2)  Press “space-4” when “type: subject 1/3” is displayed.  Then, “type: id 2/3” will be displayed.

3)  Press “a (dot 1)” or press “tab (space-4-5)” to move to the “add” button, and press “enter.”

4)  Then, the “add dialog box” will be opened, and “id to add (e-mail address)” will be displayed.  The “(e-mail address)” is the sender’s e-mail address that was focused.

5)  Press “enter” on “id to add? (e-mail address).”  Or, press “tab (space-4-5)” to move to the “confirm” button, and then press “enter.”

6)  Then, “add spam data completed” will be announced and “id: (Registered e-mail address) 1/1” will be displayed.

If you set it once, the e-mail messages that will be sent from the registered sender will not be downloaded to the Braille Sense.

At step 5 above, you can add an additional e-mail address by modifying the e-mail address that you have typed in.  If you type in a new e-mail address at “id to add? (e-mail address),” the displayed e-mail address will be erased, and the new e-mail address will be displayed.  But, if you press the “cursor routing key” at the position where you want to modify the displayed address, that address will be modified as you type in the modifications.

If you want to add another e-mail address to the spam sender, repeat the above steps, starting with step 3 above.  Or, when “id: (registered e-mail address) 1/1” is displayed, you can add it by pressing “a (dot 1)” or pressing “tab (space-4-5)” to move to the “add” button, and press “enter.”  You can check the registered e-mail addresses by pressing “space-1” or “space-4.”

If you want to register a specific host as a spam sender, do the following:

1)  Execute steps 1 through 5, as explained in section 7.5.4.

2)  Press “space-4” when “type: subject 1/3” is displayed in order to move to “type: host 3/3.”

3)  Press “a (dot 1)” on “type: host 3/3,” or press “tab (space-4-5)” to move to the “add” button, and then press “enter.”

4)  Then, the “add dialog box” will be opened, and “host to add? (host address)” will be displayed.  Here, “host address” is the host address that was focused on when you executed this command.

5)  Press “enter” on “host to add? (host address).”  Or, press “tab (space-4-5) to move to the “confirm” button, and then press “enter.”

6)  Then, the Braille Sense will say “add spam data completed” and it will show “host: (registered host address) 1/1.”

If you add the host address as above, the Braille Sense will not download the e-mail messages that are sent from the registered host.

At step 5 above, you can add the additional host address by modifying the host address that you have typed in.  If you type in a new host address at “host to add? (host address),” the displayed host address will be erased, and the new host address will be displayed.  But, if you press the “cursor routing key” at the position where you want to modify the displayed address, that address will be modified where you modify it.

If you want to add an additional host address, you can add it by repeating step 3 above.  Or, when “host: (registered host address) 1/1” is displayed, you can add it by pressing “a (dot 1)” or pressing “tab (space-4-5)” to move to the “add” button, and then press “enter.”  You can check the registered host addresses by pressing “space-1” or “space-4.”

When you register a subject, ID, or host as spam e-mail, you may make a mistake while you are typing in a subject, ID, or host.  In this case, you need to modify the subject while typing them in.

Please refer to the following correction methods:

1)  In order to delete the current letter, press “space-d (dots 1-4-5)”

2)  In order to delete the previous letter, press “backspace.”

3)  In order to delete the current line, press “backspace-1-4.”  Then, “delete line? yes” will be displayed.  Press “enter” to delete the line.  Or, if you do not want to delete the line, press “space” to toggle “yes” to “no,” and then press “enter.”

If you want to cancel setting spam at the “add dialog box,” press “tab (space-4-5)” to move to the “cancel” button, and press “enter.”  Or, just press “space-e (dots 1-5)” or “space-z (dots 1-3-5-6).”

#### 7.5.4.2 Modifying Spam Registration

In order to modify information that has been registered you can also use the methods explained in section 7.5.4.1.

In order to modify a “subject” that has been registered, do the following:

1)  Execute steps 1 through 5, as explained in section 7.5.4.

2)  Move to “list” by pressing “tab (space-4-5).”  Then, “subject: (Registered word) 1/xx” will be displayed.

3)  Press “space-4” or “space-1” to move to the word to be modified.

4)  Press “m (dots 1-3-4)” on the word to be modified.  Or, press “tab (space-4-5)” to move to the “modify” button, and press “enter.”

5)  Then, the modify dialog box will be opened, and “subject to modify? (Selected word)” will be displayed.

6)  If you type in a new word directly, the displayed word will be erased, and the new word will be displayed.  If you want to modify a part of the displayed word, press the “cursor routing key” at the position where you want to modify the word, and type in the letters that you want to modify.

7)  Press “enter” after you type in the new word.  Or, press “tab (space-4-5)” to move to the “confirm” button, and then press “enter.”

8)  Then “modify spam data completed” will be announced, and you will be returned to the “list.”  If there is more than one word in the list, you can look them up by pressing “space-4” or “space-1.”

If you want to modify the e-mail addresses or host addresses that have been registered, execute step 1 above.  Press “space-4” to move to “type: id 2/3” and “type: host 3/3.”  You can modify them with the same method as explained above.

If you mistyped any letter while modifying the subject, id, or host, you can correct it, as explained in section 7.5.4.1.

If you want to cancel registration, press “tab (space-4-5)” to move to the “cancel” button, and press “enter.”  Or, you can press “space-e (dots 1-5)” or “space-z (dots 1-3-5-6).”

#### 7.5.4.3 Deleting Spam

You can delete an item that has been registered in the spam list.

Let’s start our explanation from “subject.”

1)  Execute steps 1 through 5, as explained in section 7.5.4.

2)  Press “tab (space-4-5)” to move to the “list.”  Then, “subject: (Registered word) 1/xx” will be displayed.

3)  Press “space-4” or “space-1” to delete the registered word.

4)  Press “space-d (dots 1-4-5)” to delete the selected word.  Or, press “tab (space-4-5)” to move to the “delete” button, and press “enter.”

5)  Then, “completed deletion” will be announced and “list” will show again.  You can check the remaining words by pressing “space-4” or “space-1.”

You may delete an “id” or “host” in the spam list, too.  Execute step 1 above, and press “space-4” to move to “type: id 2/3” or “type: host 3/3,” and execute step 2 above.  The method is the same as explained above.

## 7.6. Hot Keys for E-Mail

### 7.6.1 Hot Keys That Are Used in the Inbox

Call “service manager”:  Enter-m (dots 1-3-4)

Receive new mail:  Enter-n (dots 1-3-4-5)

Write mail:  Enter-w (dots 2-4-5-6)

Reply:  Enter-r (dots 1-2-3-5)

Reply all:  Backspace-r (dots 1-2-3-5)

Forward:  Enter-f (dots 1-2-4)

Call up “send mail in storage box”:  Enter-s (dots 2-3-4)

Call up “set path”:  Enter-p (dots 1-2-3-4)

Call up “set option”:  Enter-o (dots 1-3-5)

Call up “set spam”:  Enter-e (dots 1-5)

Call up “local folder”: Enter-d (dots 1-4-5)

Find: Space-f (dots 1-2-4)

Find again: Backspace-f (dots 1-2-4)

Move to unread message: Enter-u (dots 1-3-4)

Move to folder: Backspace-x (dots 1-3-4-6)

Copy to folder: Backspace-c (dots 1-4)

Add a sender to the address list:  Enter-I (dots 2-4) (This is valid only at the “from” item of the “inbox”).

Delete received e-mail:  Space-d (dots 1-4-5) (It is valid only at the “title” and “date” item in the “inbox”).

Save the received e-mail as text file:  Space-s (dots 2-3-4)

Print the received e-mail:  Space-p (dots 1-2-3-4)

Cancel:  Space-e (dots 1-5)

Cancel and exit from e-mail program:  Space-z (dots 1-3-5-6)

### 7.6.2 Hot Keys for Sending E-Mail Including Reply, Forward, and Save in the Mail Storage Box

Search address list:  Enter-l (dots 1-2-3)

Attach file:  Enter-f (dots 1-2-4)

Send e-mail:  Enter-s (dots 2-3-4)

Save in mail storage box:  Space-s (dots 2-3-4)

Cancel:  Space-e (dots 1-5)

Cancel and exit from e-mail program:  Space-z (dots 1-3-5-6)

### 7.6.3 Hot Keys for Reading E-Mail Messages or Writing an E-mail Message

Start selection:  Enter-b (dots 1-2)

Select all:  Enter-a (dot 1)

Copy to the clipboard:  Enter-c (dots 1-4)

Cut to the clipboard:  Enter-x (dots 1-3-4-6) (It is not valid when you are reading e-mail in the “inbox”).

Paste from the clipboard:  Enter-v (dots 1-2-3-6) (It is not valid when you are reading e-mail in the “inbox”).

Delete:  Space-d (dots 1-2) (It is not valid when you are reading e-mail in the “inbox”).

Cancel block:  Space-e (dots 1-5)