BRAILLE SENSE MANUAL:

(For Software Version 4.0)

SECTION 14 Utility

TABLE OF CONTENTS

14. Utility	 2

14.1 Calculator	 2

14.1.1 General Functions	 3

14.1.2 Variable Calculators	 4

14.1.3 Sine Functions	 6

14.1.4 Cosine Functions	 7

14.1.5 Tangent Functions	 7

14.1.6 Logarithm Function	 7

14.1.7 Copy to Clipboard	 8

14.1.8 Option Setting	 8

14.1.9 Exit	 9

14.1.10 Sample Math Calculations	 9

14.2 Set Date and Time	 12

14.3 Check Date and Time	 13

14.4 Wakeup Call	 13

14.4.1 Set Time	 14

14.4.2 Alarm Sound	 14

14.4.3 Term	 14

14.4.4 Ringing Duration	 15

14.4.5 Repeat Interval	 15

14.4.6 Wakeup Call Times	 15

14.4.7 Confirm Cancel	 15

14.5 Infrared Transmission	 15

14.5.1 Transmit	 16

14.5.2 Option Setting	 16

14.6 Calendar	 17

14.6.1 Using the Calendar 	17

14.7 Pronunciation Dictionary	 18

14.7.1 Add Word	 19

14.7.2 Search Word	 19

14.7.3 Modify Word	 19

14.7.4 Delete Word 	20

14.8 Stopwatch	 20

14.9 Terminal for Screen reader	 21

14.10 Setup Internet	 22

14.10.1 LAN Setup	 22

14.10.2 Modem & Bluetooth DUN setup	 23

14.10.3 ADSL Setup	 24

*If you want to connect the Braille Sense to the Internet via a DSL connection, you should use the ADSL setup.  First, you should plug in a LAN cable to the Braille Sense’s LAN port.  If you press “enter” on “setup internet,” you will first see "CB LAN 1/3."  Then, go to ADSL by pressing “space-4” repeatedly.  Press “enter” on ADSL.  It contains the following controls:  connect list, default, register, modify, delete, and close.  You can move between the controls by pressing “tab (“space-4-5”)” or “shift-tab (“space-1-2”)”.  Within each control, you can move item by item by pressing “space-4” or “space-1.”	 24

14.10.4 Setup Wireless LAN	 25

14.11 Network Status	 28

14.12 Check Power Status	 29

14.13 Backup/Restore Option	 30

14.14 Upgrade Braille sense OS	 31

14.14.1 Upgrading the Braille Sense OS Using

the Internet	31

14.14.2 Upgrading the Braille Sense OS from a Disk 	32

14.15 Hot Keys for Utility	 33

# 14. Utility

The “utility” menu provides several special functions for use with the Braille Sense.  The “utility” menu has the following items:

“Calculator,” (where you can do simple math equations as well as scientific calculations), “set date and time,” (where you can set the Braille Sense internal clock), “check date and time,” (where you can check the date and time), “check power status,” (where you can check the remaining battery power), “setup Internet,” (where you can adjust your Internet settings), “terminal for screen reader,” (which allows you to connect the Braille Sense to your PC for use as a Braille display, external speaker, as well as many other useful functions).  To access the menu items, press “space-1” or “space-4” in the “program menu,” and press “enter” on the “utility” menu.  Or, you can press “u (dots 1-3-6)” in the “program menu” to open the utility menu.  The “utility” menu has several submenus under each of the menu items.   The following sections will discuss each of these menu options and their functions in greater detail.

## 14.1 Calculator

## Using the calculator on the Braille Sense you can do simple math equations as well as complex scientific calculations.  The results and formulas will be displayed in Braille, and spoken by the Braille Sense.  While using the calculator, you can refer to the “help” menu by pressing “space-h (dots 1-2-5).”  The “help” menu is very useful as you can quickly access hot key information for the various programs on the Braille Sense.

When you want to use the calculator you can press “c (dots 1-4)” from within the “utility” menu.  When using the calculator on the Braille Sense, you need to type using computer Braille when calculating or inserting your formulas.  You can initialize your calculation line by pressing “backspace-c (dots 1-4).”  If you insert numbers or a function into a result, the result will disappear; and if you put in operators, you can get to the next step.  In addition, you can keep your result in the calculator’s memory, and you can recall it when you need the result.  You can use the function and operators via the calculator menu or you can use the hot keys depending on which method works best for you.

All of these functions are explained in greater detail below.

### 14.1.1 General Functions

### “General functions” is a dialog box that contains the “basic operator list,” a “confirm” button, and a “cancel” button.  In the dialog box, you can navigate through the items with “tab (“space-4-5”)” and “shift-tab (“space-1-2”)”.  In the operator list, you can navigate the operators using “space-1” or “space-4.”  Or, in the list, you can press the first letter of the operator that you want to go to.  In the “calculator,” you can open the menu by pressing “space-m (dots 1-3-4)” or “F2,” and press “enter” on “general functions.”  Or, you can press “g (dots 1-2-4-5)” in “the” menu to access “general functions.”  After starting the calculation, 0 will be displayed in computer Braille.  On the calculation line, you can access the “general functions” by pressing “enter-g (dots 1-2-4-5).”  If the dialog box pops up first, you will be placed on the operator list.  Once in the list, you can select what you want, and press “enter” on it to run the operator since the default is set to the “confirm” button in the dialog.  In the dialog box, you can cancel the operator selection by pressing “enter” on “cancel,” “space-z (dots 1-3-5-6),” or “space-e (dots 1-5).”  You can use hot keys of the operators for convenient access.

### The hot keys of the operators are as follows:

Plus:  dots 3-4-6

Minus:  dots 3-6

Division:  dots 3-4

Multiplication:  dots 1-6

Percent:  dots 1-4-6

Power:  Backspace-4-5

Square Root:  Enter-q (dots 1-2-3-4-5)

Decimal point:  dots 4-6

Left parenthesis:  dots 1-2-3-5-6

Right parenthesis:  dots 2-3-4-5-6

PI:  Enter-p (dots 1-2-3-4)

Exponential function:  Enter-e (dots 1-5)

If you want to enter in negative numbers, you should type in minus (dots 3-6) and the number together in parentheses.  For example, you should type in minus 2 in the following manner:  (-2).

For the percent operation, you can get the result when you put in the numbers first and the percent sign (%) just after typing in the numbers.  Immediately after you have typed in the percent sign (%), the number that you have typed in will be converted into a percent value.  For example, 90 + 20% equals 90 + 18.  You will see the number 18 on pressing the % sign since 18 is 20% of 90.   So, you get 108 for your answer.

### 14.1.2 Variable Calculators

### The “variable calculators” will keep your resulting answers and calculations in the memory.  The “variable calculators” menu has four submenu items:  “recall item,” “delete all saved items,” “save item,” and “statistics functions.”  You can move between these items by pressing “space-4” or “space-1” repeatedly, and then press “enter” on the desired feature.  To access these menu items, press “space-m (dots 1-3-4)” to call up the menu, and press “enter” on “variable calculators.”  Or, you can press “v (dots 1-2-3-6)” when you are in the menu.

#### 14.1.2.1 Recall Items

#### “Recall items” is a dialog box, that has the following items “list,” “delete,” “display detail,” a ”confirm” button, and a “cancel” button.  You can navigate through each of these items by pressing tab (“space-4-5”) or shift-tab (“space-1-2”).  You can access “recall items” by pressing “r (dots 1-2-3-5)” in the “variable calculators” menu.  Otherwise, you can access the dialog box by pressing “backspace-r (dots 1-2-3-5)” in the calculation line.  The “item list” displays “an item name: the result value,” and you can move in the item name list by pressing “space-1” or “space-4.”  You can insert the item’s result into your calculation if you press “enter” on the item name.  You can also copy each item in the “item list.”  Press the “space” key to select an item in the “item list.”  Then, press “enter-i (dots 2-4)” to copy the selected items to the clipboard.  The “display detail” shows the item name and its result with the default set to “no.”  If you want to review the detailed information, press “space” to switch to “yes.”  Then, it will display "item name: the formula = its result value" in Braille.  “Delete” is used to get rid of your registered item.  If you want to delete an item, first, you should select the item that you want to be deleted, and then press “enter” on “delete.”  You can find the deleted item in “item list” by pressing tab (“space-4-5”).  Or, you can press “space-d (dots 1-4-5)” on what you want to delete in the item name list.  If you want to delete all of the items, you can select all of them by pressing “enter-a (dot 1),” and press “space-d (dots 1-4-5).”  If you only want to delete some of them, you can select the items by pressing “space,” and press “space-d (dots 1-4-5)” on the selected items.

#### 14.1.2.2 Delete All Saved Items

When you use this function, you can delete all of the saved items that are stored in the “recall items.”  To activate it, you can call up the menu by pressing “space-m (dots 1-3-4)” or press “F2.”  Then, use the up and down arrows to get to the “variable calculators” menu, and press “enter.”  Next, use the up and down arrows to go to “delete all saved items,” and press “enter.”  You can also press “enter-d (dots

1-4-5)” while you are in the calculation line.

#### 14.1.2.3 Save Items

“Save items” is a dialog box that has an edit window for typing in the item name, a “confirm” button and a “cancel” button.  You can move between the control items by pressing “tab (“space-4-5”)” or “shift-tab (“space-1-2”)”.  In the “variable calculators” menu, you can access “save items” by pressing “s (dots 2-3-5).”  When you are in the calculation line, you can press “backspace-s (dots 2-3-4)” to open the save items dialog box.  When you are on the calculation line if you type in the item name, and formula, “a=2+sin(30),” followed by pressing “enter,” the calculation will be saved automatically.  In your calculating result, you can access “save items” to type in an item name in the edit box, and then you can save it by pressing “enter” directly or by pressing “enter” on the “confirm” button.  If you type in function names like “sin,” “cos,” or “pi” as an item name, you will see the message “invalid item name,” and if you type in names that have already been saved, you will see a message that says, “the item name is an existing item name; do you want to change it?”

#### 14.1.2.4 Statistics Functions

If you select this menu item, a dialog box opens, which consists of a list box containing all the stored variables, a list box displaying the results of several functions, and a “close” button.  You can move between these elements by pressing “tab (space-4-5),” or “shift-tab (space-1-2).”  You can bring up this dialog box by pressing “backspace-t (dots 2-3-4-5)” in the calculator line, or by pressing “t (dots 2-3-4-5)” in the “variable calculators” menu.  In the variable list, the variable names and their current values are displayed.  You can move between these variables by pressing “space-4” or “space-1.”  In order to select a variable to be used in the calculation, press “space” on it.  This list is the same as the one in the “recall items” dialog box.  The only difference is that this dialog box always displays variable names and values, regardless of the value of “detail display” option.  After you select all of the variables to be used in the calculation, you can press “tab (space-4-5)” to move to the “result” list.   The “result” list displays the sum, average, variance, and standard deviation of selected variables.  You can move between these items by pressing “space-4” or “space-1.”  If you want to copy some results in the list to the clipboard, first press “space” on the items to be copied, and then press “enter-I (dots 2-4).”  (However, “copy to clipboard” in the menu cannot be used.  Only this hotkey will work.) Once they are copied to the clipboard, they can be pasted in the word processor or other programs.  To close the dialog box, press “space-z (dots 1-3-5-6)” or press “enter” on the “close” button.  To get the statistics about another set of variables after reading the result, press “shift-tab (space-1-2)” to go back to the variable list.  Then, press “space-e (dots 1-5)” to cancel all the selections.  Now, select all the variables to be calculated in the same way as you did before, and press “tab (space dots-4-5) to read the result.

### 14.1.3 Sine Functions

### The “sine functions” menu will be activated when you press “enter” on “sine functions,” or press “s (dots 2-3-4)” in the menu.  You can open the menu by pressing “space-m (dots 1-3-4).”  Or, you can activate the “sine functions” menu by pressing “enter-s (dots 2-3-4)” in your calculation line.  The “sine function” list consists of “sine,” “arc sine,” and “hyperbolic sine.”  If you press “enter” on one of the three functions while moving with “space-1” or “space-4,” or if you press “enter” on “confirm” on one of the functions while moving with tab (“space-4-5”), you can insert it into your calculation line after selecting one of the sine functions.  Or, you can insert one of the three sine functions by pressing the first letter of each in the list.

### 14.1.4 Cosine Functions

### You can activate the “cosine functions” by pressing “enter” on “cosine function,” or if you press “c (dots 1-4)” in the menu.  You can open the menu by pressing “space-m (dots 1-3-4).  Or, in your calculation line, you can open the “cosine function” by pressing “enter-c (dots 1-4).”  The “cosine function” list consists of “cosine,” “arc cosine,” and “hyperbolic cosine.”  If you press “enter” on one of the three functions while moving with “space-1” or “space-4,” or if you press “enter” on “confirm” while moving with tab (“space-4-5”), you can insert it into your calculation line after selecting one of the three cosine functions.  Or, you can insert one of the three cosine functions by pressing the first letter of each in the list.

### 14.1.5 Tangent Functions

### You can activate the “tangent functions” when you press “enter” on “tangent functions,” or if you press “t (dots 2-3-4-5)” in the menu.  You can also open the menu by pressing “space-m (dots 1-3-4).”  Or, in your calculation line, you can open the “tangent functions” by pressing “enter-t (dots 2-3-4-5).”  The “tangent functions” list consists of “tangent,” “arc tangent,” and “hyperbolic tangent.”  If you press “enter” on one of the three functions while moving with “space-1” or “space-4,” or if you press “enter” on “confirm” on one of the functions while moving with tab (“space-4-5”), you can insert it into your calculation line after selecting one of the three tangent functions.  Or you can insert one of the three tangent functions by pressing the first letter of each in the list.

### 14.1.6 Logarithm Function

### The “logarithm function” is a dialog box, which contains the following:  “function list,” a “confirm” button, and a “cancel” button.  You can move between the three items by pressing “tab (“space-4-5”)” or “shift-tab (“space-1-2”)”.  The function list consists of “common logarithm” and “natural logarithm,” in which you can move by pressing “space-1” or “space-4.”

### You can activate the “logarithm functions” menu by pressing “enter” on “logarithm functions,” or if you press “l (dots 1-2-3)” in the menu.  Of course, you can open the menu by pressing “space-m (dots 1-3-4).”  Or, in your calculation line, you can activate the “logarithm function” by pressing “enter-l (dots 1-2-3).”

### When running the “logarithm functions,” If you press “enter” on one of the functions while moving with “space-1” or “space-4,” or if you press “enter” on “confirm” on one of the functions while moving with “tab (“space-4-5”)”,” you can insert it into your calculation line after selecting one of the logarithm functions.  Or, you can insert “natural logarithm” by pressing “n (dots 1-3-4-5)” in the list.  You can insert “common logarithm” by pressing “c (dots 1-4).”

### 14.1.7 Copy to Clipboard

### The “copy to clipboard” function is used to copy your formula or result into the clipboard for pasting your formula or result in a document of the word processor.  You can activate it by pressing “enter” on the “copy to clipboard,” or by pressing “I (dots 2-4)” in the menu.  You can open the menu by pressing “space-m (dots 1-3-4)” while running the calculator.  Or, in the calculation line, you can press “enter-I (dots 2-4)” for the quick copy of your calculation into the clipboard directly.  After copying, you can paste it in your document of the word processor.

### When you activate “copy to clipboard,” you will hear “copy completed,” and the Braille Sense will return to your calculation line.

### 14.1.8 Option Setting

### “Option setting” is a dialog box, which contains the following:  “setting list,” a “confirm” button, and a “cancel” button.  You can move between the three items by pressing “tab (“space-4-5”)” or “shift-tab (“space-1-2”)”.  The “setting list” consists of “radian” and “degree,” in which you can move by pressing “space-1” or “space-4.”

### If you select “radian,” you can get the PI value while using the trigonometric function.  If you select “degree” and activate PI, you will get a wrong answer.  You can activate the “option setting” if you press “enter” on it, or if you press “o” in the menu.  Of course, you can open the menu by pressing “space-m (dots 1-3-4).  Or, in your calculation line, you can activate the “option setting” by pressing “enter-o (dots 1-3-5).”

### When running the “option setting, if you press “enter” on what you want in the setting list while moving with “space-1” or “space-4,” or if you press “enter” on “confirm” on what you want in the setting list, while moving with “tab (“space-4-5”)”,” you will set the option.

### 14.1.9 Exit

### This is used to exit the calculator.  You can quit a calculation if you press “enter” on “exit,” or if you press “z” in the menu.  Of course, you can open the menu by pressing “space-m (dots 1-3-4).  Or, in your calculation line, you can exit by pressing “space-z (dots 1-3-5-6)”.

### 14.1.10 Sample Math Calculations

The following are examples for each of the math functions that you can perform when using the calculator on the Braille Sense.

General functions:

1. Addition Plus

For Plus press “dots 3-4-6.”

For example, to add the numbers 10 and 20, you would enter: 10+20.

The result is displayed as 30.

2. Subtraction Minus

For Minus, press “dots 3-6.”

For example, to add the numbers 10 and 20, and then subtract 5, you would enter: 10+20-5. The result is displayed as 25.

3. Multiplication

For multiplication, press “dots 1-6.”

For example, to multiply 10 by 20, you would enter: 10*20.

The result is displayed as 200.

4. Division

For Division, press “dots 1-4.”

For example, to divide 20 by 10, you would enter: 20/10.  The result is displayed as 2.

5. Percent

For Percent, “press dots 1-4-6.”

If you want to calculate, what a 10 percent mark-up of 20 would be, enter the following, 20+10% which gives you the result 22.  You get this answer because, 2 is 10percent of 20.

To calculate a 10 percent mark-down on 20, press: 20-10% which gives the result 18.

You get this answer because, 2 is 10 percent of 20.

To calculate what percentage 20 is of 10, press: 20/10% which gives you the result 200.

To calculate 10 percent of 20, press: 20*10% which gives the result 2. That is, 10 percent is 0.1.  20*0.1 = 2.

6. Power

For Power, press “BACKSPACE” with “dots 4-5.”

For example, to calculate 2 to the power of 10, you would press: 2^10.  The answer will be given as 1024.

7  Calculating Square Root

To perform the Square root function, you can press “enter” with “dots 1-2-3-4-5.”

For example, to calculate the square root of 16, you press: Enter with dots 1-2-3-4-5, 16 “)”.  Then, press enter.

Your answer will be given as 4.

8. Exponential

For Exponential you can press “enter” with “dots 1-5.”

For example, to calculate the Exponential of 2, you would press: Enter with dots 1-5 then 2, followed by the “)” and “enter.” That is, exp (2) which gives the result 7.3890561.

9. Trigonometric Functions

Sine Functions:

For Sine, press “Enter” with “dots 2-3-4,” and then “s.” This is shown on the Braille display as, sin(.

For Arc Sine, press “enter” with “dots 2-3-4,” then “a.” This is displayed as asin(.

For Hyperbolic sine, press “enter” with “dots 2-3-4,” then “h.” This is displayed as sinh(.

In order to calculate the sine of a 30 degree angle you would type the following:  Enter with dots 2-3-4, then “s,” 30 and then” ).” Press “enter” and your answer is displayed as

0.5.

Cosine Functions:

For Cosine, you would press “enter with “dots 1-3,” and then you would press “c.” This is displayed as cos(.

For Arc cosine, press “enter,” with “dots 1-3,” then, “a.” This is displayed as acos(.

For Hyperbolic cosine, press “enter” with “dots 1-3,” then, “h.” This is displayed as cosh(.

For example, to calculate the cosine of the 30 degree angles, you press: Enter with dots 1-3, C, 30 and then “)”. Then, press “enter.”  Your answer will be given as

0.8660254.

Tangent Functions:

For Tangent, press Enter with dots 2-3-4-5 and then T. This is displayed as tan(.

For Arc tangent, press Enter with dots 2-3-4-5 then A. This is displayed as atan(.

For Hyperbolic tangent, press “enter,” with “dots 2-3-4-5,” then, “h.” This is displayed as tanh(.

For example, to calculate the tangent of the 30 degree angles, you press: Enter with dots 2-3-4-5, T, 30 and then “)”. That is, tan(30) which gives you the result of 0.57735027.

Logarithm Functions:

Common logarithm

For Common logarithm, press “Enter” with “dots 1-2-3,” and then “n.” This is displayed as log(.

For example, to calculate the common logarithm of the 100, you press: Enter with dots 1-2-3, N, 100 and then “)”. That is, log(100) which gives the result 2.

Natural Logarithm

For Natural logarithm, press “enter with “dots 1-2-3,” and then, “e.” This is displayed as ln(.

For example, to calculate the natural logarithm of the 100, you press: Enter with dots 1-2-3, E, 100 and then “)”. That is, ln(100) which gives the result 4.60517019.

## 14.2 Set Date and Time

## If you want to set the Sense internal clock, you can set it by pressing “enter” on “set date and time” in the “utility” menu.  In the “utility” menu, you can locate the set date and time by pressing “t (dots 2-3-4-5),” or you can find it by navigating the “utility” menu with “space-1” or “space-4.”

## The set date and time is a dialog box, which consists of “time zone,” “time format,” “time,” “date,” a “confirm” button, and a “cancel” button.  You can move between each of these items by pressing “tab (“space-4-5”)” or “shift-tab (“space-1-2”)”.

## If you activate “set date and time” for the first time, you may see “time zone? Pacific Time (US & Canada); Tijuana (GMPT-08:00)” which is set as the default.  In “time zone,” you can review the “time zone” list by pressing “space-4” or “space-1,” or by typing in the first letter of represented cities.  If you selected one of the time zones, you can move to the next item by pressing “tab (“space-4-5”)”,” or you can move to the previous item by pressing “shift-tab (“space-1-2”)”.

## The second item is “time format,” where you can switch between 12 hour and 24 hour time.  You can toggle between which time to use with “space-1” or “space-4.”

## Once you select a time, you can move to the next “time” by pressing “tab (“space-4-5”)”.”  In “time,” you can set your time.  If you select 12 hour clock in the “time format,” you can see A.M. or P.M. after the hour and minute.  In the 12 hour clock, you can set the time after selecting A.M. or P.M. by pressing “space-x (dots 1-3-4-6).”  In the 24 hour clock setting, you can type in a 24-hour-system number regardless of A.M. or P.M.  When you put numbers in the time, you need to put a space between the hour and the minute.  To set your time, you can move to the date by pressing “tab (“space-4-5”)”.”  In the date, you should put in the month, date, and year in that order.  You should use a 2-digit number for the month, a 2-digit number for the date, and a 4-digit number for the year while typing in a space between them.  After you have completed the necessary information, you can press “enter” on “confirm” by moving with “tab (“space-4-5”)”,” and then you will return to “set date and time.”  Or, you can press “enter” when you have completed entering all of the information because the default is set to “confirm.”

## 14.3 Check Date and Time

## You can check the current date and time here.  To activate this function, press “space-4” or “space-1” in the “utility” menu, and then press “enter” on “check date and time.”   Or, you can activate this function by pressing “d” in the menu.  Anywhere in the Braille Sense, you can press “space-t (dots 2-3-4-5)” to check the time.  If you press “space-t (dots 2-3-4-5),” the Braille Sense will say the time, and the Braille Sense will display the current time in Braille.  Then, if you press tab (“space-4-5”), the Braille Sense will say and display today’s date.

## 14.4 Wakeup Call

## If you have set up the “wakeup call” function, an alarm will sound on the Braille Sense at the specified time every day.  If you want to open “wakeup call,” go to the “program” menu, move to “utility,” and then press “enter.”  Then, use the up and down arrows to find “wakeup call,” and then press “enter” on “wakeup call.”  If this is the first time you have opened “wakeup call,” and you have not yet set the time or date in the Braille Sense, the Braille Sense will then say, “set time?”  After you set the time, the Braille Sense will display the “wakeup call” dialog box.  The Braille Sense will then say, “set time?”  The “wakeup call” dialog box contains the following:  “set time,” “bell,” “term,” “ring duration,” “repeat interval,” “wakeup call times.” a “confirm” button, and a “cancel” button.  You can move among these by pressing “tab (“space-4-5”)” and “shift-tab (“space-1-2”)”.  If you want to check the wakeup call settings, open this dialog box again.  It will show you the current wakeup call settings.  When the alarm sounds, the Braille display on the Braille Sense will also display the message “wakeup call.”  If you want to stop the alarm, press “backspace-enter.”  After you press “backspace-enter,” the Braille Sense will remain on, even though the power switch is turned to “off.”  In order to turn the Braille Sense off, you must turn the power switch to “on,” and then back to “off” after you hit “backspace-enter.”  Then, the Braille Sense will be turned off, and the alarm will not sound again until you set it again.  If you do not plan on being able to turn the unit off and on, you should turn on the “power saving mode” in the “option” menu (as explained in section 2.4) to avoid draining the battery.

### 14.4.1 Set Time

### When you open the “wakeup call” dialog box, the Braille Sense will display the “set time” field.  The value is set to the current time or the time specified previously.  Here you can directly type in the time, or set the time using the shortcut keys, as explained in section 6.3.  After you set the time, you should press “tab (“space-4-5”)” to move to the next field.  While you are anywhere in the Braille Sense, you can check the time by pressing “space-t (dots 2-3-4-5)” to check the time.

### 14.4.2 Alarm Sound

### You can select an alarm sound for the wakeup call.  If you press “tab (“space-4-5”)” on “set time,” you will hear the alarm sound that is currently set.  Press “space-1” or “space-4” to change the alarm sound, and you will hear the different alarm sounds.  Select one of the three alarm sounds that are available.  If you have selected the sound that you want, press “tab (“space-4-5”)” to move to the next field.

### 14.4.3 Term

### In this field, you can set which day of the week the alarm will go off.  There are four options: “off,” “once,” “weekday,” and “everyday.”  Move between these options by pressing “space-1” or “space-4.”  If “term” is set to “off,” the alarm will not go off at the specified time at all.  If “term” is set to “once,” the alarm will go off only once.  If “term” is set to “weekday,” the alarm will go off at the same time every weekday, Monday through Friday.  If “term” is set to “everyday,” the alarm will go off at the specified time 7 days a week, even on weekends.  When you have selected the option you like, press “tab (“space-4-5”)” to move to the next field.

### 14.4.4 Ringing Duration

This option sets the amount of time that the alarm will sound.  The options are 1 minute, 2 minutes, or 3 minutes.  Move between these options by pressing “space-1” or “space-4.”  If you set the ringing duration to “1 minute,” the alarm will sound for one minute, and then the alarm will be silent.  To stop the alarm, press “backspace-enter.”

### 14.4.5 Repeat Interval

This option sets the amount of time that will be between the times that the alarm sounds.  For example, after the alarm has stopped sounding, it will ring again in 3 minutes, if you set the repeat interval to three minutes.  The available settings are 3, 5, 10, 15, or 20 minutes.  Move between these options by pressing “space-1” or “space-4.”

### 14.4.6 Wakeup Call Times

This option allows you to decide how many times the alarm will sound.  The available settings are anywhere from 1 to 10 times.  For example, if the alarm is not turned off by pressing “backspace-enter,” it will repeat according to the number of times that you set for this option.  Move between these options by pressing “space-1” or “space-4.”

### 14.4.7 Confirm Cancel

### If you press “enter” on the “confirm” button, the changed options will be saved, and the dialog box will be closed.  If you do not want to save the changes you have made, press “enter” on the “cancel” button.  The Braille Sense will keep the previous “wakeup call” settings, and close the dialog box.

## 14.5 Infrared Transmission

## “Infrared transmission” is a program that allows you to transfer files interactively to another Braille Sense or a computer with an IRDA port.  You can activate this function by pressing “enter” on “infrared transmission” in the “utility” menu.  Or, you can activate this function by pressing “f (dots 1-2-4)” in the “utility” menu.  Via the IRDA port, you can save transferred files in the download folder of your flash disk.

## The “infrared transmission” has two buttons, which are “transmit” and “option setting.”  You can move between the two by pressing “shift-tab (“space-1-2”)” or “tab (“space-4-5).”

### 14.5.1 Transmit

### If you press “enter” on “transmit,” the Braille Sense will try to locate another device for the file transmission.  The device you are trying to send to should have an IRDA port, so that the devices can communicate with each other.   Therefore, the two ports should be faced directly at each other via a short distance (50 cm (about 20 inches) or less).  Before activating “transmit,” you should consider the location and distance between the two ports.  If the Braille Sense failed to find the other IRDA port to communicate with, it will give the message, “target search failure to transmit,” and return to “transmit.”  If you succeeded, you will hear a connection sound, and you will see “file list.”  You can transfer the files you want by pressing “enter” on it while moving with “space-4” or “space-1.”  If you want to move multiple files at once, you can do this by pressing “enter” after making a block with the “space” pressed.  Once you select the files that you want to be transferred, the files will transfer just like the file list in the file manager.  While moving files, the Braille Sense will show the file name move, and also shows the percentage of the file that has been transferred.  If the transmission has any trouble due to a location change or objects that are in the way of the IRDA port, the transmission may be cut off.  You can try rearranging the location of the devices that are trying to communicate with each other, and remove any hindrances, so you can continue to transfer files.  Once the transfer is complete, you will be returned to the file list.  If the transmission is disconnected, you will see a disconnection message and return to “transmit.”

### 14.5.2 Option Setting

### Pressing “enter” on “option setting” will pop up a dialog box where you can set the transmission condition.  The dialog box contains the following:  “searching times,” “set folder,” a “confirm” button, and a “cancel’ button.  You can move between these items by pressing “tab (“space-4-5”)” or “shift-tab (“space-1-2”)”.  In “searching times,” while activating a transmission, you can set the amount of times that the Braille Sense will attempt to look for another IRDA port to communicate with.  The default is set to “five times.”  If you want to change the default, move to the number that you want by pressing “space-1” or “space-4,” and press “enter,” or use tab (“space-4-5”) to move to “confirm,” and press “enter.”  When you use the “set folder,” option you are able to set the folder where your transferred files will be saved. The default is set to “flashdisk/download.”  If you want to change the folder, move to the folder you want to use, and press “enter” on the selected folder, just like you do in the file manager.  Remember, in a dialog box, you can use the “confirm” button for applying your changes to the setting, or you can press “enter” on the “cancel” button to discard your changes.

## 14.6 Calendar

## The “calendar” is the sixth submenu of the “utility” menu.  You can activate the calendar by pressing “enter” on “calendar” in the “utility” menu, or by pressing “l (dots 1-2-3).”  The following options will be available when you are using the calendar. “solar calendar,” “lunar calendar,” and “day of week.”  In the calendar, you can check the three kinds of information starting from the year 1841 to the year 2043.  If you type in a date that is not in the range and press “enter,” you will get the message, “input value error,” and return to where you where you were typing in the date.

### 14.6.1 Using the Calendar

### The “calendar” is composed of the “solar calendar,” “lunar calendar,” and “day of week.”  In the “lunar calendar,” a leap month will display the month with “l (dots 1-2-3).”  The calendar works according to a date unit.  The default is the date that the Braille Sense was set to the last time the date was set.  You can check the date in two ways.

### The following is an explanation of how to check the dates.

Moving items

You can switch between “solar calendar,” “lunar calendar,” and “day of week” by pressing “tab (“space-4-5”)” or “shift-tab (“space-1-2”)”.

Typing in date

In the calendar, you can type in a month, date, and year.  For the year, you should type in a 4-digit number, and for the month and date, you should type in 2-digit numbers.  It is possible to type these numbers in using computer Braille and regular grade 1 Braille without using the number sign.  You can put “space,” “/ (slash),” or a “- (dash)” between the month, date, and year.  If you press “enter” after typing in a date, you can hear the date once, and go to the next item.  For example, if you put the date in the “solar calendar,” you will be on the “lunar calendar.”

Hotkeys for switching dates

After putting in a date, you can check the “solar calendar,” “lunar calendar,” and “day of week” by pressing “tab (‘space-4-5’)” or “shift-tab (‘space-1-2’).”

- Move to the next day:  Space-6

- Move to the previous day:  Space-3

- Move to the next week:  Space-5

- Move to the previous week:  Space-2

- Move to the next month:  Space-4

- Move to the previous month:  Space-1

- Move to the next year:  Space-5-6

- Move to the previous year:  Space-2-3

## 14.7 Pronunciation Dictionary

## The “pronunciation dictionary” is used to register or modify the Braille Sense’s pronunciation while it is speaking.  You can activate it by pressing “p (dots 1-2-3-4)” in the “utility” menu.  If you activate the pronunciation dictionary, you will find the following four items: “add word,” “search word,” “modify word,” and “delete word.”  You can move between the four items by pressing “space-4” or “space-1.”  If you press “enter” on the item you want, it will be activated.

### 14.7.1 Add Word

“Add word” is used to add a new pronunciation.  If you activate this function, the Braille Sense will display the message “add key?.”  Here, you can type in a word that needs to be pronounced properly, and press “enter.”  Then, the Braille Sense will display “conversion value?”  Here, you can make the Braille Sense pronounce the word the way you want it to be pronounced, and then press “enter.”  After adding the word that you want, you will automatically return to the menu.  For example, “add key?,” type in “WHO,” and press “enter.”  In the “conversion value?,” type in “world health organization,” and press “enter.”  Then, when “WHO” appears in a document, the Sense will say “world health organization.”  Meeting the word “who,” however, it is not the same as “WHO.”  The pronunciation dictionary will distinguish between the lower case letters and upper case letters.  In the dictionary, you can use the lower and upper case letters, apostrophe, numbers, non-alphanumeric characters like a #, $, %, and punctuation marks (except for the final character).

### 14.7.2 Search Word

### "Search word” is used to check how words would be pronounced.  If you activate this function, you will find the message “search key?.”  Here, if you type in a word to find, and press “enter,” you will hear the message “search succeed.  key is xxx, conversion value is xxx.”  If the text is too long to display in a 32-cell display, you can review the remaining text by pressing the scroll buttons.  Pressing “space-z (dots 1-3-5-6)” will move you to the menu.

### 14.7.3 Modify Word

### “Modify word” is used to change a registered word’s pronunciation.  If you activate this function, you will find the message “modify key?.”  Here, type in the word whose pronunciation you want to, and press “enter.”  Then, you will see the message “conversion value?.”  Here, you can type in the word you want to modify, and press “enter.”  The new pronunciation will be saved, and you will return to the menu.  If you want to check the modification that you have made, you can do so by using the “search word” function (as explained in section 14.7.2).

### 14.7.4 Delete Word

### “Delete word” is used to delete a registered word.  If you activate this function, you will see the message “delete key?.”  If you type in a word to delete here, and press “enter,” the word will be deleted.  After the word is deleted, you will be returned to the menu.

## 14.8 Stopwatch

## When you press “enter” on the “stopwatch,” you will hear “stopwatch ready,” and your display will show “00:00:00.”  If you press “enter,” the stopwatch will start.  If you press “enter” again, it will pause, and the Braille Sense will say the elapsed time, and display it in Braille.  Pressing “enter” again will restart the stopwatch from where it paused.

## If you press “backspace” on “work” or “pause,” it will let you know the elapsed time and will initialize into “00:00:00.”  While the stopwatch is working, pressing “space” will make the Braille Sense say the elapsed time at the moment.  In this case, the Braille display is still just displaying the clock.

## While the stopwatch is running, pressing “space-F2” will stop displaying the time while the stopwatch is still running.  If you press “space-F2,” the time will be displayed again.

## If you press “tab (“space-4-5”)” or “shift-tab (“space-1-2”)” before the start of the stopwatch or while it is running, you can switch to the “count down timer.”  If you activate the “count down timer,” you will be asked to put in hours, minutes, and seconds.  When typing in the hours, minutes, and seconds, you should type in a two-digit number for the hour, minute and the second, regardless of whether you are using computer Braille or regular grade 1 Braille without using a number sign.  If you press “enter” without typing in any numbers, the “count down timer” will start with the default, which is set to one minute.  When started, the “count down timer” will display “count down xx minute,” and it will start running.  If you press “enter” in the “count down timer,” the Braille Sense will tell you the remaining time, and then it will pause.  If you press “enter” again, it will restart.  Pressing “backspace” will initialize the “count down timer.”  If the timer reaches zero, you will hear a 10-second alarm, and the “count down timer” will initialize.  If you press “backspace-enter” while the alarm is sounding, the alarm will stop and your timer will initialize, and will be ready to accept a new time.

## If you press “tab (space-4-5)” or “shift-tab (space-1-2)” again anywhere in the count down timer, the stopwatch will be ready.  If you want to exit the stopwatch or “count down timer,” press “space-z (dots 1-3-5-6)” anywhere within the stopwatch or “count down timer,” and you will return to the “stopwatch” menu.

## 14.9 Terminal for Screen reader

The “terminal for screen reader” function allows the Braille Sense to be used as a Braille display when hooked up to a computer that uses a compatible screen reader such as Window-Eyes.  Before using this function, make sure that the Braille Sense is connected to your personal computer via a serial cable, bluetooth serial or to the USB client port (USB type B port).  To turn on this function, go to the “utility” menu, and press “enter.”  Then, press “space-4,” until you hear “terminal for screen reader.”  Then, press “enter.”  If you activate "terminal for screen reader," you will see the list of connection methods.  In the list, you will see "serial port," "bluetooth serial port" and "USB port."  Press Space-1 or Space-4 until you reach what you want to select.  If you have selected one, press Enter and you will hear "terminal mode."  The Braille Sense will no longer say anything, as it is now simply acting as a Braille display for your computer.  If you fail to connect, you will be returned to the list of connection methods

For information and support files on connecting the Braille Sense to the USB port with Window-Eyes see:

GWKB1043 - Configuring the Braille Sense to Work as a Braille Display via the USB Port

http://www.gwmicro.com/Support/Knowledge_Base/?kbnumber=GWKB1043

For information and support files on connecting the Braille Sense to the USB port with JAWS see:

GWKB1046 - Using the Braille Sense as a Refreshable Braille Display with JAWS

http://www.gwmicro.com/Support/Knowledge_Base/?kbnumber=GWKB1046

For more information on Bluetooth, refer to Section 11.

## 14.10 Setup Internet

## The “setup Internet” function is used to set up your IP configuration.  In “setup Internet,” you will find “LAN,” “modem,” “ADSL,” and “wireless LAN.”  You can move between the four items by pressing “space-4” or “space-1.”  The “LAN” setup should be used if you are connecting to the Internet via a network or router.  The “modem” setup should be used if you are connecting to the Internet via a dial-up modem.  The “ADSL” setup should be used if you are connecting to the Internet via a DSL modem.  To connect to the Internet via a wireless network, use the “wireless LAN” function.  If you want to activate “setup Internet,” press “enter” on “setup Internet” in the “utility” menu, or press “I (dots 2-4)” in the “utility” menu.  You can move through the “utility” menu items with “space-1” or “space-4.”  The functions of this menu are explained in greater detail in the following sections.

### 14.10.1 LAN Setup

### If you are going to set up a LAN, you should first connect a LAN cable to the Braille Sense.  In this setup, you can decide whether you will use the automatic or manual setup to setup your IP configuration.  If you want to set it up automatically, press “enter” on “setup IP? Automatic.”  Then, it will be set up automatically.  For the automatic setup, your host server should provide DHCP service (Dynamic Host Configuration Protocol).

### If you want to set up your IP configuration manually, move to “manual” by pressing “space-4” from the “setup IP? Automatic,” and press “enter.”  Then, you will find the following:  “IP address,” “subnet mask address,” “gateway address,” and “DNS address.”  These settings require specific information.  Please contact your Internet service provider for the right information.  To type in the information, you should use computer Braille, and after typing in the information, you can move to the next item by pressing “space-4.”  Finally after completing the “DNS address,” press “enter.”  Then, you will see the message "setting completed."  You will be returned to "setup internet" with your information saved.  You can then use e-mail or the web browser.

### 14.10.2 Modem & Bluetooth DUN setup

If you are going to set up the Braille Sense to connect to the Internet via a modem, you should first put in a CF type modem card into the CF slot, and connect a phone line to the modem on the CF card.  Then, activate “setup Internet” in the “utility” menu.  After you have activated “setup Internet,” move to the modem by pressing “space-1” or “space-4,” and then press “enter.” If you wish to connect to Mobile network using Bluetooth DUN, you can move to ‘setup internet’ in the ‘utility’ menu and select ‘Bluetooth DUN’ and enter on it. In this case, you don’t need to insert CF modem card into the Braille Sense and its user interface is same as setting up connection using the modem.

The modem setup contains the following:  “connect list,” “default,” “register,” “modify,” “delete,” and “close.”  You can move between these by pressing “tab (“space-4-5”)” or “shift-tab (“space-1-2”)”.  Within each item, you can move by pressing “space-4” or “space-1.”

The “connect list” has registered connection names.  You can move between these by pressing “space-4” or “space-1.”

The “default” is to set to the default connection name in the Internet setup.  If you want to choose a connection as the default, go to the “connect list,” and move to “default” by pressing “tab (“space-4-5”)”.”  Then, press “enter.”

“Register” is a dialog box where you can set up the environment regarding the connect group names.   It has “connect name list,” a “confirm” button, and a “cancel” button.  You can move between these by pressing “tab (“space-4-5”)” or “shift-tab (“space-1-2”)”.  The “connect name list” has “connect name,” “connect number,” “id,” “password,” and “modem speed.”  In “connect name,” you can type in the host name to connect with, and in the “connect number,” you can type in the number of the host.  Here you can type in numbers and commas (,) only.  You can type in numbers using computer Braille.  In “ID” and “password,” use computer Braille to type the ID and password that are required by the host server. In case of Bluetooth DUN, you can make the connection to Mobile network without any password according to Mobile server.  In the modem speed, select one from the list by pressing “backspace” or “space.”  After completing each item, you can move to the next item by pressing “tab (“space-4-5”)”.”  Finally, with “modem speed” completed, you can press “enter” to save your registration.  Before saving it, you can cancel your choices by moving to “cancel” by pressing “tab (“space-4-5”)”,” and press “enter” on it.

“Modify” is used to modify the connect group.  You can do this by moving to what you want to select in the “connect list,” and then press “enter.”  You can also press “tab (“space-4-5”)” to locate “modify.”  Otherwise, you can press “enter-m (1-3-4)” on the “connect name” in the “connect list” directly.  “Modify” has the same items as “register,” and you use the menus in the same way.

In “delete,” if you want to delete an item, you can move to what you want to delete in the “connect list,” and then press “enter” on “delete.”  Use “tab (“space-4-5”)” to move between the items.  Or, you can delete an item by pressing “space-d (dots 1-4-5)” on what you want to delete in the “connect list.”

If you want to exit the “modem setup,” you can press “space-z (dots 1-3-5-6),” or you can press “enter” on “close” by moving to it by pressing “tab (“space-4-5”)”.

### 14.10.3 ADSL Setup

### If you want to connect the Braille Sense to the Internet via a DSL connection, you should use the ADSL setup.  First, you should plug in a LAN cable to the Braille Sense’s LAN port.  If you press “enter” on “setup internet,” you will first see "CB LAN 1/3."  Then, go to ADSL by pressing “space-4” repeatedly.  Press “enter” on ADSL.  It contains the following controls:  connect list, default, register, modify, delete, and close.  You can move between the controls by pressing “tab (“space-4-5”)” or “shift-tab (“space-1-2”)”.  Within each control, you can move item by item by pressing “space-4” or “space-1.”

### In the “connect list,” you will see the registered connect names.  Select one by pressing “enter” on it.  In “default,” you can make one of the registered connect names the default connection.  If you make a connection the default connection, that will be the connection that is used to connect to the Internet first.  In “register,” you can manage the connect names in the list.  It has three controls:  edit field, a confirm button, and a cancel button.  You can move between the three controls by pressing “tab (“space-4-5”)” or “shift-tab (“space-1-2”)”.  The edit field contains the following:  connect name, ID, and password.  You can move between the controls by pressing ‘”space-4” or “space-1.”  When in the “connect name” control, type in the server host name.  In “ID” and “password,” you can type in what you want in ASCII.  After completed filling in the appropriate information, press “enter” or press “enter” on “confirm” after pressing “tab (“space-4-5”)” moving.  To cancel everything that has been typed in to the edit field, go to “cancel,” and press “enter.”

### In “modify,” you can modify a connect name.  If you want to modify a connection in the “connect list,” select the connect name you want to modify in the list.  Then, press “enter” on “modify.”  Or, after you have selected the connect name, press “enter-m (dots 1-3-4).”  “Modify” contains the same items as “register,” and the text is typed in the same way.

### Using “delete,” you can delete selected connect names.  If you want to delete a connect name in the “connect list,” select the connect name you want to delete in the list, and then press “enter” on “delete.”  Or, after you have selected the connect name, press “space-d (dots 1-4-5).”

### If you want to exit the ADSL setup, press “space-z (dots 1-3-5-6),” or press “enter” on “close.”

### 14.10.4 Setup Wireless LAN

### If you want to connect to a wireless network, you should use the “setup wireless LAN” function.  In order to use the “setup wireless LAN” function, you must first insert a wireless LAN compact flash card in the CF slot that is located closest to the front panel on the Braille Sense.

### Go to the “utility” menu that is located in the “program” menu by pressing “F1,” and then “space-4” to move to the “utility” menu.  Then, press “enter” on “utility.”  Next, move to "setup Internet" by pressing “space-1” or “space-4,” and then press “enter” on “setup Internet.”  Then, move to "wireless LAN" by pressing "space-4,” and then press “enter” on “wireless LAN.”

### The "wireless LAN" dialog box consists of the "access point" list, "status" static box, "signal strength" static box, "set profile" button, and “advanced” button.  You can move between these controls by pressing "tab (“space-4-5”)" or "shift-tab (“space-1-2”)”.  If you do not have a wireless LAN card installed, you will hear “no item” when you press “enter” on “wireless LAN.”

### When you first open the "wireless LAN" dialog box, the focus is on the "access point” list.  You cannot modify this list.  The list is simply showing wireless access points that are available to the Braille Sense.  “Access point” list will let you know the signal strength (dB) of each item.  Ex: "linksys (30dB)."  If there is more than one access point in the list, you can move between them by pressing “space-1” or “space-4.”  You may not initially have any access points available because the Braille Sense requires a few seconds to find the available wireless devices.  Move to the "access point" that you want to connect to.  Then, continue to the "status" static box by pressing “tab (“space-4-5”)”.  The "status" static box shows the status of the wireless LAN connectivity, and it displays "not connected" unless you setup the wireless LAN in the "set profile" dialog box, which can be opened by pressing "enter" on the "set profile" button.

### After reviewing the status, move to the "signal strength" static box by pressing “tab (“space-4-5”)”.  The "signal strength" static box informs you of how intense the signal the Braille Sense is receiving from the wireless access point.  The different strengths are:  "very low,” "low,” "good,” "very good,” and "excellent.”  This static box, regardless of the setting in "set profile,” informs you about the intensity of the signal from the wireless access point.  However, the “signal strength” static box displays "no signal" when the wireless LAN is disconnected.  If you want to know the signal strength of a specific access point, move to the wireless access point name.  Then, press “tab (“space-4-5”)” to move to the "signal strength" static box, which will tell you the signal strength of the specific access point.

### The "set profile" dialog box is used to setup the wireless LAN.  If you press "enter" on the "set profile" button while the wireless LAN card is disconnected, "access point does not exist" will be displayed.  Pressing "enter" on the "set profile" button, while the wireless LAN card is properly connected, displays controls to setup the wireless LAN.  The "set profile" dialog box has 12 controls.  You can navigate among these controls by pressing “tab (“space-4-5”)” or “shift-tab (“space-1-2”)”.  The following is a list of the 12 controls:

- “setup IP?  Automatic” combo box:  You can switch between automatic and manual by pressing “space-4” or “space-1.”  If you select “manual,” you will need to enter your IP address, subnet mask, gateway IP address, and DNS IP address.  You can move between these computer edit boxes by pressing “space-4” or “space-1.”  Press “tab (“space-4-5”)” to move to the next control.

- "network name (SSID)" computer edit box:  Displays what you are focused on in the "access point" list.  You can type in a different name if you want to refer to this network as something different.

- "network mode” combo box:  Here you can select between "infrastructure," and "802.11 ad-hoc" mode by pressing "space-1" or "space-4.”

- "network authentication (shared mode)" checkbox:  Check this box if the authentication uses "shared mode.”  Otherwise, uncheck the box by pressing "space.”  The default for the box is checked.

- "data encryption (WEP enabled)" check box:  Check this box if "WEP (Wired Equivalent Privacy)" encryption is used.  The default is checked.  To uncheck it, press "space.”

- "the key is provided automatically" check box: If the "access point" does not provide a key for users automatically, uncheck this check box by pressing "space.”  The default is that the check box is checked.

- "network key" computer edit box:  This is a computer edit box to type the network key if the key is not provided automatically.

- "key format" combo box:  The encryption format for the key input can be selected between "ASCII characters" and "hexadecimal digits" by pressing "space-1" or "space-4.”

- "key length" combo box:  Encrypted key length can be selected between 64 bits (5 characters) and 128 bits (13 characters) by pressing "space-1" or "space-4.”

- "key index" edit box:  This is an edit box to type the port number of the wireless access point that matches the network key input.

- "confirm" button:  Pressing this button applies the changes.

- "cancel" button:  You can press this button to cancel the setup.

Unchecking "data encryption (WEP)" skips numbers 6 through 10, and checking "the key is provided automatically" check box will omit the numbers 7 through 10.

After completing "set profile,” the status of the wireless LAN connectivity can be viewed by moving to the "status" static box.

The “profile list” group contains the “profile list,” a “connect” button, “add common dialog” button, a “remove” button, and “properties common dialog” button controls.

- “profile list”:  Here you will see the profile items that you have registered in the “set profile common dialog” or in the “add common dialog.”  You can check the list by pressing “space-4” or “space-1.”

- “connect” button:  After selecting any profile item in the profile list, press “enter” on the “connect” button by pressing “tab (“space-4-5”),” and the Braille Sense will try to connect to the internet.

- “add common dialog” button:  If you don't have an access point registered, you can add one here.  “Add common dialog” has the same structure as the “set profile common dialog.”  If there is no access point currently near you, you will see “no item” in the “network name (SSID).”  However, this function allows you to setup a wireless card to access a wireless network that you are not near, or if the wireless access point is not broadcasting the SSID.  This way, you will still be able to connect to the access point when you are near it.

- “remove” button:  If you want to delete a selected profile item in the profile list, go to the “remove” button by pressing “tab (“space-4-5”),” and then press “enter” on “remove.”

“properties common dialog” button:  If you want to get information about a selected profile item in the profile list, press “enter” on the “properties common dialog” button after focusing on a profile item by using “tab (“space-4-5”)”.  Then, you will see the same information as in the “set profile common dialog.”  You can reconfirm the registered information in each item.  You can modify an item here directly.

You can press “space-e(dots 1-5)” or “space-z(dots 1-3-5-6)” to get out of the “Wireless LAN” dialog box.

## 14.11 Network Status

Using this program, you can check if the Braille Sense is currently connected to the Internet.  This program provides you with the information about the current connection type, the current IP address of the unit, subnet mask, gateway, DNS, and mac address.  To run this program, press “enter” on “network status,” or press “n (dots 1-3-4-5)” in the “utility” menu.  If you start the program, a list of information about the connection type and address will be displayed.  If you press “tab (space-4-5),” the Braille Sense will display a “renew” button.  If you press “tab (space-4-5) again, “exit” will be displayed.  If you press “shift-tab (space-1-2),” the Braille Sense will move between these elements in the opposite direction.  The first item in the information list (status) shows if the Braille Sense is currently connected to the Internet (online/offline), and what the current connection type is (LAN, Modem, ADSL, or Wireless LAN).  The next item is the “current IP address,” followed by “subnet mask,” and then “gateway,” “DNS” and finally “mac address.”  You can move between these items by pressing “space-4” or “space-1.”  If the Braille Sense is not currently connected to the Internet, the status information shows that the Braille Sense is offline.  If you press “enter” on the “renew” button, it tries to connect to Internet using the parameters specified in “setup Internet” once again, and displays on/offline status.  To end this program, press “space-z (dots 1-3-5-6)” at any time in the program.  Or pressing “enter” on the “exit” button will close the program too.  You can also press “space-n (dots 1-3-4-5)” while you are in the “program” menu to open network status.

## 14.12 Check Power Status

## You can activate “check power status” function by pressing “b” or press “enter” on the “check power status” in the “utility” menu.   You can move to it by pressing “space-4” or “space-1” in the “utility” menu.  Anywhere in the Braille Sense, you can activate this function by pressing “space-1-6.”  In “check power status,” you can check two kinds of information regarding the Braille Sense's power.  It will let you know the recharging percentage and if the Braille Sense is using the AC adapter or the battery.  If you activate this function, you will see “charge xx%, use battery or AC power.”  After checking it, you can exit by pressing “space-z (1-3-5-6),” which will return you to the menu.

## 14.13 Backup/Restore Option

## “Backup/restore option setting” is a program where you can backup/restore the option settings for each program in the Braille Sense all at once. This program can be found in the “utility” menu. You can run this program by pressing “k (dots 1-3)” in the “utility” menu. If you run this program, a dialog box will be opened. This dialog box contains: “program option check box list”, “select all” button, “backup” button, “restore” button, and “cancel” button. You can move between the check boxes in the “program option list”, by pressing space-1” or “space-4.”  You can move between these controls by pressing “tab (space-4-5)” or “shift-tab (space-1-2).”  The “program option check box list” contains “option settings”, “file manager option”, “word processor option”, “address manager option”, “schedule manager option”, “media player option”, “web browser option”, “e-mail option”, “messenger option”, and “database manager option”.

## In this list, you move to the program option and check it by pressing “space.” If you want to uncheck it you can do so by pressing “space” again. After you select all the options you want to backup or restore, press “enter” on the “backup” or “restore” button. If you want to select all of the programs in the list, press “enter” on the “select all” button. If you press “enter” on “backup” or “restore” button, the dialog box will be closed. The backed up options will be saved to the “flashdisk/database/option” folder as files with the same names as each program.

## If you press a “backup” button, it lets you see a list in which you can select a path where to store a backup file. It shows up “flashdisk” as default values. Here you move to disk or folder where you want to store a file, by using navigation key in a “file manager” program. And you select a folder or disk by pressing “space”, press “enter”, it turns up that a backup file is stored in selected folder. Each one of these options has its own backup file. This means that when you backup each of the 5 options, it will generate 5 backup files. If you need to store a backup file in a root of disk, refer to this way. When it comes out a name of disk, you press “enter” after selecting a disk by pressing “space”. If you press a “restore” button, it lets you see a list in which you can select a path where to restore a backup file. A way how to move or select a folder or disk is the same as “backup”. If you select a folder that is not back upped, you can be seen a message which tells that there is no backup option file.

## If you cold reset your Braille Sense, or its battery is fully discharged, the main option settings and settings for each program will be lost, and initialized to the default values. If you backup options using this program right before you cold reset, you can restore the option settings you set previously.

## 14.14 Upgrade Braille sense OS

## If you activate this function, you can update the Braille Sense either through an Internet connection or from a disk that contains the upgrade file.  If you decide to upgrade using an Internet connection, you must first complete the “setup Internet” function.  If you update the Braille Sense, all of the settings will be returned to their default status.  Data in the flashdisk will be protected.  However, we recommend that you backup your files before updating the Braille Sense OS.

## While updating, you cannot cancel the update.  Note that you must not press the reset button while updating, and you should avoid power failure.  We recommend that you do not touch the Braille Sense while it is updating.  It may take up to 30 minutes to update the Braille Sense, though it depends on the speed of your Internet connection.  On completing the setup, the Braille Sense will reboot again to finish the update.

### 14.14.1 Upgrading the Braille Sense OS Using the Internet

### To upgrade the Braille Sense OS via the Internet, complete the following steps:

- Make sure an Internet connection is available.  Press “enter” on “Upgrade Braille Sense OS” in the “utility” menu.  Then, "Upgrade? online" will appear.

- Now, press “enter” on “Upgrade? Online.”  The Braille Sense will connect to the HIMS server, and your current software version will be compared to the HIMS server version.  You will then be asked if you want to continue to upgrade.

If you press “enter” on “no,” the upgrade will be cancelled.  Press “space” to toggle between “yes” and “no.”  If you press “enter” on “yes,” the Braille Sense will download the upgrade file, the upgrade will stop.  If the upgrade fails, it will give you the message “Internet connection is not available.  Check your cable and try again.”  To try the upgrade again, press “space-z (1-3-5-6)” to cancel, and then activate upgrade Braille Sense OS again.  If the connection is successful, the updated files will be downloaded, and the Braille Sense will start upgrading.  If you do not have enough memory available on the flashdisk, the Braille Sense will be unable to update until you make room for the upgrade files by deleting any unnecessary files from the Braille Sense.

- After downloading the upgrade file, Braille Sense will automatically start the upgrading process.  Please remember that you must not do anything to the Braille Sense until the upgrade is complete.  When the upgrade is complete, you will be returned to the “program” menu, and you will hear “file manager.”

- After you have completed the upgrade, you should check the software version by pressing “space-v (dots 1-2-3-6)” while you are in the “program” menu.

### 14.14.2 Upgrading the Braille Sense OS from a Disk

### To upgrade the Braille Sense OS from a disk, complete the following steps:

- Connect a disk (such as a compact flash card or USB memory stick) to the Braille Sense.  The disk must have the upgrade file stored on it.  Then, press “enter” on “Upgrade Braille Sense OS.”

- Then, "Upgrade? online" will appear.  Press “space-4” to go to the “Upgrade? Offline.”  Then, press “enter.”  Then, you will see the disk list.  Press “space-4” repeatedly to go to the disk that contains the upgrade file.  Then, press “enter.”

- You will then be asked if you want to continue to upgrade.  If you press “enter” on “no,” the upgrade will be cancelled.  Press “space” to toggle between “yes” and “no.”  If you press “enter” on “yes,” the Braille Sense will start upgrading.  This step will not appear when your current software version is not the latest version.

- The Braille Sense will automatically start the upgrading process.  Please remember that you must not do anything to the Braille Sense until the upgrade is complete.  When the upgrade is complete, you will be returned to the “program” menu, and you will hear “file manager.”

- After you have completed the upgrade, you should check the software version by pressing “space-v (dots 1-2-3-6)” while you are in the “program” menu.

## 14.15 Hot Keys for Utility

Calculator:  c (dots 1-4)

Calculator initialize:  Backspace-c (dots 1-4)

Delete numbers or operators:  Backspace

General function:  Enter-g (dots 1-2-4-5)

Plus:  dots 3-4-6

Minus:  dots 3-6

Division:  dots 3-4

Multiplication:  dots 1-6

Percent:  dots 1-4-6

Power:  Backspace-dots4-5

Square Root:  Enter-q (dots 1-2-3-4-5)

Decimal Point:  dots 4-6

Left Parenthesis:  dots 1-2-3-5-6

Right Parenthesis:  dots 2-3-4-5-6

PI:  Enter-p (dots 1-2-3-4)

Exp: Enter-e (dots 1-5)

Variable Function:

Recall Items:  Backspace-r (dots 1-2-3-5)

Delete all saved items: Enter-d (dots 1-4-5)

Save Items: Backspace-s (dots 2-3-4)

Trigonometric Function:  Enter-s (dots 2-3-4)

Sine:  Enter-s (dots 2-3-4), s  (dots 2-3-4)

Arc Sine:  Enter-s (dots 2-3-4), a (dot 1)

Hyperbolic Sine:  Enter-s (dots 2-3-4), h (dots 1-2-5)

Cosine:  Enter-c (dots 1-4), c (dots 1-4)

Arc Cosine:  Enter-c (dots 1-4), a (dot 1)

Hyperbolic Cosine:  Enter-c (dots 1-4), h (dots 1-2-5)

Tangent:  Enter-t (dots 2-3-4-5), t (dots 2-3-4-5)

Arc Tangent:  Enter-t (dots 2-3-4-5), a (dot 1)

Hyperbolic Tangent:  Enter-t (dots 2-3-4-5), h (dots 1-2-5)

Logarithm Function:  Enter-l (dots 1-2-3)

Natural Logarithm:  Enter-l (dots 1-2-3), e (dots 1-5)

Common Logarithm:  Enter-l (dots 1-2-3), n (dots 1-3-4-5)

Copy to Clipboard:  Enter-I (dots 2-4)

Option Setting:  Enter-o (dots 1-3-5)

Setting date and time: t (dots 2-3-4-5)

Move to the next day:  Space-6

Move to the previous day:  Space-3

Move to the next week:  Space-5

Move to the previous week:  Space-2

Move to the next month:  Space-4

Move to the previous month:  Space-1

Move to the next year:  Space-5-6

Move to the previous year:  Space-2-3

Move to 1 hour after:  Space-4

Move to 1 hour before:  Space-1

Move to 10 minutes after:  Space-5

Move to 10 minutes before:  Space-2

Move to 1 minute after:  Space-6

Move to 1 minute before:  Space-3

Switch Between a.m. and p.m.:  Space-x (dots 1-3-4-6)

Checking date and time:  Space-t (dots 2-3-4-5)

Calendar:  l (dots 1-2-3)

Move items

Move to the previous item:  Tab (space-dots 4-5)

Move to the next item:  Shift-tab (space-dots 1-2)

Move key for date in calendar

Move to the next day:  Space-6

Move to the previous day:  Space-3

Move to the next week:  Space-5

Move to the previous week:  Space-2

Move to the next month:  Space-4

Move to the previous month:  Space-1

Move to the next year:  Space-5-6

Move to the previous year:  Space-2-3

Pronunciation dictionary:  p (dots 1-2-3-4)

Stopwatch:  w (dots 2-4-5-6)

Stopwatch start, pause, restart:  enter

Check the elapsed time:  backspace

Stop Braille display:  Space-F2

Restart Braille display:  Space-F2

Switch between stopwatch and count down timer:  “tab (“space-4-5”)” or “shift-tab (“space-1-2”)”

Count down timer pause or restart:  enter

Count down timer Initialize:  backspace

Exit stopwatch or count down timer:  Space-z (dots 1-3-5-6)

Setup internet:  I (dots 2-4)

Check power status:  Space-dots 1-6

Check network status:  Space-n (dots 1-3-4-5)

Backup/restore option: k (dots 1-3)

Move to previous item: Space-1 or left scroll key

Move to next item: Space-4 or right scroll key

Move to previous control: Shift-tab (space-1-2 or space-F3)

Move to next control: Tab (space-4-5 or F3)

Upgrade Braille Sense OS:  u (dots 1-3-6)

Check download information:  Space-I (dots 2-4)

Cancel download and exit:  Space-z (dots 1-3-5-6)