How to modify the APOLLO2.INI file to Match Your Speech Synthesizer's

Configuration.

----------------------------------------------------------------------

The Dolphin Speech Synthesizers are very flexiable, and to setup the

driver software to match your speech synthesizer's configuration you

must edit the APOLLO2.INI file associated with the Juno, Europa and Apollo2

driver.

To completly set it up for An Open Book or any application that uses

an SSIL driver you must:

1. Specify COM port

2. Specify Baud rate

3. Specify Apollo2, Juno or Europa card

4. Specify the language ROMs installed

5. Specify the exception dictionarys and phoneme files

1. HOW TO CHANGE COM PORT, BAUD RATE FOR APOLLO2, JUNO OR SPECIFY EUROPA CARD

In the section [apollo driver] you can specify the type selection

as being equal to a "com" device for Apollo2 or Juno, or as a internal

expansion card as "europa" for Europa speech synthesizers.

Find the command line and specify either "com" or "europa".

These are case-sensitive.

As example:

Type=com

To specify the COM port number find the command line "PortNumber" and

make the appropriate choice.

As an example, for COM 1:

PortNumber=1

To specify the Baudrate (com port only), the rates allowed are 300,

1200, 9600 and 19200. Make sure that your apollo 2 is set to the same rate

as specified in the APOLLO2.INI file.

As an example, for 9600 baud:

Baudrate=9600

In this section you can specify the Europa 2 PC-Card port number in decimal.

The Default port = 776.   In hexadecimal that is 308.

As an example, for 776:

CardPort=776

2. HOW TO RECOGNIZE THE LANGUAGE ROMS AND MULTIPLE LANGUAGE INSTALLATIONS

In APOLLO2.INI file in the below the Cardport command line you will find

the command lines for specifiy the ROMS installed in your synthesizer.

You first need to specify the ROM information, you do this by specifing

the ROMXID number. This number is the standard telephone country code

for the language you are installing. The ROMXID must be 4 digits

(the 1st digit is a spare one for expansion), the ROMXID is padded with zeros.

You must also specify the ROMXSTRING. This is the name of the language

of the ROM and it will be name that will appear in your program. The

ROMXSTRING is not essential, but it useful for menu driven ROM switching

programs like An Open Book.

The "X" in ROMXID and ROMXSTRING is the position or sequence of the ROMs

in your speech synthesizer. This is noted on a label that is found on

the bottom of Apollo2 and Juno speech syntheiszers. You will need a sighted

helper to read this label. For Europa speech synthesizers it is the sequence

of ROM installed on the PC card, going from left to right. You will

need a sighted helper to read the labels on the ROMS.

As an example for a U.S. English ROM installed as the first ROM:

rom1id=0001

rom1string=U.S. English

NOTE: To set the default voice for Open Book, you must edit the settings

file (DEFAULT.INI) and set the "Language=" entry to point to the appropriate

rom id.  Remember that Open Book menu items are numbered starting from 0,

rather than 1.  Thus, to change the menu voice for Open Book to use rom

number 2, find the [Menu Voice] section, and make sure the language entry is

set to "Language=1".

3. ADDITIONAL LANGUAGE ROM EXAMPLES

What follows are additional rom#id and rom#strings for additional

To make active delete the ";" which proceeds the string and change #

for the number of the ROM socket, which contains the ROM for that language

The sequence of rom#id and rom#strings in this file must match the

sequence/position of the ROM in your Apollo2

The U.S. English example above means that you have only one ROM, it is

U.S. English and it is in ROM socket number one and it is the only ROM in

the Apollo2. If you have additional ROMs you must activate the appropriate

rom#id and rom#strings in the proper sequence. To activate an additional

language, delete the ; which proceeds each string in the APOLLO2.INI file

and replace the # with the number of it's ROM socket.

As an example for U.S. English in ROM 1 and Spanish in ROM 2, you would

have:

rom1id=0001

rom1string=U.S. English

rom2id=0034

rom2string=Spanish

The country codes and strings for other languages that are supported are:

rom#id=0044

rom#string=U.K. English

rom#id=0045

rom#id=Danish

rom#id=0031

rom#string=Dutch

rom#id=0033

rom#string=French

rom#id=0049

rom#string=German

rom#id=0039

rom#string=Italian

rom#id=0047

rom#string=Norwegian

rom#id=0351

rom#string=Portuguese

rom#id=0034

rom#string=Spanish

rom#id=0046

rom#string=Swedish

;Exception dictionary, number and phoneme file names

;You must preceed the file name with the drive and path for the WINDOWS

;directory. These files must be in the WINDOWS directory

;For ROM1, U.S. English

;No 0001.DIC file exists for U.S. English

;No 0001.NUM file exists for U.S. English

;No 0001.PHO file exists for U.S. English

;Note: The dictionary, number and phoneme file must be in the Windows directory

dictionarypath=c:\windows\

;SSIL specific information - used by ssil-synd translator - apollo.c

[apollo ssil]

;number of preset voices. All preset sections are [apollo preset #]

presets=5

; the default voice - presets are created with these parameters

; that are then modified. This is also the apollo default power on state

[apollo default]

name=Voice 1

outputmode=0

language=1

rate=3

volume=15

pitch=8

intonation=4

punctuation=0

wordpause=0

phrasepause=11

spellmode=0

breath=8

hypermode=0

markspace=10

gender=1

voicesource=0

;NB language parameter is a virtual language number, 1 to # of roms fitted

;it is mapped by the driver into the actual rom number (as used by @= )

;the mapping process misses out any non-existent roms.

;EG if you only have a 4-language big rom, addressed with @=1/5/6/7

;you use language=1/2/3/4 respectively.

;presets - supported parameters and values allowed in preset sections

;name=string    string is max 32 characters please!

;COMMAND          RANGE                           APOLLO COMMAND

;language=n     0 to #of roms installed             @=

;rate=n         0 to 15                             @W

;volume=n       0 to 15                             @A

;pitch=n        0 to 15                             @F

;intonation=n   0 to 7                              @R

;wordpause=n    0 to 9                              @Q

;phrasepause=n  0 to 15                             @D

;breath=n       0 to 9                              @B

;hypermode=n    0 to 1                              @H

;markspace=n    10 to 35                            @M

;gender=n       1 to 6    (1-3 male, 4-6 non-male)  @V/@F/@M/@B

;voicesource=n  0 to 7                              @$

;spellmode      0 to 1                             N/A - done in driver

[apollo preset 1]

name=Voice 2

gender=2

[apollo preset 2]

name=Voice 3

gender=3

[apollo preset 3]

name=Voice 4

gender=4

[apollo preset 4]

name=Voice 5

gender=5

[apollo preset 5]

name=Voice 6

gender=6

;if you add or delete a preset remember to change 'presets=' !