ENABLE READER 4.0         OPERATIONS MANUAL         CHAPTER 5-1           CHAPTER 5  TALKING TUTORIAL PROGRAMMER'S AID   5.0  INTRODUCTION  The Talking Tutorial Programmer's Aid is designed for the  experienced assembly level programmer.  Interfacing between  higher level lanuguages such as PASCAL and C BASIC are possible,  yet we do not provide these bridges.  There are ten calls that  can be accessed through the INT 21 MSDOS Supervisor Call  structure.  Load the correct registers with the values  necessary, and then do INT 21.  Enable Reader 4.0 will save all  registers except the FLAGS, where the ZERO Flag will be set if  the call was not one of the available Enable Reader 4.0  Functions.  The Enable Reader Programmer's Aid will check to see  if the instruction was successfully completed.    When writing tutorials with the Enable Reader 4.0 System, the  information that is printed to the screen will be spoken.  Most  of the tutorial will just use this feature.  The Programmer's  Aid is designed to provide byte I/O to the synthesizer without  printing the information to the video screen, and to use Help  Functions in the Enable Reader 4.0 program to enhance the way  data will be spoken.  For example, if you need to speak  something in a way that would be confusing to the student if it  were printed to the screen, such as spelling out a word or line,  it is possible to send the data through the Enable Reader 4.0  driver to the synthesizer in Spell Mode.  This can be used to  great advantage with programs that teach spelling and reading  skills.     5.1  FUNCTION CALLS  The following calls can be made to the INT 21 Call in MSDOS by  loading the AH Register with FE HEX (254 Decimal), and then  loading the AL register with the appropriate Ascii Character  designated to call the function.    The Enable Reader Driver is both Stream Oriented and  Hierarchical in design.  This means that entry points are  accumulative, and data passing through the driver's other entry  points gains the same attributes.  By using byte-stream  programming, and not using macros and calls to each subroutine,  the data is processed more rapidly.    The three most used entry points are:  1) DRIVER OUTPUT Call  that sends a byte through the driver using all the Help  functions that may be turned on, and the Punctuation routines;  2) DIRECT OUTPUT Call that sends a byte directly to the  synthesizer, bypassing the driver and the punctuation filter;  and 3) MESSAGE OUTPUT Call that turns off the Help functions in  the driver, and still allows for punctuation filtering.  The  DIRECT OUTPUT Call will allow 8 bits to be sent, while the other  two will only allow 7 bits.    ENABLE READER 4.0         OPERATIONS MANUAL         CHAPTER 5-2It has taken three years and fourteen versions of the program to  identify speech primitives, assign hierarchy to Help Functions,  integrate Punctuation Levels, and provide the four Entry Points;  so that when you drop a byte in the top, it comes out the bottom  without using look-up tables or macros.    The Enable Reader Driver has a Character Filter and an Ascii  Sort Routine.  The Character Filter filters out all bytes over  80 HEX (graphics), and sends all bytes under 20 HEX (Control  characters) to the Punctuation Level Routines.  This allows no  graphic characters to pass through the driver.  It is possible  to send 8 bit bytes to the synthesizer through the DIRECT OUTPUT  call below.                      TALKING TUTORIAL FUNCTION CALLS  DRIVER OUTPUT                   FUNCTION 01  The DRIVER OUTPUT Call will output the character in DL through  the Enable Reader driver, without turning off the Help  Functions.  It will be filtered by the Ascii sort routine in the  Enable Reader Driver, and if it is punctuation it will be sent  according to the Level of punctuation selected.    See:  DIRECT OUTPUT to send a byte directly to the synthesizer. See:  MESSAGE OUTPUT for sending a byte with Help Functions       turned off.  ENTRY CONDITIONS:  AL=01H AH=FEH DL=Character To Output     DIRECT OUTPUT                   FUNCTION  02  The DIRECT OUTPUT Call will send the byte in DL directly to the  synthesizer, bypassing the Enable Reader driver and Ascii filter  sort routine.    ENTRY CONDITIONS:  AL=02H AH=FEH DL=Character To Output     ENABLE READER 4.0         OPERATIONS MANUAL         CHAPTER 5-3MESSAGE OUTPUT                  FUNCTION  03  The MESSAGE OUTPUT Call allows character filtration and  punctuation, but turns off the Help Functions in the driver.    ENTRY CONDITIONS:  AL=03H AH=FEH DL=Character To Output     AUTO                            FUNCTION  A  The AUTO Call turns on or off the Spell Mode.  Read the  discussion on AUTO in Chapter 4 Section 4.2 Help Keys.    ENTRY CONDITIONS:  AL=41H (ASCII A) AH=FEH     PUNCTUATION LEVELS              FUNCTION  N  The PUNCTUATION LEVELS Call selects one of five Punctuation  Character Sets that can be accessed.  The fifth level is "Output  Punctuation", which allows all Ascii characters to be sent to  the synthesizer without being filtered or translated into the  Enable Reader 4.0 Punctuation Words.  Read the discussion on  PUNCTUATION LEVELS in Chapter 4 Section 4.2 Help Keys.    ENTRY CONDITIONS:  AL=4EH (ASCII N) AH=FEH     HELP WORD LEVELS                FUNCTION  O  The HELP Call selects one of four levels of Help in Enable  Reader 4.0.  By selecting "No Help" at the start of your  program, you can use the other Help Functions without having  them speak their prompts.  Read the discussion on HELP WORDS in  Chapter 4 Section 4.2 Help Keys.    ENTRY CONDITIONS:  AL=4FH (ASCII O) AH=FEH     ENABLE READER 4.0         OPERATIONS MANUAL         CHAPTER 5-4QUIT                            FUNCTION  Q  The QUIT Call will stop the synthesizer from speaking.  For a  discussion on the QUIT Function, read Chapter 4 Section 4.5  Command Keys.    ENTRY CONDITIONS:  AL=51H (ASCII Q) AH=FEH     LETTER-TO-WORD TRANSLATOR       FUNCTION  Z  The TRANSLATOR Call turns on or off the Letter-To-Word  Translator.  For a discussion on the TRANSLATOR Function, read  Chapter 4 Section 4.2 Help Keys.    ENTRY CONDITIONS:  AL=5AH (ASCII Z) AH=FEH     UPPER CASE IDENTIFICATION       FUNCTION  T  The UPPERCASE IDENTIFICATION Function turns on or off the  identification of capital letters.  For a discussion on the  UPCASE Function, read Chapter 4 Section 4.2 Help Keys.    ENTRY CONDITIONS:  AL=54H (ASCII T) AH=FEH     VIDEO OUTPUT                    FUNCTION  V  The VIDEO OUTPUT Call turns on or off the output to the  synthesizer from data that is printed on the video screen.  For  a discussion on the VIDEO Function, read Chapter 4 Section 4.6  System Keys.    ENTRY CONDITIONS:  AL=56H (ASCII V) AH=FEH  END OF   CHAPTER 5  TALKING TUTORIAL PROGRAMMER'S AID ##################################################################################################