ENABLE READER 4.0         OPERATIONS MANUAL         CHAPTER 4-19                CHAPTER 4  FUNCTION KEYS (Continued)   4.3  CURSOR MOVEMENT KEYS  CATEGORY DESCRIPTION:  The Cursor Movement Keys are used to move a blinking pointer on  the screen to the position you specify.  In this section of the  manual, the words "CURSOR" and "POINTER" will be used  interchangeably.  This is due to user feedback, in that they  knew what a Pointer was, but not a Cursor.  Since we used the  word "cursor" in the Enable Reader 2.0 version, we decided in  order to keep upward compatiblility for the original users with  all 8, 16, and 32 bit versions, we would use both "pointer" and  "cursor".    The Cursor Movement Keys and the Reading Keys were designed  around the concept of "HOMING IN".  This is the logical process  of reviewing the data on the video screen by starting with the  full screen.  Then by successively breaking the screen down into  smaller and smaller parts (windows, lines, words, characters),  the data can be found and identified (letter translation, space  and uppercase identification, spell mode).  This is the concept  of "Homing In".    The blinking pointer on the screen is visible to sighted co- workers, and as each function is executed the cursor position is  updated.  This allows visually impaired and blind people to  demonstrate visually to sighted people where they are on the  screen when reviewing.  If the function executed is not a Cursor  Movement or Reading Key, the cursor will remain in the same  position.      KEYNAME:  DOWN  KEYSTROKE:  LETTER D as in DELTA  KEYTYPE:  SINGLE LEVEL  SUMMARY:  MOVE POINTER DOWN ONE LINE UNTIL BOTTOM OF SCREEN  DESCRIPTION:  The DOWN Key will move the cursor down one line while it remains  in the same column.  When the cursor reaches the bottom of the  screen, the synthesizer will say "BOTTOM OF SCREEN" and remain  on the bottom line in the same column.  The "Bottom Of Screen"  message will always be spoken, even when all the Help Words are  turned off.    ENABLE READER 4.0         OPERATIONS MANUAL         CHAPTER 4-20 USAGE:  Although the DOWN key can move you to the bottom of the screen,  it is faster to use the JUMP key, the Y key (moves to a line),  or the GOTO key to move to a specified line.  The Y key and the  GOTO key will place you on the line you want immediately, yet  sometimes more than one line will need reviewing.  When reading  a section of the screen, it may be faster to move to the end of  the current line by using the ENDLINE key, and to use the DOWN  key and the LINE key combination to read lines as you move the  pointer down the screen, than to redefine the Window Keys.    CROSS INDEX:  Y, Goto, Up:  Section 4.3 Cursor Movement Keys. Jump:  Section 4.4 Reading Keys.    KEYNAME:  UP  KEYSTROKE:  LETTER U as in UNITED  KEYTYPE:  SINGLE LEVEL  SUMMARY:  MOVE THE POINTER UP ONE LINE UNTIL TOP OF SCREEN  DESCRIPTION:  The UP Key works exactly like the DOWN Key, except when it  reaches the top of the screen the synthesizer says "TOP OF  SCREEN" and remains on the top line in the same column.  The  "Top Of Screen" message will always be spoken, even when all the  Help Words are turned off.    USAGE:  It is sometimes faster to move the cursor Home and come down a  few lines, than to use the UP key.  The Y key and the GOTO key  will place you on the line you want immediately, yet sometimes  more than one line will need reviewing.  When reading a section  of the screen, it may be faster to move to the end of the  current line by using the ENDLINE key, and to use the UP key and  the LINE key combination to read lines as you move the pointer  up the screen, than to redefine the Window Keys.    CROSS INDEX:  Y, Goto, Down:  Section 4.3 Cursor Movement Keys. Jump:  Section 4.4 Reading Keys.    ENABLE READER 4.0         OPERATIONS MANUAL         CHAPTER 4-21 KEYNAME:  HOME  KEYSTROKE:  LETTER H as in HOTEL  KEYTYPE:  SINGLE LEVEL  SUMMARY:  MOVE POINTER TO TOP LEFT OF VIDEO SCREEN  DESCRIPTION:  The HOME Key will place the pointer in the upper left corner of  the screen.  The Keyword "HOME" will be spoken unless the HELP  WORDS key is toggled into NO HELP WORDS.    USAGE:  The HOME key is used when a screen with mixed data formats  requires a line-by-line, top-to-bottom analysis of how the data  is printed on the video screen.  A typical sequence would be to  Home the pointer, and move it out to the end of the line with  the ENDLINE key.  Then, by using the LINE key to read a line,  and the DOWN key to move the pointer down the right side of the  video screen, you can read each line of data.  If you hear a  number and information, and another number out of sequence  followed by information, it could be columns of data.    CROSS INDEX:  Help Words:  Section 4.2 Help Keys.    KEYNAME:  RETURN  KEYSTROKE:  LETTER R as in RODEO  KEYTYPE:  SINGLE LEVEL  SUMMARY:  MOVE THE POINTER TO THE BEGINNING OF THE CURRENT LINE  DESCRIPTION:  The RETURN Key will move the pointer to the beginning of the  current line the pointer is on.  The Keyword "RETURN" will be  spoken unless the HELP WORDS key is toggled into NO HELP WORDS.    USAGE:  The RETURN key is used whenever you have located something to be  reviewed that is closer to the beginning of the line than to the  end.  It is faster to return to the beginning, and use the WORD  key to read a word, than to back up through a line with the  WORDBACK key.  This will also give the text more meaning as it  is read forward.    ENABLE READER 4.0         OPERATIONS MANUAL         CHAPTER 4-22 Many "menu driven" programs that use single-line prompts start  at the beginning of the line.  If the line is difficult to  understand, it is necessary to review the current line from the  beginning, a word at a time.  This is done by going into the  Read Mode, typing the RETURN keystroke, and then reading the  current line with the WORD key.    CROSS INDEX:  Endline, Y:  Section 4.3 Cursor Movement Keys. Word, Wordback:  Section 4.4 Reading Keys.    KEYNAME:  ENDLINE  KEYSTROKE:  LETTER F as in FOREST  KEYTYPE:  SINGLE LEVEL  SUMMARY:  MOVE THE POINTER TO THE END OF THE CURRENT LINE  DESCRIPTION:  The ENDLINE key will move the pointer to the end of the current  line the pointer is on.  The Keyword "ENDLINE" will be spoken  unless the HELP WORDS key is toggled into NO HELP WORDS.    USAGE:  The ENDLINE key is used whenever you have located something to  be reviewed that is closer to the end of the line than to the  beginning.  It is faster to move to the end of the line, than to  use the WORD key to read through the line to the end.  Also,  many "menu driven" programs use single-line prompts that ask the  user for input at the end of the line.  It is easy to review the  current line by going into the Read Mode, typing ENDLINE, and  then reading the current line with the LINE key.    Many times it will require the reading of a line above or a line  below the current line to gain continuity of text.  Once the  pointer is at the end of the line, use the UP or DOWN keys to  move the cursor to a new line, and then execute the LINE  function to read the new line.    CROSS INDEX:  Return, Up, Down:  Section 4.3 Cursor Movement Keys. Line, Endpage, Jump:  Section 4.4 Reading Keys.    ENABLE READER 4.0         OPERATIONS MANUAL         CHAPTER 4-23 KEYNAME:  ENTER LINE  KEYSTROKE:  LETTER Y as in YELLOW  KEYTYPE:  MULTI-LEVEL  SUMMARY:  MOVE POINTER TO THE BEGINNING OF THE SPECIFIED LINE  DESCRIPTION:  The Y key is a two level key that prompts for the line you want  to move the pointer to.  After typing the Keystroke  Y , you  will be prompted with "ENTER LINE".  Enter the two digits of the  line you want to move the cursor to.  They will be spoken as  they are typed.  As with the JUMP key, digits 0 through 9 must  be preceded by a 0 (zero).  The cursor will move to the  specified line, and speak its position if the Help Word Level is  set to ALL WORDS.  If you make an input error or type a line  number that is too large, the message "Out Of Range" will be  spoken and the cursor will remain at its current position.    USAGE:  The Y Key is one of a set of two keys that divides the video  screen into an X-Y cross known as a Descartes Plane.  The Y Axis  runs vertically up and down the screen, and corresponds to the  Lines.  The X Key is the second key of the set, and the X Axis  runs horizontally across the screen.  It runs left-to-right, and  corresponds to the Columns.    The two keys can be used to place the pointer at any point on  the screen by Line and Column.  The Y Key will prompt you for  input with "Enter Line".  Then, if entered correctly, will move  the pointer to the beginning of the specified line.  If the Help  Word Level is set to ALL WORDS, the pointer position will be  spoken.  The Y key does not read the line selected.  The JUMP  function is used to move to a specified line and read it.    The Y Key is used most with databases and spreadsheets to move  the cursor to the beginning of a menu or column of data that  starts at the beginning of the line.  Then the Window Keys are  used for reading columns; or the WORD key will read data a word  at a time, starting from the beginning of the line.    CROSS INDEX:  X, Goto:  Section 4.3 Cursor Movement Keys. Jump, Window Read 6, 7, 8:  Section 4.4 Reading Keys.    ENABLE READER 4.0         OPERATIONS MANUAL         CHAPTER 4-24 KEYNAME:  ENTER COLUMN  KEYSTROKE:  LETTER X as in XRAY  KEYTYPE:  MULTI-LEVEL  SUMMARY:  MOVE THE POINTER TO THE SPECIFIED COLUMN ON

THE           CURRENT LINE  DESCRIPTION:  The X Key is a two level key that prompts for the column on the  current line you want to move the pointer to.  After typing the  Keystroke  X , you will be prompted with "ENTER COLUMN".  Enter  the two digits of the column you want to move the cursor to.   They will be spoken as they are typed.  As with the JUMP key,  digits 0 through 9 must be preceded by a 0 (zero).  The cursor  will move to the specified column of the current line, and speak  its position if the Help Word Level is set to ALL WORDS.  If you  make an input error or type a line number that is too large, the  message "Out Of Range" will be spoken and the cursor will remain  at its current position.    USAGE:  The X Key is one of a set of two keys that divides the video  screen into an X-Y cross known as a Descartes Plane.  The Y Axis  runs vertically up and down the screen, and corresponds to the  Lines.  The X Key is the second key of the set, and the X Axis  runs horizontally across the screen.  It runs left-to-right, and  corresponds to the Columns.    The two keys can be used to place the pointer at any point on  the screen by Line and Column.  The X Key will prompt you for  input with "Enter Column".  Then, if entered correctly, will  move the pointer to the specified column on the current line.   If the Help Word Level is set to ALL WORDS, the pointer position  will be spoken.    The X Key is used to move the pointer to the start of columns of  data, as in a database or spreadsheet; and then you can use the  Window Keys to read the section of the screen containing the  column.    CROSS INDEX:  Y, Goto:  Section 4.3 Cursor Movement Keys. Jump, Window Read 6, 7, 8:  Section 4.4 Reading Keys.    ENABLE READER 4.0         OPERATIONS MANUAL         CHAPTER 4-25 KEYNAME:  GOTO  KEYSTROKE:  LETTER G as in GENIE  KEYTYPE:  MULTI-LEVEL  SUMMARY:  MOVE POINTER TO SPECIFIED LINE AND COLUMN  DESCRIPTION:  The GOTO Key is a three level key that is a combination of the Y  key and the X key.  After typing the Keystroke  G , you will be  prompted with "ENTER LINE".  Upon entering a valid line number,  you will then be prompted for the column with "ENTER COLUMN".   If both inputs are valid, the cursor will move to the specified  line and column, and then speak its position if the Help Level  is set to ALL WORDS.    Both the queries for Line and Column must be two digits.  Digits  0 through 9 must be preceded by a 0 (zero).  If you enter an  invalid number or make an input mistake, the message "Out Of  Range" will be spoken.  If the mistake occurs during the Line  input, the cursor will remain in its original position.  If the  error occurs during the Column input, the cursor is positioned  at the beginning of the valid line.  At this point, you can move  to the desired column by using the X key.    USAGE:  The GOTO Key is used with programs you have been using, and know  the position of columns of data, menus, or command lines.  Once  in position, many of the Speaking Keys are used to read words,  lines, or windows of data.    CROSS INDEX:  Y, X:  Section 4.3 Cursor Movement Keys. Jump, Window Read 6, 7, 8:  Section 4.4 Reading Keys.    END OF   SECTION 4.3  CURSOR MOVEMENT KEYS #############################################################################################################