ENABLE READER 4.0        OPERATIONS MANUAL         CHAPTER 4-1                        CHAPTER 4  FUNCTION KEYS    CONTENTS    4.0  Introduction  4.1  Using This Chapter Of The Manual  4.2  Help Keys  4.3  Cursor Movement Keys  4.4  Reading Keys  4.5  Command Keys  4.6  System Keys      4.0  INTRODUCTION    In order to run application programs in any computer, the   machine requires a program that will provide a means to   communicate between its various parts.  This program is called   an Operating System.  For example, the keyboard allows   characters to be typed, and the video screen allows them to be   viewed or spoken.  The operating system connects, or in computer   jargon "interfaces", these two parts together with a small   routine called the Monitor.      Generally, a Screen Review program links to the Operating   System, allowing what is typed or printed to the video screen to   be sent to a special routine that examines it before sending it   to the synthesizer.  Since the Enable Reader 4.0 program must   link with the Operating System, it seemed that the three year   evolution of the project would eventually lead to the total   integration of the Enable Reader System and the MSDOS Operating   System.  This makes MSDOS the world's first programmable talking   Operating System.      The Enable Reader Speech System is actually two programs in   one.  The Full Screen Speech Review System is designed for   visually impaired and blind people.  With Full Screen Speech   Review, a visually impaired person has access to the wide   selection of commercial software available for MSDOS compatible   machines, such as the IBM PC and dozens of other machines.     The second program built into the Enable Reader 4.0 Speech   System is the Talking Tutorial Programmer's Aid.  The   professional programmer now has access to many of the speech   functions of the Enable Reader 4.0 Speech System for writing   talking tutorials for software.  This gives blind and visually   impaired people the best of both worlds.  One, they have access   to the Review Mode of the Enable Reader 4.0 Speech System for   running commercial software; and two, any program that is   written with the Talking Tutorial Programmer's Aid will run for   them.                 ENABLE READER 4.0        OPERATIONS MANUAL         CHAPTER 4-2   The Enable Reader program is now designed to be a "Programmable   Resident Operating System Extension".  What "programmable" means   is that it is actually a part of the Operating System, and can   be used by programmers to write software that speaks by   accessing functions that you use in the Speech Review System.    It is "resident" in that the entire program is loaded into   memory at one time, and does not use time-consuming reads from   the disk drives to obtain parts necessary for its operation.  It   is an "extension" in that it extends the MSDOS Operating System   with a set of speech routines that are accessed through the same   supervisor call interrupts used by MSDOS or through keyboard enhancemant program such as NEWKEY or SUPERKEY.  When in the Review  Mode (where data can be reviewed from the video screen), the  Enable Reader 4.0 System is a stand-alone Operating System.    It may be easier to understand just how the Enable Reader   program is accessed and how it operates, if you consider it to   be a separate invisible Operating System that can be called from   the MSDOS Operating System or an applications program at any   time.      When running the Operating System or an applications program,   you can type a simple toggle (a key that switches between two   states of operation) to enter and exit the invisible talking   operating system -- the Enable Reader 4.0 Speech System.  The   video screen and memory are locked down for your security while   you review the data on the video screen.  This way, nothing can   be changed in memory or on the screen while reviewing.      The two modes of operation are the INPUT MODE and the READ MODE.    The Input Mode is the side of the program where the operating   system and application programs run.  The Read Mode is the   Speech Review System where there are five categories of   functions used for screen review.  The CONTROL and LEFT OR RIGHT SHIFT  key combination will toggle you between the two modes.  You must  hold down the  CONTROL  key first, and then type the  LEFT OR RIGHT SHIFT  key.    Another method of using the Review Functions is to use the Silent Macro Mode entry point.  Unlike the CONTROL SHIFT (HARDWARE INTERRUPT) method, the Silent Macro Mode is an ASCII or typeable character. Used with keyboard enhancement programs you can create unlimited functions that will automatically enter the READ MODE review the data and EXIT.  The key to type when setting up your own macros is CONTROL RIGHT BRACKET.  When you need to review the macro program prompts, use the hardware interrupt method of CONTROL LEFT OR RIGHT SHIFT.  Having both of these access methods, Enable Reader runs with more software than any other screen review system without them.               ENABLE READER 4.0        OPERATIONS MANUAL         CHAPTER 4-3   There are three key combinations that will enter and exit the  READ MODE.  They are:        Control Right-Shift,      Control Left-Shift,      Control Right-Bracket.  THIS IS A RIGHT BRACKET  ]     The Control Right Bracket  ^]   combination is compatible with both Kurtzweil 3000 and Votan VPC 2000 speech recognition systems.   When entering either mode, the synthesizer will announce it.    For example, when entering the Read Mode the synthesizer will   say  READ DATA , or when entering the Input Mode it will say    INPUT DATA.      All of the function keys are accessible only in the Read Mode.    The keys are NOT "case dependent", which means they can be typed   as upper or lower case letters.  Each function will perform its   task, and then return to the Read Mode Monitor where you may   type another function.  You will remain in the Read Mode until   you type  CONTROL  and  LEFT OR RIGHT SHIFT  to exit to the Input Mode.     Some of the keys affect both the Read Mode and the Input Mode.    For example, the AUTO key when toggled off will put the entire   Review System into "Spell Mode".  All of the Reading Keys, such   as WORD, LINE, or PAGE, will speak all text as single   characters.  When in the Input Mode, all text that is typed will   be echoed immediately as single characters.  All of the   functions and details of their operation will be described in   the following sections of this chapter.       KEYBOARD KEY ENUNCICATION  The Enable Reader Professional Speech System has identification of the keyboard keys in the INPUT MODE. The speaking of the keyboard keys are tied to the help levels and with only a few exceptions will be spoken unless the HELP LEVEL is set to no help.    The Backspace key will speak its name and the last character  typed on the screen.  If pressed more that once it will only say  the word BACKSPACE.    The Carriage Return or ENTER KEY will not speak as it is not  necessary once the position of the key is known.  It was  eliminated by the beta testers due to the constant use and  annoying speech.    The function keys now speak their names as F1, F2, etc.   The Alternate letter keys speak.  When the ALT key and a letter  are type the word ALT and then the letter will be spoken.             ENABLE READER 4.0        OPERATIONS MANUAL         CHAPTER 4-4   The Escape, Delete, and Tab keys speak their names.    Many of the other keys are toggles or are toggled on when the NUM LOCK key is on.  When the following keys are typed they will  announce whether they are toggled on or off.    CAPS LOCK          INSERT        NUM LOCK          SCROLL LOCK   When the NUM LOCK key is toggled on the following keys on the ten key pad will speak the following when typed.        KEYSTROKE           PHRASE SPOKEN       HOME                HOME      END                 END      PGUP                PAGE UP      PGDN                PAGE DOWN      LEFT ARROW          LEFT      RIGHT ARROW         RIGHT      UP ARROW            UP      DOWN ARROW          DOWN     4.1  USING THIS CHAPTER OF THE MANUAL    There are five sets of Home Based Function Keys described in this section of the manual.  The Integration keys are so different they are described in their own chapter.     The five sets of keys are:             1 HELP KEYS          2 CURSOR MOVEMENT KEYS          3 READING KEYS          4 COMMAND KEYS          5 SYSTEM KEYS    Each Set of Keys will be described at the beginning of that   respective Key Set.  In addition, there are appendices that   contain the word definitions of the Enable Reader 4.0   Punctuation Set, and the Letter-To-Word Translator List of   words.      The format of describing each key is divided into seven   sections, described below:         SECTION 1     KEYNAME         The name of the key is the one or two word descriptor of        its function.  For example, the WORD key will read a word,        or the LINE key will read a line.                  ENABLE READER 4.0        OPERATIONS MANUAL         CHAPTER 4-5        SECTION 2     KEYSTROKE         This is the actual key that must be pressed to initiate the        function you choose to execute.  If it is a Control Key        function, you must first press the  CONTROL  key, and while        holding it down, type the secondary key -- such as CONTROL A.             SECTION 3     KEYTYPE         There are four "types" of keys:              1 SINGLE LEVEL:  A key that executes its function

immediately, and then returns to the Read Mode                    Monitor.                2 MULTI-LEVEL:   A key that requires user input; and                    when completed, returns to the Read Mode                    Monitor.                3 USER DEFINED:  A key that requires the user to first                    run a function that will define the key for                    operation.                4 TOGGLE:        A key that acts the same as an ON/OFF                    switch, or if it has more than two levels, will                    continue through the different levels back to                    the original level.             SECTION 4     SUMMARY         The Summary of a key gives a brief description of the        function performed, while the Description of the key below        gives complete details of how to operate the Key Function.           SECTION 5     DESCRIPTION         This section attempts to give the reasoning behind the        Function Key, how it is integrated into the Read Mode        System, and what special features it may have when used in        combination with other keys.             SECTION 6     USAGE         The Usage section gives an example of how the function        could be used in combination with other keys, what it is        primarily used for, and if there are other methods of        accomplishing the same task.                  ENABLE READER 4.0        OPERATIONS MANUAL         CHAPTER 4-6        SECTION 7     CROSS INDEX         The Cross Index section defines those keys that may be        affected by the use of the defined key, or other        combinations that may be used with it.        There are many different ways to combine keys so they will   perform the same function.  The use of the Enable Reader System   to achieve a level of professional competence will require   experimentation and usage in your own computer environment.      The methods of reviewing are so various and dependent on your   applications that we can not give you a strict definition of how   and when to use specific setups with a specific applications   program.      There are literally hundreds of thousands of programs, and   millions of visually impaired and blind people worldwide.  What this  section of the manual attempts to do is to give general rules of thumb for running most software applications we have encountered to date.  Hopefully, you will write the Enable Talking Software Company and share your knowledge with us so we may pass it along in a newsletter on national databases.      4.2  HELP KEYS    CATEGORY DESCRIPTION:    The HELP Functions were the first keys to be defined in the   original Enable Reader 1.0 Prototype Version.  During the course   of the past three years they have been refined and extended.    The thirteen  Help Keys are now an integrated part of the Read Mode System, and when turned on or off, affect other functions that are used with them.  The following Help Keys are written in   alphabetical order.       KEYNAME:  ATTRIBUTE IDENTIFICATION   KEYSTROKE:  CONTROL G AS IN GENIE   KEYTYPE:  TWO LEVEL TOGGLE    The ATTRIBUTE Key is a toggle, which means it works the same as a two way switch.  Type CONTROL G once to TURN ON ATTRIBUTEIDENTIFICATION.  If you type it again ATTRIBUTE IDENTIFICATIONWILL BE TURNED OFF.      SUMMARY:  TURNS ON OR OFF ATTRIBUTE OR COLOR IDENTIFICATION.               ENABLE READER 4.0        OPERATIONS MANUAL         CHAPTER 4-7   DESCRIPTION:   The ATTRIBUTE function  will help users to run programs such  as LOTUS 123, DBASE III, WORD PERFECT,  and HYPERACCESS.  Many programs require that you know that a field or command is highlighted, in reverse video, underlined, color blue on a white backgound, or combinations of foreground and background colors in order to proceed with the program.  The ATTRIBUTE IDENTIFICATION KEY, gives you that information for most of the possible ATTRIBUTE combinations available in the monochrome mode and all 128 foreground/background color combinations in the color mode.   ATTRIBUTE IDENTIFCATION is toggled on and off by typing the  CONTROL G KEY.  It may be easier to remember this if you think Of  G for graphics.  The prompt spoken is "attributes or color on" when the key is active and "attributes or color off" when it is inactive.  The Attribute Identification function for color works  in the review mode and in the INPUT mode.    Many programs write the character and attribute colors separately  and this  can lead to erroneous identification in the INPUT mode. It is best to use color identification in the screen review mode until a sighted person can verify the colors are correct as you  type them.   The attribute bytes in the color graphics mode give the names of  the colors instead of the monochrome values.  The five available attributes are Normal, Reverse Video, Underlined, Blinking, and High Intensity. The following are the possible combinations and the phrases spoken:   MONOCHROME MODE        ATTRIBUTES                    PHRASE SPOKEN        Normal                        No Phrase Spoken      Reverse Video                 Reverse      Blinking                      Blinking      High Intensity                High      Underlined                    Underlined      Reverse Video-Blinking        Reverse Blinking      Underlined-High Intensity     Underlined-High      Reverse Video-High            Reverse-High        Intensity      Blinking-Underlined           Blinking-Underlined      Blinking-Underlined-High      Blinking-Underlined-high        Intensity    COLOR MODE  NOTE: IN COLOR MODE THE FIRST COLOR SPOKEN IS THE FOREGROUND        COLOR AND THE SECOND IS THE BACKGROUND COLOR.           ENABLE READER 4.0        OPERATIONS MANUAL         CHAPTER 4-8       16 FOREGROUND COLOR ATTRIBUTES   (SAME PHRASE IS SPOKEN)	              BLACK         BLUE             GREEN            CYAN     RED           MAGENTA          BROWN            LIGHT GREY     DARK GREY     LIGHT BLUE       LIGHT GREEN      LIGHT CYAN     LIGHT RED     LIGHT MAGENTA    YELLOW           WHITE                8 BACKGROUND COLOR ATTRIBUTES    (SAME PHRASE IS SPOKEN)                     BLACK         BLUE             GREEN            CYAN     RED           MAGENTA          BROWN            LIGHT GREY  USAGE:  The Attributes or Color key will automatically check to see whether the program you are running is in monochrome or color.  Many programs overlay screens on top of each other that are different colors with different colored characters and some word processors will put characters in different colors depending on whether they are to be printed as bold, underlined, etc.  Other programs will require that you know some prompt is in reverse video or high intensity to make a menu choice.  Hopefully, the documentation of the program will indicate this.  Most documentation is so poorly written that you may need a sighted person to teach you how the program is running visually.  CROSS INDEX:   NONE    KEYNAME:  AUTO    KEYSTROKE:  LETTER A as in ALIEN    KEYTYPE:  TWO LEVEL TOGGLE    The AUTO Key is a toggle, which means it works the same as a two   way switch.  Press the key once to have Spell Mode, and again to   return it to Whole Word Mode.      SUMMARY:  TURNS ON OR OFF WHOLE WORD OR SPELL MODE    DESCRIPTION:    The AUTO toggle has two states of operation.  Each state will   announce when it is entered.  One, the AUTO ON state, or Whole   Word Mode, is set when the Enable Reader program is first   loaded.  The second is the AUTO OFF state, or Spell Mode, that   is entered when the  A  key is pressed.                ENABLE READER 4.0        OPERATIONS MANUAL         CHAPTER 4-9   In the Whole Word Mode, all blocks of letters, numbers, and   punctuation between two spaces will be spoken as one word.  For   example, if the letters are the word  COMPUTER , the synthesizer   will say  COMPUTER .  If there is a quote mark immediately   before and after the word COMPUTER, the synthesizer will say   QUOTE COMPUTER QUOTE.      In the Spell Mode, all characters are spoken individually.  This   means that when a WORD contains letters, they will be spelled to   you:  COMPUTER  will be spoken  C O M P U T E R .  If there are   numbers or punctuation, they will be spoken as whole words.  For   example,  "COMPUTER"  will be spoken as QUOTE C O M P U T E R   QUOTE.  If the word contains a punctuation character, it will   always be spoken as a whole word, and will not be spelled to   you.      The setting of the Spell Mode will not only affect the Read   Mode, but will also configure the Input Mode to immediately   speak all typed characters, including numbers and punctuation.    This effect is called "echoing".      USAGE:    The Spell Mode is used to spell out hard-to-understand words or   groups of letters.  Once it is turned on, it affects all of the   Reading Keys.  With AUTO OFF, the WORD key will spell a word,   the LINE key will spell a line, et cetera.  If you are reading a   directory of files on a disk and can not understand a file name,   it may be useful to turn off the AUTO function, thereby putting   the Reading Keys into Spell Mode; then go back and review.  When   used with the Window Keys, you can define the filename column as   a window and have each line of the directory spelled.      It is also important to keep track of whether the Spell Mode key   is ON or OFF, as the prompts in an application program will be   spelled to you.  If you forget to turn off the Spell Mode and   type the PAGE or ENDPAGE function, it will spell an entire

screen, thereby hanging up the synthesizer and the computer for   minutes while the data is sent.  As a precaution, there are QUIT   functions built into PAGE and ENDPAGE that will let you escape   and stop the synthesizer speech.     CROSS INDEX:    All Keys in Section 4.4 Reading Keys.  Note:  Escape procedures in Page and Endpage functions.                      ENABLE READER 4.0        OPERATIONS MANUAL         CHAPTER 4-10   KEYNAME:  CURSOR AT    KEYSTROKE:  LETTER C as in CEDAR    KEYTYPE:  SINGLE LEVEL    SUMMARY:  SPEAKS READ MODE CURSOR POSITION BY LINE AND COLUMN    DESCRIPTION:    The CURSOR AT Key will give you an exact screen location by line   number and column number.  The screen is divided into 25 lines   by 80 columns.  Lines are read from the top to bottom of the   screen, and always start with line 0 (zero).  The bottom line is   line 24.  Columns are read from the left to the right, and start   with column 0 (zero).  The far right margin is column 79.      USAGE:    The CURSOR AT function is typically used to identify locations   of columns or command lines that are used consistently in a   program.  Once they are located, the JUMP key can be used to   jump to a line and read it.  Alternately, the WINDOW keys can be   defined to read a section of the screen in columns or   paragraphs.      CROSS INDEX:    Window Setup:  Section 4.4 Reading Keys.  Jump, Y, X:  Section 4.3 Cursor Movement Keys.     KEYNAME:  IDENTIFY    KEYSTROKE:  LETTER I as in INDIA    KEYTYPE:  MULTI-LEVEL    SUMMARY:  IDENTIFIES READ MODE FUNCTIONS WITH BRIEF DESCRIPTION    DESCRIPTION:    The IDENTIFY Key is designed to give you a short description of   each function key while you are in the Read Mode.  To use the   IDENTIFY Key, first type the letter  I , then any key on the   keyboard.  The key will speak its function, or say  NOT A   FUNCTION KEY.  The IDENTIFY key will only speak one key function   definition, and then return you to the Read Mode Monitor.  For   the full text of the definitions of each key, see the section on   IDENTIFY Key descriptions.                  ENABLE READER 4.0        OPERATIONS MANUAL         CHAPTER 4-11   USAGE:    The IDENTIFY function is used to learn the different functions,   or to review a function before using it so you do not change   screen position inadvertantly.      CROSS INDEX:    None.        KEYNAME:  FILTER PUNCTUATION    KEYSTROKE:  LETTER M as in MIKE    KEYTYPE:  MULTI-LEVEL    SUMMARY:  USER DEFINED PUNCTUATION CHARACTER FILTER   DESCRIPTION:    The FILTER PUNCTUATION function provides for selectively turning   off and on individual punctuation characters.  The function will   prompt you to enter the punctuation character, and then will   announce whether it is filtered on or off.  The filter is   affected by the PUNCTUATION LEVEL function below.      If you want to filter on or off a character, type  M  and wait   for the  ENTER PUNCTUATION  prompt.  Next type any punctuation   character and listen to whether it is filtered on or off.  The   function acts like a toggle, so that if you type the same   character again it will give you the opposite message.  If the   character is not a punctuation character, the message  NOT   PUNCTUATION  will be spoken, and you will return to the  ENTER   PUNCTUATION  prompt.  After the character is identified as on or   off, you will be prompted again to  ENTER PUNCTUATION .  As many   punctuation characters as is necessary to be filtered may be   entered after typing  M  to use the FILTER PUNCTUATION function.    To exit the function, just type the  RETURN  key.  The  END OF   FILTER  message will be spoken, and you will return to the Read   Mode Monitor.      USAGE:    The FILTER PUNCTUATION function is used whenever there are long   strings of punctuation characters used in application menus or   when accessing databases.  For sighted people, these strings of   equal signs or minuses are used to separate columns or rows of   data, and by filtering them off you will not have to listen to   80 equal signs between lines.              ENABLE READER 4.0        OPERATIONS MANUAL         CHAPTER 4-12   This method of filtering was preferred over the counting method, whereby the 80 characters  are counted and a total given.  Because when you know that a program has lines that you do not want to hear, there is no need  to be reminded of this again and again as you run the same  program year in and year out.      CROSS INDEX:    Punctuation Level:  Section 4.2 Help Keys.  Enable Reader 4.0 Punctuation Set:  Appendix A.     KEYNAME:  PUNCTUATION LEVEL    KEYSTROKE:  LETTER N as in NATIVE    KEYTYPE:  FIVE LEVEL TOGGLE    SUMMARY:  SELECTS BLOCKS OF PUNCTUATION CHARACTERS TO FILTER             OUT, OR ALLOWS DIRECT OUTPUT OF ASCII PUNCTUATION             CODES TO THE SPEECH SYNTHESIZER.      DESCRIPTION:    To provide a means of moving from one computer to another, the   Home Based Keys were used.  In order to transfer between   synthesizers without having to learn a new punctuation set for   each machine, the Enable Reader System uses one set of   punctuation for all machines.  Some synthesizers require that   punctuation characters be used for sending commands to change   their parameters.  The punctuation must be turned off to allow   the specific character to be transmitted, instead of the word   that Enable Reader translates it into.  The  N  key has five   levels of punctuation. They are ALL, MOST, SOME, NONE, and   direct OUTPUT TO SYNTHESIZER.  Each time the  N  Key is typed,   it moves down one level until it reaches OUTPUT TO SYNTHESIZER,   where it returns to the top level of ALL PUNCTUATION.      The five levels of punctuation and their prompts are:         1 ALL:   All punctuation characters will be spoken.                PROMPT: ALL PUNCTUATION         2 MOST:  All punctuation except  [ \ ] ^ _ ` { | }  (Ascii                Characters # 92 - 127)  will be spoken.                PROMPT: MOST PUNCTUATION         3 SOME:  All punctuation except those filtered off by MOST                and , - . / : ; < = > ? @   (Ascii Characters # 44                - 127)  will be spoken.                PROMPT: SOME PUNCTUATION         4 NONE:  No punctuation will be spoken, and the FILTER                PUNCTUATION function is not applicable.                PROMPT: NO PUNCTUATION           ENABLE READER 4.0        OPERATIONS MANUAL         CHAPTER 4-13        5 OUT:   All punctuation bypasses the Enable Reader System                Punctuation Set, and is sent as Ascii characters                to the speech synthesizer.                PROMPT: PUNCTUATION OUT    USAGE:    The PUNCTUATION LEVEL function is used to quickly filter out   blocks of punctuation characters that are found to be an   annoyance when reading files or accessing databases with a   modem.  The most used punctuation characters in menu strings can   be turned off by using the SOME PUNCTUATION setting, as it turns   off the minus, period, equal sign, question mark, and comma.      CROSS INDEX:    Filter Punctuation:  Section 4.2 Help Keys.  Enable Reader 4.0 Punctuation Set:  Appendix A.        KEYNAME:  HELP WORDS    KEYSTROKE:  LETTER O as in OPEN    KEYTYPE:  FOUR LEVEL TOGGLE    SUMMARY:  SELECTS LEVEL OF KEYNAMES AND PROMPTS TO BE SPOKEN BY             FUNCTION KEYS     DESCRIPTION:    All of the Reading Keys have words that describe their function,   and then will execute it.  The Multi-Level keys have descriptive   prompts that ask for input or give an error message.  The   Pointer Movement Keys will speak their Keyname and execute their   functions.  If they try to move off the screen, the  TOP OF   SCREEN  or  BOTTOM OF SCREEN  may be spoken.     Some of the function keys have descriptive words that are   primarily used when first beginning to use the program.  They   will speak the KEYWORD and then execute it.  For example, type   the STEP Key and it will speak  STEP , and then execute its   function.  If the HELP WORDS are off, it will not say  STEP ,   but only execute its function.      The HELP KEYS toggles will always speak their KEYNAMES when they   are typed.  The multi-level HELP keys can have their Keynames   and Prompts for keyboard input turned off.  The HELP Levels are   designed so that the functions can be used more efficiently and   not break your concentration.                 ENABLE READER 4.0        OPERATIONS MANUAL         CHAPTER 4-14   The four levels of HELP WORDS and their prompts are:         1 All:   All HELP WORDS, KEYNAMES, PROMPTS and RANGES are                spoken.                PROMPT: ALL HELP WORDS         2 MOST:  The KEYNAMES of READING KEYS, MULTI-LEVEL POINTER                MOVEMENT KEYS, and USER DEFINED SETUP KEYS are not                spoken.                PROMPT: MOST HELP WORDS         3 SOME:  The POINTER MOVEMENT KEYNAMES, such as UP and                DOWN, are not spoken.                PROMPT: SOME HELP WORDS         4 NONE:  Only the quit key speaks the word QUIT and Disk                    access errors are spoken.                PROMPT: NO HELP WORDS    USAGE:    The reasoning behind having levels of prompts is that after you   learn to use the function keys to locate data and read the   screen, it is more efficient, and there is more of a continuous   flow, if the STEP Key only speaks a character and then moves   ahead one, instead of saying  STEP letter STEP letter STEP   letter.  By typing the HELP WORDS Key and toggling to  MOST HELP   WORDS , none of the Reading Keys' or Cursor Movement Keys'

keynames will be spoken.      Try using the different functions individually at every HELP   WORD level to learn what names and prompts are left on or turned   off at each level.      CROSS INDEX:    None.        KEYNAME:  UPCASE    KEYSTROKE:  LETTER T as in TEACUP    KEYTYPE:  TWO LEVEL TOGGLE    SUMMARY:  TURNS ON OR OFF UPPER CASE LETTER RECOGNITION    DESCRIPTION:    The UPCASE function examines every letter to determine if it is   upper or lower case.  If it is upper case, the word  UPCASE    will be spoken before each upper case letter.  When the first   letter of a word is capitalized, the word will be spoken as    UPCASE word.  For example, if the word  Computer  is   capitalized, it will be spoken as UPCASE COMPUTER.            ENABLE READER 4.0        OPERATIONS MANUAL         CHAPTER 4-15   The UPCASE function will identify upper case letters in both the   Read Mode and Input Mode.  When used in combination with the   TRANSLATOR function, the UPCASE function will take precedence,   speak  UPCASE , then translate the letter.      USAGE:    When entering data or running an application program, it is   sometimes necessary to enter upper case (capital) letters.  The   beginning letter of every sentence is capitalized.      CROSS INDEX:    Page, Endpage:  Section 4.4 Reading Keys.   KEYNAME:  TRANSLATOR    KEYSTROKE:  LETTER Z as in ZEBRA    KEYTYPE:  TWO LEVEL TOGGLE    SUMMARY:  TURNS ON OR OFF LETTER-TO-WORD TRANSLATOR    DESCRIPTION:    The TRANSLATOR function is used when it is necessary to   distinguish between hard-to-hear letters.  It only translates   letters, since numbers and punctuation are always spoken as   whole words.  When the TRANSLATOR function is turned on, all the   READING KEYS will translate letters to words from a table of   words used for this function.  For example, the WORD function   will translate the word  DATE  as  DELTA ALIEN TEACUP ECHO .  If   you leave the TRANSLATOR function on, and use the PAGE function   to read the whole video screen, it could take minutes for the   screen to be sent to the synthesizer.  The PAGE function has an   escape key that allows you to exit the PAGE function and stop   the synthesizer speech by sending the QUIT function.  The   Translator function works in both the Read Mode and the Input   mode.      USAGE:    The TRANSLATOR function was developed by a blind administrator   at the University Of California, Berkeley campus when he was   having a difficult time first using a speech synthesizer, and   could not understand certain letters.  We chose not to support   the Ham Radio Standard Word Translation List, due to the fact   that many synthesizers will pronounce the same words   differently.  It took many hours of patiently trying many   translation words with different synthesizers before this set   was chosen.      CROSS INDEX:    Page, Endpage:  Section 4.4 Reading Keys.  Letter-To-Word Translator Table:  Appendix B.          ENABLE READER 4.0        OPERATIONS MANUAL         CHAPTER 4-16   KEYNAME:  SPACE IDENTIFICATION    KEYSTROKE:  SLASH KEY    KEYTYPE:  TWO LEVEL TOGGLE    SUMMARY:  TURNS ON OR OFF SPACE CHARACTER IDENTIFICATION    DESCRIPTION:    The SPACE function will turn on or off the space character   identification.  When the SPACE function is on, all spaces will   be spoken when using the Reading Keys.  Since both the WORD and   WORDBACK keys ignore spaces by jumping to the first letter of   the word, they will not identify them.  As with the UPCASE   function, the use of the PAGE function with the SPACE   IDENTIFICATION on will hang up the speech synthesizer for many   minutes.  The PAGE function has an escape key that allows you to   exit the PAGE function and stop the synthesizer speech by   sending the QUIT function.  The SPACE IDENTIFICATION function   works in both the Read Mode and the Input Mode.      USAGE:    Space identification is necesary when absolute accuracy is   required, as when typing in commands where syntax is critical.    It is also used when combining or editing text, and the document   must be visually correct.      CROSS INDEX:    Page, Endpage:  Section 4.4 Reading Keys.        KEYNAME:  CONTROL IDENTIFICATION    KEYSTROKE:  CONTROL O as in OPEN    KEYTYPE:  TWO LEVEL TOGGLE    SUMMARY:  TURNS ON OR OFF CONTROL KEY IDENTIFICATION    DESCRIPTION:    The CONTROL KEY IDENTIFICATION function will identify when a   Control Key has been typed from the keyboard.  The MSDOS   Operating System will print most of the Control characters on   the screen, and with the CONTROL KEY IDENTIFICATION on, will   speak the key twice -- once when it is typed, and again when it   is printed on the screen.                 ENABLE READER 4.0        OPERATIONS MANUAL         CHAPTER 4-17   USAGE:    Control Key identification is necessary in learning to use   programs that use Control keys to execute functions.  The   Wordstar wordprocessing program is based on a Control Key   System.      CROSS INDEX:    Video:  Section 4.6 System Keys.      KEYNAME:  SPACETALK    KEYSTROKE:  CONTROL Y as in YELLOW   KEYTYPE:  TWO LEVEL TOGGLE    SUMMARY:  TURNS ON OR OFF FORCED SPEECH WHEN SPACE BAR IS PRESSED   DESCRIPTION:    The Spacetalk key is a toggle that turns on or off forced speech when the space bar is type. This feature works well when wordprocessing as it will speak each word as you type it.  This key is defaulted ON in the original defaults when the program first starts up.  USAGE:   Although the spacetalk key works well with word processors it also can be used effectively in any program that requires data input.  This key is used in conjuction with the delay key so it will not only speak when the space bar is pressed, but when you pause for a pre-selected time or when when a cariage return is pressed at the end of a paragraph.  CROSS INDEX:    Delay: Section 4.2 Help Keys    KEYNAME:  WANG TERMINAL MODE  WARNING:  DO NOT USE THIS KEY WHEN RUNNING THE DOS ALONE AS IT MAY            INTERFERE WITH INTERNAL KEYBOARD HANDLING!!!  KEYSTROKE:  CONTROL U as in UNITED    KEYTYPE:  TWO LEVEL TOGGLE    SUMMARY:  TURNS ON OR OFF KEYBOARD INTERRUPT BYPASS             ENABLE READER 4.0        OPERATIONS MANUAL         CHAPTER 4-18   DESCRIPTION:    The Wang Terminal Mode key is somewhat misnamed.  Although it is written to be used with DPZ software's WANG terminal emulation program, it is really designed to work with any program that takes control of the keyboard dynamically.  These programs typically have sections of the program that runs as a shell, a protected mode, or emulators that use separate keyboard routines for set up modes, etc.  When they take control of the keyboard they may cut off the keyboard routine from passing characters to the screen review program or even worse they can pass all the review characters typed to a mainframe.  The Wang Terminal Mode key eliminates this in most programs of this type.  Since there is no way to know how a programmer wrote their keyboard interface routines, there are no rules, but only exceptions when to use this key.  CROSS INDEX:    Type; Video:  Section 4.6 System Keys.    END OF   SECTION 4.2  HELP KEYS                                       ####