ENABLE READER 4.0         OPERATIONS  MANUAL        CHAPTER 2-1                             CHAPTER 2  GETTING STARTED  2.0  PRELIMINARY DISCUSSION  This chapter covers the procedures for setting up Enable Reader. You should do this before actually running the Enable Reader  program, which will be detailed in the next chapter.    The first topic is a description of the different versions of  the Enable Reader Professional Speech System .  Then the contentsof the Enable Reader  package is described, detailing theOperations Manual and the Enable Reader Disks, and how they aremeant to be used for learning Enable Reader.    Then there is an explanation of how to connect your speech  synthesizer to the computer, followed by the procedure to create work disks from the disks that came with the Enable Reader  package.  The last section discusses how auto execution works,  and how you can change it for your own purposes.  Once you have  completed the above sections, you are ready to actually run the  Enable Reader program.    You will just read or listen to the first part of this chapter.  When you reach  SECTION 2.4  ATTACHING THE VOICE SYNTHESIZER,  you will actually follow the instructions given from then on.     2.1  VERSIONS OF ENABLE READER PROFESSIONAL  The Enable Reader program currently supports seven speech  synthesizers:    *ARTIC BOARD          (ARTIC TECHNOLOGY)   CALLTEXT 5050        (SPEECH PLUS)   DECTALK DT C01       (DIGITAL EQUIPMENT)   ECHO GP              (STREET ELECTRONICS)  *VOTALKER IB BOARD    (VOTRAX, INC.)           VOTRAX PSS           (VOTRAX, INC.)   VOTRAX PSS/B         (VOTRAX, INC.)   *NOTE: BOTH THE ARTIC AND VOTALKER BOARDS REQUIRE THE PORTTTALK          PROGRAM FOR OPERATION. THE ENABLE READER PROGRAM USES A         SPECIAL HARDWARE INTERRUPT AS ONE METHOD OF ENTERING THE        SCREEN REVIEW MODE - CONTROL SHIFT.  THIS OPTION IS NOT           AVAILABLE WHEN RUNNING THE ABOVE BOARDS DUE TO CONFLICTS        IN THE BOARDS MEMORY RESIDENT SOFTWARE AND PORTTALK.              THIS MEANS THAT PROGRAMS SUCH AS SUPERKEY MUST USE THE        SOFTWARE INTERRUPT - CONTROL RIGHT BRACKET - AND WILL NOT         ALLOW SCREEN REVIEW ON MANY PROGRAMS THAT WRITE THEIR        KEYBOARD HANDLING ROUTINES SUCH AS SIDEKICK AND SUPERKEY.                WE STRONGLY RECCOMMEND THE PURCAHSE OF A SYNTHESIZER              ACCESSED THROUGH A SERIAL PORT. NOT ONLY ARE THEY PORTABLE         FROM MACHINE TO MACHINE, BUT ARE MORE COMPATIBLE WITH         IBM PC SOFTWARE.ENABLE READER 4.0         OPERATIONS  MANUAL        CHAPTER 2-2      There is a different version of the Enable Reader program for  each synthesizer. Each version starts up and has default controlsfor that synthesizer.  Any version of the program can run with anysynthesizer by just loading in the correct table. This can be doneeven when the program was started up with another synthesizer. You can identify which version of Enable Reader you are running  by listening to the Startup Message that is spoken when you  first execute Enable Reader.  Thus you can check whether the  version of the program matches your synthesizer.  The Startup  Message and the name of the Enable Reader program for each of  the synthesizers is given here:   SYNTHESIZER:      ARTIC BOARDPROGRAM NAME:     ARTIC.EXE  STARTUP MESSAGE:  THE ENABLE READER PROFESSIONAL SPEECH SYSTEM .                  VERSION 4.0 FOR THE ARTIC SPEECH SYNTHESIZER.                  COPYRIGHTED 1987 BY ENABLE TALKING SOFTWARE.                      ALL RIGHTS RESERVED.                  THIS IS A USER SUPPORTED PROGRAM.SYNTHESIZER:      CALLTEXT 5050PROGRAM NAME:     ENABLE_C.EXE  STARTUP MESSAGE:  THE ENABLE READER PROFESSIONAL SPEECH SYSTEM .                  VERSION 4.0 FOR THE CALLTEXT 5050 SPEECH                          SYNTHESIZER.                  COPYRIGHTED 1987 BY ENABLE TALKING SOFTWARE.                      ALL RIGHTS RESERVED.                  THIS IS A USER SUPPORTED PROGRAM. SYNTHESIZER:      DECTALK DT C01 PROGRAM NAME:     ENABLE_D.EXE  STARTUP MESSAGE:  THE ENABLE READER PROFESSIONAL SPEECH SYSTEM .                  VERSION 4.0 FOR THE DECTALK SPEECH SYNTHESIZER.                  COPYRIGHTED 1987 BY ENABLE TALKING SOFTWARE.                      ALL RIGHTS RESERVED.                  THIS IS A USER SUPPORTED PROGRAM. SYNTHESIZER:      ECHO GP PROGRAM NAME:     ENABLE_E.EXE  STARTUP MESSAGE:  THE ENABLE READER PROFESSIONAL SPEECH SYSTEM .                  VERSION 4.0 FOR THE ECHO GP SPEECH SYNTHESIZER.                  COPYRIGHTED 1987 BY ENABLE TALKING SOFTWARE.                      ALL RIGHTS RESERVED.                  THIS IS A USER SUPPORTED PROGRAM.ENABLE READER 4.0         OPERATIONS  MANUAL        CHAPTER 2-3      SYNTHESIZER:      VOTALKERPROGRAM NAME:     VOTALKER.EXE  STARTUP MESSAGE:  THE ENABLE READER PROFESSIONAL SPEECH SYSTEM .                  VERSION 4.0 FOR THE VOTALKER SPEECH SYNTHESIZER.                  COPYRIGHTED 1987 BY ENABLE TALKING SOFTWARE.                      ALL RIGHTS RESERVED.                  THIS IS A USER SUPPORTED PROGRAM.SYNTHESIZER:      VOTRAX PSSPROGRAM NAME:     ENABLE_V.EXE  STARTUP MESSAGE:  THE ENABLE READER PROFESSIONAL SPEECH SYSTEM .                  VERSION 4.0 FOR THE VOTRAX PSS SPEECH                             SYNTHESIZER.                  COPYRIGHTED 1987 BY ENABLE TALKING SOFTWARE.                      ALL RIGHTS RESERVED.                  THIS IS A USER SUPPORTED PROGRAM.SYNTHESIZER:      VOTRAX PSS/BPROGRAM NAME:     ENABLE_B.EXE  STARTUP MESSAGE:  THE ENABLE READER PROFESSIONAL SPEECH SYSTEM .                  VERSION 4.0 FOR THE VOTRAX PSS/B SPEECH                           SYNTHESIZER.                  COPYRIGHTED 1987 BY ENABLE TALKING SOFTWARE.                      ALL RIGHTS RESERVED.                  THIS IS A USER SUPPORTED PROGRAM. 2.2  THE OPERATIONS MANUAL  The Enable Reader 4.0 Professional Speech Systems OperationsManual is provided  on three different media:   1.  PRINTED on 8-1/2 x 11 paper.    2.  RECORDED on SIX 90 minute audio cassette tapes.       (AVAILABLE SOON)    3.  RECORDED ON A FLOPPY DISKETTE.  The disk is called the MANUAL     Disk, and the Manual is broken up into eleven text files on      the disk.  There is also a file named  README.DOC  which      contains updates to the Manual.          README.DOC       Getting Started; Updates to Manual.       INTRO.DOC        Acknowledgements                       Introduction,                        Table of Contents                       Chapter 1.       STARTUP.DOC      Chapter 2.       USING.DOC        Chapter 3. ENABLE READER 4.0         OPERATIONS  MANUAL        CHAPTER 2-4            HELPKEYS.DOC     Chapter 4      CURSKEYS.DOC     Chapter 4        READKEYS.DOC     Chapter 4        SYSTKEYS.DOC     Chapter 4        AIDKEYS.DOC      Chapter 5.       INTKEYS.DOC      Chapter 6.       WORDSTAR.DOC     Chapter 7. 	 WORDPERF.DOC     Chapter 8.  	 SUPERKEY.DOC     Chapter 9.       APPENDIX.DOC     Appendix Tables, Trademarks.  Thus the Operations Manual can be read from the printed  material, or listened to by playing the cassettes in a tape  recorder.  Once you are familiar with Enable Reader, you can use it to listen to the Manual.  You can use the DOS  TYPE  command  to display the files on the screen and speak them.  Or, if you  have a word processor that will handle standard Ascii text  files, you can just load the MANUAL Disk files into your word  processing program on your computer, and listen to them through  your speech synthesizer.    When written text is spoken through a speech synthesizer, it  will not always sound exactly as the text implies.  For  instance, punctuation marks are used to denote pauses, end of  sentences, et cetera, but a voice synthesizer will actually  speak some of the punctuation marks.  Sections of the Manual on  the disk may be deliberately oriented towards listening through  a synthesizer.  Therefore, a lot of the punctuation (mostly  commas) may have been taken out of some of the text files.  This makes the files a little harder to read, but a lot easier to  listen to.  The printed Manual is the same text as the files on the MANUALDisk, so it too may be missing a lot of punctuation in somesections.  If you listen to the text files and there are too manycommas spoken, you can use the  M  PUNCTUATION FILTER command, orthe  N  PUNCTUATION LEVEL command to screen out the commas.  (Youwill learn about these punctuation commands later.)   The recorded tapes of the Manual match the printed manual.  (In  fact, the recordings were made by reading the manual out loud.)  Thus when learning to use Enable Reader, a blind person can  listen to the tapes, and a sighted person can follow along with  the printed Manual.     2.3  CONTENTS OF ENABLE READER DISKS  There are FOUR floppy disks in the Enable Reader package.  Thenames of these disks, are 1) PROGRAM, 2) MANUAL, 3) VOTRAXWORDPROCESSING, and 4) DECTALK WORDPROCESSING.  A description oftheir contents, follows:    ENABLE READER 4.0         OPERATIONS  MANUAL        CHAPTER 2-5      PROGRAM Disk - The program diskette is divided into seven sections called subdirectories.  The main directory is called the root directory andlooks as follows when DIR <ENTER> is typed.Volume in drive A is 1 PROGRAMDirectory of A:/README                   xxxxx  12-22-86    1:00pARTIC          <DIR>            12-22-86    1:00pCALLTEXT       <DIR>            12-22-86    1:00pDECTALK        <DIR>            12-22-86    1:00pECHO           <DIR>            12-22-86    1:00pVOTALKER       <DIR>            12-22-86    1:00pVOTRAX

<DIR>            12-22-86    1:00pVOTRAX_B       <DIR>            12-22-86    1:00p          8 File(s)           xxxxx bytes freeThe seven sub-directories contain the following synthesizerprograms and tables:SUB-DIRECTORY:      SYNTHESIZER:ARTIC               ARTIC BOARDCALLTEXT            CALLTEXT 5050DECTALK             DECTALK DTC01ECHO                ECHO GPVOTALKER            VOTALKER IBVOTRAX              VOTRAX PSSVOTRAX_B            VOTRAX PSS/B The SEVEN sub-directories and their contents are accessed bychanging directories with the Change Directory Command.  For example, to access the contents of the sub-directories type at theDOS command prompt   cd/   and then the name of the subdirectorydesired followed by a carriage return.  TO ACCESS THE VOTRAX PSSPROGRAM AND THE TABLES FOR GENERAL OPERATION TYPE:A>cd/votrax  <ENTER>THIS IS READ  "C D BACKSLASH VOTRAX ENTER" Once the command is typed the sub-directory is available forreading with the DIR command.  This will allow you to look at thecontents in the directory of the diskette.  Later in this chapteryou will be instructed on how to create your own diskettes. Below are the contents of each sub-directory:1.  One of the files is the Enable Reader Professional Speech         program for your particular speech synthesizer.  This program     is generic, in the sense that it can run with any supported       synthesizer by just loading in the correct table for that         synthesizer.       ENABLE READER 4.0         OPERATIONS  MANUAL        CHAPTER 2-6          In chapter 4 you will be taught how to load        and save tables.  We send a ready to run program as a             convenience so you can get up and running without having to       know how the Enable Reader program. This program is ONE of           the following files, depending on which sub-directory you have    entered:         ARTIC.EXE        - ARTIC speech synthesizer.        ENABLE_C.EXE     - CALLTEXT 5050 speech synthesizer.        ENABLE_D.EXE     - DECTALK DT C 0 1 speech synthesizer.        ENABLE_E.EXE     - ECHO GP speech synthesizer.        VOTALKER.EXE     - VOTALKER IB speech synthesizer.        ENABLE_V.EXE     - VOTRAX PSS speech synthesizer.           ENABLE_B.EXE     - Votrax PSS/B speech synthesizer.    2.  Another file is the Enable Default Table for Enable Reader,       called   ENABLE.TBL . This file is automatically read by the      Enable Reader program when Enable Reader is first executed.       It contains the command settings and parameters used by           Enable Reader.  While you are using Enable Reader, you can        change settings in it and then save the changes.   If the         changes are saved as the ENABLE.TBL file,  the next time the      program is run it will start up with the new changes as           default settings.      3.  There are TABLES for the Enable Reader Professional Speech    System for different PORTS at different SPEEDS.  Any of these    tables can be renamed to ENABLE.TBL in order to have the     Enable Reader Professional Speech System start for any    supported speech synthesizer or they can be loaded directly    while Enable Reader is running in order to change the     Port/Setup parameters of a new synthesizer on the fly.  Many    times the system you need to connect to (ie. a mini or    mainframe computer or an IBM LAN ) is preset to a certain     configuration.  The following tables can allow the Enable     Reader Program to be set at different speeds and for different    communications ports:              	   1200 BAUD/ COMMUNICATION PORT 1            1200COM1.CTX     - CALLTEXT 5050 speech synthesizer	              1200COM1.DEC     - DECtalk DT C 0 1 speech synthesizer.           1200COM1.ECO     - Echo GP speech synthesizer.           1200COM1.VTX     - Votrax PSS speech synthesizer.           1200COM1.VXB     - Votrax PSS/B speech synthesizer.    ENABLE READER 4.0         OPERATIONS  MANUAL        CHAPTER 2-7                        1200 BAUD/ COMMUNICATION PORT 2            1200COM2.CTX     - CALLTEXT 5050 speech synthesizer	              1200COM2.DEC     - DECtalk DT C 0 1 speech synthesizer.           1200COM2.ECO     - Echo GP speech synthesizer.           1200COM2.VXT     - Votrax PSS speech synthesizer.           1200COM2.VXB     - Votrax PSS/B speech synthesizer.             	   9600 BAUD/ COMMUNICATION PORT 1            9600COM1.CTX     - CALLTEXT 5050 speech synthesizer	              9600COM1.DEC     - DECtalk DT C 0 1 speech synthesizer.           9600COM1.ECO     - Echo GP speech synthesizer.           9600COM1.VTX     - Votrax PSS speech synthesizer.           9600COM1.VXB     - Votrax PSS/B speech synthesizer.             	   9600 BAUD/ COMMUNICATION PORT 2            9600COM2.CTX     - CALLTEXT 5050 speech synthesizer	              9600COM2.DEC     - DECtalk DT C 0 1 speech synthesizer.           9600COM2.ECO     - Echo GP speech synthesizer.           9600COM2.VTX     - Votrax PSS speech synthesizer.           9600COM2.VXB     - Votrax PSS/B speech synthesizer.              ARTIC AND VOTALKER/ COMMUNICATIONS PORT 1 AND 2        ENABLE.TBL       - COM PORT 1         PORT2.TBL        - COM PORT 2 MANUAL DISK -  The contents of the Enable Reader MANUAL Disk are as follows:   1.  An Update File for the Operations Manual, called       README.DOC.  This file contains changes and updates to the      Operations Manual.    ENABLE READER 4.0         OPERATIONS  MANUAL        CHAPTER 2-8      2.  The Enable Reader Professional Speech Systems Operations          Manual.  The manual is divided into fourteen files, whose         names all end with ".DOC" .  The order of the filenames in        the directory of the MANUAL Disk is the same order in which       the text occurs in the Manual.    3.  A sample Ascii file called  EXAMPLE .  You can display it on      the screen, and try out the Enable Reader screen review      commands on it.   VOTRAX AND DECTALK DISKS:The VOTRAX and DECTALK disks have two sections.  The rootdirectory and the sub-directory called WORDSTAR. Both diskscontain the same structure.  They have programs, tables, and macrofiles for SUPERKEY for the respective synthesiser the disk isnamed for. The ROOT directory contains the AUTOMATION PACKAGE FORWORDPERFECT.  The WORDSTAR sub-directory contains the AUTOMATIONPACKAGE FOR WORDSTAR.THE WORDSTAR SUB-DIRECTORY CONTAINS THE FOLLOWING:1.  Integration Tables for Wordstar.  We have included these      18 tables as a convenience for those of you who will      be using Wordstar.  Thus you do not have to create your own      tables to use with the Integration feature of the Enable      Reader program.  These tables are all of the files whose      names end with ".TBL" , except ENABLE.TBL .  Each of these      tables is to be used with a different Wordstar screen      display.  Chapter 7 in the Manual contains a list of these      Tables and where each Table is used. They will be                 automatically loaded for you. 2.  Superkey macro files for Wordstar.  There are 9 corresponding    and macro files for the major screens in Wordstar.                The Superkey macro files are automatically loaded when they       are needed.  A more detailed discussiion can be found in          Chapters 7, 8, and 9 on their contents, how they are created,     and how they run.  3.  A sample Wordstar text file called  TESTFILE .  You can use      this file with Wordstar to try out the "Matchup" command of      the Integration feature.  The file itself contains the      instructions for doing so.    4.  The Startup File, called  RUN.BAT .  It tells the computer      to load and activate Superkey, the Enable Reader program, then    prompts you whether to enter the date and time or run             Wordstar.  5.  Another file named  INIT.BAT configures the Superkey Program      before running the Automation Package.  It only needs to be       run once when Superkey is first copied into the Word              Proccessing sub-directory on your machine.  ENABLE READER 4.0         OPERATIONS  MANUAL        CHAPTER 2-9      THE ROOT DIRECTORY CONTAINS:1.  Integration Tables for WORD PERFECT.  There are 5     tables created as a convenience for those of you who will      be using Word Perfect.  Thus you do not have to create your       own tables to use with the Integration feature of the Enable      Reader program.  These tables are all of the files whose      names end with ".TBL" , except ENABLE.TBL .  Each of these      tables is to be used with different Word Perfect screen      displays.  Chapter 8 in the Manual contains a list of these      Tables and where each Table is used.   2.  Superkey macro files for Word Perfect.  There are 43    corresponding tables and macro files for the major screens in     Wordstar. The Superkey macro files are automatically loaded       when they  are needed.  A more detailed discussiion can be        found in Chapters 7, 8, and 9 on their contents, how they are     created, and how they run.3.  A sample Word Perfect text file called  TESTFILE .  You can       use this file with Word Perfect to try out the "Matchup"          command of the Integration feature.  The file itself contains     the instructions for doing so.   4.  The Startup File, called  RUN.BAT .  It tells the computer      to load and activate Superkey, the Enable Reader program, then    prompts you whether to enter the date and time or run             Word Perfect.  5.  Another file named  INIT.BAT configures the Superkey Program      before running the Automation Package.  It only needs to be       run once when

Superkey is first copied into the Word              Proccessing sub-directory on your machine.    2.4  ATTACHING THE VOICE SYNTHESIZER  Up to this point, you have been just reading or listening to the Manual.  This rest of this chapter describes the setup  procedures that you need to do before running Enable Reader.   You should actually follow the instructions given.     All seven versions of Enable Reader are pre-set to use Serial PortOne, and handshake using the XON/XOFF protocol.  The CALLTEXT,ECHO GP, VOTRAX PSS, and the VOTRAX PSS/B versions run at 9600Baud; the DECtalk Version runs at 1200  Baud.    Your particular speech synthesizer is to be connected directly  to your computer with a cable, and no special interfaces are  required.  The cable is connected between Serial Port One on the computer, and the Serial port on the synthesizer.  A cable can  be provided for your synthesizer if you do not have one.   ENABLE READER 4.0         OPERATIONS  MANUAL        CHAPTER 2-10     WARNING - You can NOT use just any cable that happens to fit  into the serial port connectors on your computer and your  synthesizer.  The cable has to be made specifically for  connecting your particular computer and synthesizer together.   Otherwise you may damage your synthesizer.  Also be sure to plug the correct end of the cable into each device -- the two ends of the cable might not be wired identically.    The CALLTEXT and ECHO GP has only one port, and it is serial. However, the Votrax and DECtalk each have two ports on the back. The serial port on the Votrax is the left side port as you facethe synthesizer from the front, and it is labeled  SERIAL PORT . It is located in the middle of the machine.  The DECtalk has two  Serial ports, but you want to use the one labeled  HOST PORT ,  which is the right side port as you face the synthesizer from  the front.    The Votrax synthesizer also has some DIP switches that need to  be set.  Refer to the Votrax Operator's Manual for the settings. You should set the switches for:  9600 baud; XON/XOFF protocol;  7 bit word length; power-up message ON; SERIAL port; RUN mode.     2.5  SETTING UP YOUR DISKS  NOTE - WE STRONGLY RECOMMEND THAT THE FOLLOWING STEPS BE DONE IN         THE PRESENCE OF A SIGHTED PERSON.  UNTIL YOU ARE USING         ENABLE READER, YOU WILL NEED SOMEONE TO TELL YOU WHAT THE         COMPUTER PRINTING TO THE SCREEN.    The Enable Reader package comes with FOUR disks:  a PROGRAM  Disk, a MANUAL Disk, and two AUTOMATION PACKAGE DISKS for theVOTRAX PSS and the Dectalk speech synthesizer. The first thing to do before using Enable Reader is to protectyourself in case you  accidentally damage your only copies of thesoftware.  You need  to make copies of each of the disks in theEnable Reader  package.  Then you can use the copies, and storethe originals in a safe place.  Next you need to put the EnableReader program  on a System Disk.  The following will guide youthrough the steps for accomplishing these tasks.   WE DO NOT RECCOMEND RUNNING THE AUTOMATION PACKAGES WITHOUT A HARDDISK.  THE TABLES AND MACROS MUST BE AVAILABLE AT ALL TIMES. THISREQUIRES THE STORAGE CAPACITY OF A HARD DISK.If you have a computer with two floppy disk drives, proceed with the next section entitled "For Computers With Two Floppy Disk  Drives".  If you are using a Hard Disk, skip ahead to the  section "For Computers With A Hard Disk".   ENABLE READER 4.0         OPERATIONS  MANUAL        CHAPTER 2-11      FOR COMPUTERS WITH TWO FLOPPY DISK DRIVES:  1.   Turn on your computer and boot it with an MSDOS or PCDOS       System Disk in Drive A .  The DOS has to be Version 2.00 or       higher.  When the boot process is finished, there will be       an  A>  showing on the screen signifying that the default       drive is Drive A .    2.   If you want to listen to the computer's responses during       the following operations, you can use Enable Reader to do       so.  If your synthesizer is not yet hooked up to your       computer, or you otherwise do not wish to run Enable Reader       during these operations, then skip this step and proceed       with  Step 3  to make copies of the Enable Reader Disks.         To run Enable Reader:        A)  First turn on your synthesizer.         B)  Put the Enable Reader PROGRAM Disk into Drive A .            CHANGE SUB-DIRECTORIES TO YOUR SYNTHESIZER TYPE.          Next, type in the name of your version of the Enable              Reader program, and then hit the  ENTER  key.  (To find           which name to type in, refer back to  SECTION 2.1                 VERSIONS OF ENABLE READER  PROFESSIONAL, or  SECTION 2.3          CONTENTS OF ENABLE READER DISKS, in this manual.)  For            example, if your synthesizer is the Votrax PSS, you will          type:               A>ENABLE_V.EXE   <ENTER>         C)  You should hear the Enable Reader Startup Message.                Enable Reader is now running.  It is set to speak any           characters that are sent to the screen to be displayed.   3.   Make copies of the Enable Reader Disks.  To do this:        1)  Put a DOS System Disk (with FORMAT.COM and           DISKCOPY.COM on it) into Drive A .         2)  Type:                A>DISKCOPY A: B:   <ENTER>         3)  When the computer says to put disks into the           drives, put the Enable Reader PROGRAM Disk into Drive A           and put a blank disk into Drive B .  Then hit any key to          begin duplicating the Enable Reader Disk.    ENABLE READER 4.0         OPERATIONS  MANUAL        CHAPTER 2-12          4)  When the copying is done, the computer will ask you               if you wish to copy another disk.  Take the disks out of          the drives.  The disk from Drive B is your copy of the           Enable Reader PROGRAM Disk.  Put a label and a write-          protect tab on it.  Type    Y    for yes to tell the           computer that you want to copy another disk.         5)  Now repeat the copy procedure THREE more times,           following the computer's instructions.  Copy the Enable           Reader MANUAL , and the TWO AUTOMATION DISKS.    After the last copy, type    N    for NO to tell the computer that you do not want to make any more copies.    You should put the original Enable Reader Disks in a safe place, and use the copies from now on.  Next, you will put the Enable  Reader program on a System Disk.    4.   Format a blank disk, putting the Operating System on it in       the process.  To do this:        1)  Put the DOS Disk back in Drive A and type:                       A>FORMAT B:/S   <ENTER>         2)  When the computer tells you, put a blank disk into           Drive B and hit any key to go ahead with the formatting.       3)  When the formatting is done, the computer will ask           you if you wish to format any more disks.  Just hit the           letter N key to tell it NO .  The  A>  prompt will                return.  You now have a formatted disk with the                   Operating System on it.    5.   We will now copy the Enable Reader Program, and the     ENABLE .TBL default file. for the synthesizer you wish to         work with. To do this:        1)  Put the PROGRAM Disk into Drive A , and the disk           you just formatted into Drive B .         2)  Change to the Synthesizer Sub-directory using the Change          Directory Command  CD\ .  Then type:                                  A>COPY *.* B:/V   <ENTER>         The PROGRAM, TABLES INCLUDING THE DEFAULT TABLE, AND              AUTOMATIC STARTUP FILES on the PROGRAM Disk will now be           copied onto your formatted disk.          THIS DISK IS NOW AN ENABLE READER SYSTEM DISK AND                 CAN BE USED WITH THOUSANDS OF IBM PC PROGRAMS.  ENABLE READER 4.0         OPERATIONS  MANUAL        CHAPTER 2-13     When you use this Enable Reader System Disk to boot your  computer, Enable Reader will automatically start up for you.    Now that your disks are set up, you are ready to use Enable  Reader.  Chapter 3 gives a general summary of how to work with  the Enable Reader features.  Chapters 4 and 6 give detailed  descriptions of each of the commands.  Chapters 7, 8, and 9 givedetailed instructions on how to run the Automation Package withEnable Reader.   The next section is for setting up Enable Reader with a hard  disk.  If you have a computer with two floppy disk drives, and  if you have already set up your disks by following the  instructions in the previous section, then skip ahead to   SECTION 2.6  AUTO EXECUTION  in this manual.  If you have a  computer with a hard disk, proceed with the next section  entitled "For Computers With A Hard Disk".     FOR COMPUTERS WITH A HARD DISK:  1.   Turn on your computer and go through the boot process.  The       DOS has to be Version 2.00 or higher.    2.   If you want to listen to the computer's responses during       the following operations, you can use Enable Reader to do       so.  If your synthesizer is not yet hooked up to your       computer, or you otherwise do not wish to run Enable Reader       during these operations, then skip this step and proceed       with  Step 3  to make copies of the Enable Reader Disks.         To run Enable Reader:           A)  First turn on your synthesizer.             B)  Type:                     A:    <ENTER>                          This will set the default drive to Drive A .             C)  Put the Enable Reader PROGRAM Disk into Drive A .                CHANGE DIRECTORIES TO YOUR SYNTHESIZER TYPE.             Next, type in the name of your version of the                     Enable Reader program,

and then hit the  ENTER  key.             (To find out which name to type in, refer back to                 SECTION 2.1  VERSIONS OF ENABLE READER, or  SECTION               2.3 CONTENTS OF ENABLE READER   DISKS, in this                    manual.)              ENABLE READER 4.0         OPERATIONS  MANUAL        CHAPTER 2-14                  For example, if your synthesizer is the Votrax PSS,               you will type:                     ENABLE_V.EXE   <ENTER>             4)  You should hear the Enable Reader Startup Message.                Enable Reader is now running.  It is set to speak                 any characters that are sent to the screen to be                  displayed.    3.   Make copies of the Enable Reader Disks.  To do this:            1)  If the file  DISKCOPY.COM  is on your Hard Disk,               type:                      C:     <ENTER>             This will set the default drive to Drive C  .             2)  If the file  DISKCOPY.COM  is NOT on your Hard               Disk, put a DOS System Disk (with  DISKCOPY.COM  on               it) into Drive A , and type:                      A:     <ENTER>             3)  Type:                      DISKCOPY A: B:   <ENTER>                     4)  When the computer says to put disks into the               drives, put the Enable Reader PROGRAM Disk into                   Drive A and put a blank disk into Drive B .  If you               have just one floppy drive, then just put the Enable              Reader PROGRAM Disk  into Drive A ; the computer                  will prompt you to switch disks.  Then hit any key                to begin duplicating the Enable Reader Disk.             5)  When the copying is done, the computer will ask you               if you wish to copy another disk.  Take the disks                 out of the drives.  The disk from Drive B is your                 copy of the Enable Reader PROGRAM Disk.  Put a label              and a write protect tab on it.  Type    Y    for yes              to tell the computer that you want to copy another                disk.             6)  Now repeat the copy procedure THREE more times,               following the computer's instructions.  Copy the                  Enable Reader MANUAL, and two AUTOMATION DISKS.   After the last copy, type    N    to tell the computer that      you do not want to make any more copies.    You should put the original Enable Reader Disks in a safe place, and use the copies from now on.  Next you will put the Enable  Reader program on your Hard Disk.    ENABLE READER 4.0         OPERATIONS  MANUAL        CHAPTER 2-15     4.   Copy the necessary files from the Enable Reader PROGRAM       Disk onto your Hard Disk.  To do this:            1)  Put the PROGRAM Disk into Drive A .             2)  Change from C: to A: and change direcrtories to the               synthesizer you are working with. then change back                to C:  For example, after the C> prompt type: 		             a:             <ENTER>             CD\VOTRAX      <ENTER>             C:             <ENTER>                      3)  From the C: prompt type:                 COPY A:*.EXE C:/V   <ENTER>                 COPY A:ENABLE.TBL C:/V  <ENTER>                  COPY A:AUTOEXEC.BAT C:/V <ENTER>         The PROGRAM, DEFAULT, AND AUTOMATIC STARTUP FILES on              the PROGRAM Disk will now be copied onto your hard disk.           THE HARD DISK IS NOW SET UP TO AUTOMATICALLY RUN THE              ENABLE READER PROGRAM AND CAN BE USED WITH THOUSANDS OF           IBM PC PROGRAMS.          IF YOU WOULD LIKE TO CREATE AN AUTOMATION SUB-DIRECTORY          FOR THE WORDSTAR OR WORD PERFECT PROGRAM PROCEED WITH STEPS          3 THROUGH 6  BELOW.  IF YOU ARE NOT GOING TO MAKE AN          AUTOMATION SUB-DIRECTORY THEN SKIP AHEAD TO SECTION 2.6 AUTO         EXECTION FILES.       3)  It is reccomended that a special sub-directory is                 created to run the Wordstar and Word Perfect automation           packages.  Below is an example of how to create and copy          files into a sub-directory.  First, we create a sub-              directory with the MAKE DIRECTORY command.  To create a           sub-directory for the Word Perfect Automation Package             you would type from the C> prompt:			       mkdir \wordperf <ENTER>         Or for the Wordstar Automation Package type:                      mkdir \wordstar <ENTER>                          4)  Now we need to change SUB-DIRECTORIES on BOTH Drive A:            and drive C:.  Then we will copy the Automation Package           over.ENABLE READER 4.0         OPERATIONS  MANUAL        CHAPTER 2-16              From the C> prompt, change to the Sub-directory you               created by typing:                                 cd\wordperf  <ENTER>         or for the Wordstar sub-directory type:                     cd\wordstar  <ENTER>         The sub-directory for Wordstar on the Automation Disk             in drive A: is accessed by  changing directories with             the CD/ or Change Directory command. For example, to              access the contents of the sub-directory type at the DOS          command  prompt:                       a:          <ENTER>         This will change to the Floppy diskette as the default            drive.  then type:                      cd\WORDSTAR  <ENTER>         Once the command is typed the sub-directory is available          for use.  Next we will copy all the files in this          sub-directory by typing:                      copy a:*.* c:/v  <ENTER>         This will transfer all the files needed to run the                Enable Reader program and Superkey with the Wordstar              or Word Perfect program.        4)  The final copy of files is to copy the Superkey Program           and the word processing program onto the newly created            system disk.  First, place the Superkey disk into         drive A:  and type:                    A>copy a:key.exe c:/v <ENTER>         Then type:                    A>copy a:key.hlp c:/v <ENTER>            FOR WORDSTAR              Next, place the Wordstar Master diskette into Drive A:            and type:                    A>copy a:ws.com c:/v  <ENTER>         Then type:                               A>copy a:*.ovl c:/v   <ENTER>        ENABLE READER 4.0         OPERATIONS  MANUAL        CHAPTER 2-17              FOR WORD PERFECT         You must copy all disks onto the hard disk in the same            sub-directory.  This is done by placing each disk in turn          into Drive A: and typing:                     A>copy *.* c:/v      <ENTER>     5)  When the  A>  prompt returns, the copying is done.            The sub-directory in Drive C: is your Enable Reader               Automation System .        6)  Take the disk out of Drive A:    When you want to run the Auotmation Package, change to thissub-directory that contains the Enable Reader automation packageand type:                   RUN    <ENTER KEY> The Enable Reader automation package will automatically start upfor you.    Now that your disks are set up, you are ready to use Enable  Reader.  Chapter 3 gives a general summary of how to work with  the Enable Reader features.  Chapters 4 and 6 give detailed  descriptions of each of the commands.  Chapters 7, 8, and 9 givedetailed instructions on how to run the Automation Package withEnable Reader.   If you would like to have the Enable Reader program  automatically start up for you whenever you boot your  computer, you will have to add a command for the name of  the Enable Reader program to your  AUTOEXEC.BAT  file.   (See SECTION 2.6  AUTOEXECUTION.)  This is not necessary at      this time, so if you are not sure how to do this, you can  skip it for now.    Now that your Hard Disk is set up, you are ready to use Enable  Reader.  Chapter 3 gives a general summary of how to work with  the Enable Reader features.  Chapters 4 and 6 give detailed  descriptions of each of the commands.     2.6  AUTO EXECUTION  The MSDOS Operating System looks for a specific Startup File  called  AUTOEXEC.BAT  when it first starts up.  If this file  exists on the disk, then the computer executes the instructions  in the file before turning control of the computer over to you.   The following discussion is only a short example of how to write a simple startup file.  For a more complete explanation, refer  to the MSDOS manual for the sections on COPY and on BATCH FILES.  ENABLE READER 4.0         OPERATIONS  MANUAL        CHAPTER 2-18     The Enable Reader PROGRAM Disk contains an  AUTOEXEC.BAT  file.   This startup file contains only three instructions.  These  instructions will load and run the Enable Reader program, ask  you to enter the date, and then ask you to enter the time.   These instructions are the same as could be typed in from the  keyboard when running the Operating System.  Using an  AUTOEXEC.BAT  file means you do not have to type the name of the  Enable Reader program every time you startup the machine.  The   AUTOEXEC.BAT  file can be created from the Operating System  without having to use a word processor.  The following procedure was used to create the file we supplied you.   1.   Type     COPY CON: AUTOEXEC.BAT           <ENTER>2.   Type     program name  (eg. ENABLE_V.EXE) <ENTER>3.   Type     DATE                             <ENTER>4.   Type     TIME                             <ENTER> 5.   Hit the  F6  key, then hit the  ENTER  key.  (NOTE:  The F6       key enters a  CONTROL Z  , so if you want, you can hold       down the  CONTROL  key, tap

the  Z  key, release the       CONTROL key, and tap the  ENTER  key for this step.)   The computer then creates the  AUTOEXEC.BAT  file and saves it  on the disk.  From now on, every time you start or reboot the  computer with this disk, the instructions in this file are  executed first.    If you have followed the instructions for creating an Enable  Reader System Disk on a floppy disk, then the  AUTOEXEC.BAT   file on that disk will automatically start Enable Reader  whenever you boot your computer with that disk.    If you are booting from a hard disk, chances are that there  already is an  AUTOEXEC.BAT  file on the hard disk.  If you want  Enable Reader to start automatically when you boot the hard  disk, you need to add the name of your Enable Reader program to  the existing  AUTOEXEC.BAT  file.  You can do this by looking at the current contents of  AUTOEXEC.BAT , and retyping all the  instructions with the name of the Enable program inserted.  Or  you can use a word processing program to add the Enable program  to  AUTOEXEC.BAT .  The Enable program gets inserted into the  list of commands AFTER any hard disk initialization commands,  and BEFORE any other commands.  If your hard disk does not  already have an  AUTOEXEC.BAT  file on it, you can just copy the AUTOEXEC.BAT  file from the Enable Reader PROGRAM Disk onto your hard disk.  Putting the name of your Enable Reader program into  the  AUTOEXEC.BAT  file on your hard disk is not really  necessary at this time, so if you are not sure how to do it, you can skip it for now.     END OF   CHAPTER 2  GETTING STARTED ########################################################################