CHAPTER 7   THE FLOATING-POINT PROCESSOR

In this chapter, we'll refer to the various Central Processing

Units (CPUs) as the "86".  Thus "86" refers to either the 8088,

8086, 80186, 80286, etc.  We'll refer to the various coprocessors

as the "87".  Thus "87" refers to either the 8087, the 287, the

387, or the special IIT-2C87 processor.

The 8087 and 287 Coprocessors

All IBM-PC's, and most clones, contain a socket for a floating

point coprocessor.  If you shell out between $80 and $300, and

plug the appropriate chip into that socket, then a host of

floating point instructions is added to the assembly language

instruction set.

The original IBM-PC, and the XT, accept the original floating

point chip, the 8087.  The AT accepts a later update, the 287.

From a programming standpoint, the two chips are nearly

identical: the 287 adds the instructions FSETPM and FSTSW AX, and

ignores the instructions FENI and FDISI.  There is, however, a

rather nasty design flaw in the 8087, that was corrected in the

287.

To understand the flaw, you must understand how the 86 and 87

work as coprocessors. Whenever the 86 sees a floating point

instruction, it communicates the instruction, and any associated

memory operands, to the 87.  Then the 86 goes on to its next

instruction, operating in parallel with the 87.  That's OK, so

long as the following instructions don't do one of the following:

1. Execute another floating point instruction; or

2. Try to read the results of the still-executing floating

point instruction.

If they do, then you must provide an instruction called WAIT (or

synonymously FWAIT), which halts the 86 until the 87 is finished.

For almost all floating point instructions, it should not be

necessary to provide an explicit FWAIT; the 86 ought to know that

it should wait.  For the 8087, it IS necessary to give an

explicit FWAIT before each floating point instruction: that is

the flaw.

Because of the flaw, all assemblers supporting the 8087 will

silently insert an FWAIT code (hex 9B) before all 87

instructions, except those few (the FN instructions other than

FNOP) not requiring the FWAIT.  A86 provides the switch +F (the F

must be capitalized), to signal that the 287 is the target

processor.  A86 also provides the directive ".287", compatible

with Microsoft's assembler, that you can insert into your

programs to accomplish the same thing as +F. However, the actions

taken by A86 and Microsoft when seeing .287 are completely

disjoint!  To wit:

7-2

* A86 ceases outputting FWAIT directives that are unnecessary for

the 287.  For reasons beyond my comprehension, Microsoft

continues to put them out.  Can someone enlighten me as to why

Microsoft is putting out those codes?

* A86 ignores the instructions FENI, FDISI, FNENI, and FNDISI

after it sees a .287 directive.  Microsoft continues to

assemble these instructions.

* Microsoft recognizes the new 287 instructions, if and only if

it sees the .287 directive.  A86 recognizes them even if .287

is not given.  In general, I don't attempt to police your

instruction usage-- if you use an instruction available on a

limited number of processors, I trust that you are programming

for one of those processors.

In summary, if your program will be running only on machines with

a 287, you can give ".287" directive. Your programs will be

significantly shorter than if they were assembled by Microsoft.

If you want your programs to run on all machines containing a

floating point chip, you should refrain from specifying .287.

WARNING: The most common mistake 87 programmers make is to try to

read the results of an 87 operation in 86 memory, before the

results are ready.  At least on my AT, the system often crashes

when you do this!  If your program runs correctly when single

stepped, but crashes when set loose, then chances are you need an

extra explicit FWAIT somewhere.

Extra Coprocessor Support

A86 now supports two additional coprocessors available for

PC-compatibles: the 80387, available for 386-based machines, and

the IIT-2C87, a 287-plug-compatible chip that adds a couple of

unique instructions. The IIT-2C87 has two extra banks of on-chip

8-number stacks, that can be switched in with the FBANK

instruction, and a matrix multiply instrction that uses all three

banks as input.  (For details contact Specialty Software

Development Corp., 110 Wild Basin Road, Austin TX 78746.) Both

chips incorporate the correction to the 8087's FWAIT design flaw,

so you can assemble with the .287 directive.  The extra

instructions for these chips are marked by "387 only:" and "IIT

only:" in the chart at the end of this chapter.

Emulating the 8087 by Software

There is a software package provided with many compilers

(Borland's Turbo C and most Microsoft compilers, for example)

that emulates the 8087 instruction set.  The emulator is very

cleverly implemented so that the programmer need not know whether

a floating point chip will be available, or whether emulation

will be necessary.  This is done by having the linker replace all

floating point machine instructions with INT calls to certain

interrupts, dedicated to emulation.  The interrupt handlers

interpret the operands to the instructions, and emulate the 8087.

7-3

You can tell A86 that the emulator might be used, by providing a

+f switch in the invocation line, or in the A86 environment

variable (make sure the f is lower case).  Since your program

will be linked to the emulator, you must be producing an OBJ

file, not a COM file, for emulation support to take effect.

Whenever a floating point instruction is assembled, A86 will

generate an external reference at the opcode for the instruction.

Then, if the emulation package is linked with your program, the

opcodes will be replaced by the INT calls. If a special

non-emulation module is linked, the opcodes will be left alone,

and the floating point instructions will be executed directly.

The Floating Point Stack

The 87 has its own register set, of 8 floating point numbers

occupying 10 bytes each, plus 14 bytes of status and control

information.  Many of the 87's instructions cause the numbers to

act like a stack, much like a Hewlett-Packard calculator.  For

this reason, the numbers are called the floating point stack.

The standard name for the top element of the floating point stack

is either ST or ST(0); the others are named ST(1) through ST(7).

Thus, for example, the instruction to add stack element number 3

into the top stack element is usually coded FADD ST,ST(3).

I find this notation painfully verbose.  Especially bad are the

parentheses, which are hard to type, and which add visual clutter

to the program.  To alleviate this problem while retaining

language compatibility, I name my stack elements simply 0 through

7.  I recognize ST as a synonym for 0.  I allow expression

elements to be concatenated; concatenation is the same as

addition.  Thus, when A86 sees ST(3), it computes 0+3 = 3.  So

you can code the old way, FADD ST,ST(3), or you can code the

concise way, FADD 0,3 or simply FADD 3.

Floating Point Initializations

In general, you use the 87 by loading numbers from 86 memory to

the 87 stack (using FLD instructions), calculating on the 87

stack, and storing the results back to 86 memory (using FST and

FSTP instructions).  There are seven constant numbers built into

the 87 instruction set: zero, one, Pi, and four logarithmic

conversion constants.  These can be loaded using the FLD0, FLD1,

FLDPI, FLDL2T, FLDL2E, FLDLG2, and FLDLN2 instructions.  All

other constants must be declared in, then loaded from, 86 memory.

Integer constant words and doublewords can be loaded via FILD.

Non-integer constant doubleword, quadwords, and ten-byte numbers

can be loaded via FLD.

7-4

A86 allows you to declare constants loaded via FLD as floating

point numbers, using scientific notation if you like.  As an

exclusive feature, A86 allows you to use any of the 4 arithmetic

functions +, -, *, / in expressions involving floating point

numbers.  A86 will even do type conversion if one of the two

operands is given as an integer; though for clarity I recommend

that you always give floating point constants with their decimal

point.

Built-In Constant Names

A86 offers another exclusive feature: the built-in symbols

PI   ratio of circumference to diameter of a circle

L2T  log base 2 of 10

L2E  log base 2 of the calculus constant e = 2.71828...

LG2  log base 10 of 2

LN2  natural log (base e) of 2

You can use these symbols in expressions, to declare useful

constants.  For example, you can declare the degrees-to-radians

conversion constant:

DEG_TO_RAD  DT  PI/180.

Special Immediate FLD Form

Yet another exclusive A86 feature is the instruction form FLD

constant.  This form is intended primarily to facilitate "fooling

around" with the 87 when using D86; but it is also useful for

quick-and-dirty programs.  For example, the instruction FLD 12.3

generates the following sequence of code bytes (without

explicitly using the local labels given):

CS FLD T[M1]

JMP >M2

M1  DT 12.3

M2:

Obviously, this form is not terrifically efficient: you can

always save the JMP by placing the constant outside of the

instruction stream; and the CS override might not be needed.  But

the form is very, very convenient!

NOTE that the preceding 2 sections imply that you can get

careless and code, for example, FLD PI when you intended FLDPI.

Though the two are functionally equivalent, the first form takes

a whopping 17 bytes; and second, only 2 bytes.  Be careful!

7-5

Floating Point Operand Types

The list of floating point instructions contains a variety of

operand types.  Here is a brief explanation of those types:

0        stands for the top element of the floating point stack.

A synonym for 0 is ST or ST(0).

i        stands for element number i of the floating point stack.

i can range from 0 through 7.  A synonym for i is ST(i).

mem10r   is a 10-byte memory quantity (typically declared with a

DT directive) containing a full precision floating point

number. Intel recommends that you NOT store your numbers

in full precision; that you use the following double

precision format instead.  Full precision numbers are

intended for storage of intermediate results (on the

stack); they exist to insure maximum accuracy for

calculations on double precision numbers, which is the

official external format of 87 numbers.

mem8r    is an 8-byte memory quantity (typically declared with a

DQ directive) containing a double precision floating

point number.  This is the best format for floating

point numbers on the 87.  The 87 takes the same amount

of time on double precision calculations as it does on

single precision.  The only extra time is the memory

access of 4 more bytes; negligible in comparison to the

calculation time.

mem4r    is a 4-byte quantity (typically defined with a DD

directive) containing a single precision floating point

number.

mem10d   is a 10-byte quantity (also defined via DT) containing a

special Binary Coded Decimal format recognized by the

FBLD and FBSTP instructions.  This format is useful for

input and output of floating point numbers.

mem4i    is a 4-byte quantity representing a signed integer in

two's-complement notation.

mem2i    is a 2-byte quantity representing a signed integer in

two's-complement notation.

mem14    and mem94 are 14- and 94-byte buffers containing the 87

machine state.

7-6

Operand Choices in A86

In the "standard" assembly language, the choice of operands for

floating point instructions seems inconsistent to me.  For

example, to subtract stack i from 0, you must provide two

operands; to do the equivalent comparison, you must provide only

one operand.  A86 smooths out these inconsistencies by allowing

more choices for operands: FADD i is equivalent to FADD 0,i. FCOM

0,i is equivalent to FCOM i.  The same holds for the other main

arithmetic instructions.  FXCH 0,i and FXCH i,0 are allowed. So

if you wish to retain compatibility with other assemblers, you

should use their more restrictive instruction list, not the

following one.

The 87 Instruction Set

Following is the 87 instruction set.  The "w" in the opcode field

is the FWAIT opcode, hex 9B, which is suppressed if .287 is

selected.  Again, "0", "1", and "i" stand for the associated

floating point stack registers, not constant numbers!  Constant

numbers in the descriptions are given with decimal points: 0.0,

1.0, 2.0, 10.0.

Opcode    Instruction     Description

w  D9 F0     F2XM1           0 := (2.0 ** 0) - 1.0

w  DB F1     F4X4            IIT only: 4 by 4 matrix multiply

w  D9 E1     FABS            0 := |0|

w  DE C1     FADD            1 := 1 + 0, pop

w  D8 C0+i   FADD i          0 := i + 0

w  DC C0+i   FADD i,0        i := i + 0

w  D8 C0+i   FADD 0,i        0 := i + 0

w  D8 /0     FADD mem4r      0 := 0 + mem4r

w  DC /0     FADD mem8r      0 := 0 + mem8r

w  DE C0+i   FADDP i,0       i := i + 0, pop

w  DB E8     FBANK 0         IIT only: set bank pointer to default

w  DB EB     FBANK 1         IIT only: set bank pointer to bank 1

w  DB EA     FBANK 2         IIT only: set bank pointer to bank 2

w  DF /4     FBLD mem10d     push, 0 := mem10d

w  DF /6     FBSTP mem10d    mem10d := 0, pop

7-7

w  D9 E0     FCHS            0 := -0

9B  DB E2     FCLEX           clear exceptions

w  D8 D1     FCOM            compare 0 - 1

w  D8 D0+i   FCOM 0,i        compare 0 - i

w  D8 D0+i   FCOM i          compare 0 - i

w  D8 /2     FCOM mem4r      compare 0 - mem4r

w  DC /2     FCOM mem8r      compare 0 - mem8r

w  D8 D9     FCOMP           compare 0 - 1, pop

w  D8 D8+i   FCOMP 0,i       compare 0 - i, pop

w  D8 D8+i   FCOMP i         compare 0 - i, pop

w  D8 /3     FCOMP mem4r     compare 0 - mem4r, pop

w  DC /3     FCOMP mem8r     compare 0 - mem8r, pop

w  DE D9     FCOMPP          compare 0 - 1, pop both

w  D9 FF     FCOS            387 only: push, 1/0 := cosine(old 0)

w  D9 F6     FDECSTP         decrement stack pointer

w  DB E1     FDISI           disable interrupts (.287 ignore)

w  DE F9     FDIV            1 := 1 / 0, pop

w  D8 F0+i   FDIV i          0 := 0 / i

w  DC F8+i   FDIV i,0        i := i / 0

w  D8 F0+i   FDIV 0,i        0 := 0 / i

w  D8 /6     FDIV mem4r      0 := 0 / mem4r

w  DC /6     FDIV mem8r      0 := 0 / mem8r

w  DE F8+i   FDIVP i,0       i := i / 0, pop

w  DE F1     FDIVR           1 := 0 / 1, pop

w  D8 F8+i   FDIVR i         0 := i / 0

w  DC F0+i   FDIVR i,0       i := 0 / i

w  D8 F8+i   FDIVR 0,i       0 := i / 0

w  D8 /7     FDIVR mem4r     0 := mem4r / 0

w  DC /7     FDIVR mem8r     0 := mem8r / 0

w  DE F0+i   FDIVRP i,0      i := 0 / i, pop

w  DB E0     FENI            enable interrupts (.287 ignore)

w  DD C0+i   FFREE i         empty i

w  DE /0     FIADD mem2i     0 := 0 + mem4i

w  DA /0     FIADD mem4i     0 := 0 + mem2i

w  DE /2     FICOM mem2i     compare 0 - mem2i

w  DA /2     FICOM mem4i     compare 0 - mem4i

w  DE /3     FICOMP mem2i    compare 0 - mem2i, pop

w  DA /3     FICOMP mem4i    compare 0 - mem4i, pop

w  DE /6     FIDIV mem2i     0 := 0 / mem2i

w  DA /6     FIDIV mem4i     0 := 0 / mem4i

w  DE /7     FIDIVR mem2i    0 := mem2i / 0

w  DA /7     FIDIVR mem4i    0 := mem4i / 0

w  DF /0     FILD mem2i      push, 0 := mem2i

w  DB /0     FILD mem4i      push, 0 := mem4i

w  DF /5     FILD mem8i      push, 0 := mem8i

7-8

w  DE /1     FIMUL mem2i     0 := 0 * mem2i

w  DA /1     FIMUL mem4i     0 := 0 * mem4i

w  D9 F7     FINCSTP         increment stack pointer

9B  DB E3     FINIT           initialize 87

w  DF /2     FIST mem2i      mem2i := 0

w  DB /2     FIST mem4i      mem4i := 0

w  DF /3     FISTP mem2i     mem2i := 0, pop

w  DB /3     FISTP mem4i     mem4i := 0, pop

w  DF /7     FISTP mem8i     mem8i := 0, pop

w  DE /4     FISUB mem2i     0 := 0 - mem2i

w  DA /4     FISUB mem4i     0 := 0 - mem4i

w  DE /5     FISUBR mem2i    0 := mem2i - 0

w  DA /5     FISUBR mem4i    0 := mem4i - 0

w  D9 C0+i   FLD i           push, 0 := old i

w  DB /5     FLD mem10r      push, 0 := mem10r

w  D9 /0     FLD mem4r       push, 0 := mem4r

w  DD /0     FLD mem8r       push, 0 := mem8r

w  D9 E8     FLD1            push, 0 := 1.0

w  D9 /5     FLDCW mem2i     control word := mem2i

w  D9 /4     FLDENV mem14    environment := mem14

w  D9 EA     FLDL2E          push, 0 := log base 2.0 of e

w  D9 E9     FLDL2T          push, 0 := log base 2.0 of 10.0

w  D9 EC     FLDLG2          push, 0 := log base 10.0 of 2.0

w  D9 ED     FLDLN2          push, 0 := log base e of 2.0

w  D9 EB     FLDPI           push, 0 := Pi

w  D9 EE     FLDZ            push, 0 := +0.0

w  DE C9     FMUL            1 := 1 * 0, pop

w  D8 C8+i   FMUL i          0 := 0 * i

w  DC C8+i   FMUL i,0        i := i * 0

w  D8 C8+i   FMUL 0,i        0 := 0 * i

w  D8 /1     FMUL mem4r      0 := 0 * mem4r

w  DC /1     FMUL mem8r      0 := 0 * mem8r

w  DE C8+i   FMULP i,0       i := i * 0, pop

DB E2     FNCLEX          nowait clear exceptions

DB E1     FNDISI          disable interrupts (.287 ignore)

DB E0     FNENI           enable interrupts (.287 ignore)

DB E3     FNINIT          nowait initialize 87

w  D9 D0     FNOP            no operation

DD /6     FNSAVE mem94    mem94 := 87 state

D9 /7     FNSTCW mem2i    mem2i := control word

D9 /6     FNSTENV mem14   mem14 := environment

DF E0     FNSTSW AX       AX := status word

DD /7     FNSTSW mem2i    mem2i := status word

w  D9 F3     FPATAN          0 := arctan(1/0), pop

w  D9 F8     FPREM           0 := REPEAT(0 - 1)

w  D9 F5     FPREM1          387 only: 0 := REPEAT(0 - 1) IEEE compat.

w  D9 F2     FPTAN           push, 1/0 := tan(old 0)

7-9

w  D9 FC     FRNDINT         0 := round(0)

w  DD /4     FRSTOR mem94    87 state := mem94

w  DD /6     FSAVE mem94     mem94 := 87 state

w  D9 FD     FSCALE          0 := 0 * 2.0 ** 1

9B  DB E4     FSETPM          set protection mode

w  D9 FE     FSIN            387 only: push, 1/0 := sine(old 0)

w  D9 FB     FSINCOS         387 only: push, 1 := sine, 0 := cos(old 0)

w  D9 FA     FSQRT           0 := square root of 0

w  DD D0+i   FST i           i := 0

w  D9 /2     FST mem4r       mem4r := 0

w  DD /2     FST mem8r       mem8r := 0

w  D9 /7     FSTCW mem2i     mem2i := control word

w  D9 /6     FSTENV mem14    mem14 := environment

w  DD D8+i   FSTP i          i := 0, pop

w  DB /7     FSTP mem10r     mem10r := 0, pop

w  D9 /3     FSTP mem4r      mem4r := 0, pop

w  DD /3     FSTP mem8r      mem8r := 0, pop

w  DF E0     FSTSW AX        AX := status word

w  DD /7     FSTSW mem2i     mem2i := status word

w  DE E9     FSUB            1 := 1 - 0, pop

w  D8 E0+i   FSUB i          0 := 0 - i

w  DC E8+i   FSUB i,0        i := i - 0

w  D8 E0+i   FSUB 0,i        0 := 0 - i

w  D8 /4     FSUB mem4r      0 := 0 - mem4r

w  DC /4     FSUB mem8r      0 := 0 - mem8r

w  DE E8+i   FSUBP i,0       i := i - 0, pop

w  DE E1     FSUBR           1 := 0 - 1, pop

w  D8 E8+i   FSUBR i         0 := i - 0

w  DC E0+i   FSUBR i,0       i := 0 - i

w  D8 E8+i   FSUBR 0,i       0 := i - 0

w  D8 /5     FSUBR mem4r     0 := mem4r - 0

w  DC /5     FSUBR mem8r     0 := mem8r - 0

w  DE E0+i   FSUBRP i,0      i := 0 - i, pop

w  D9 E4     FTST            compare 0 - 0.0

w  DD E0+i   FUCOM i         387 only: unordered compare 0 - i

w  DD E1     FUCOM           387 only: unordered compare 0 - 1

w  DD E8+i   FUCOMP i        387 only: unordered compare 0 - i, pop

w  DD E9     FUCOMP          387 only: unordered compare 0 - 1, pop

w  DA E9     FUCOMPP         387 only: unordered compare 0 - 1, pop both

9B            FWAIT           wait for 87 ready

w  D9 E5     FXAM            C3 -- C0 := type of 0

w  D9 C9     FXCH            exchange 0 and 1

w  D9 C8+i   FXCH 0,i        exchange 0 and i

w  D9 C8+i   FXCH i          exchange 0 and i

w  D9 C8+i   FXCH i,0        exchange 0 and i

w  D9 F4     FXTRACT         push, 1 := expo, 0 := sig

w  D9 F1     FYL2X           0 := 1 * log base 2.0 of 0, pop

w  D9 F9     FYL2XP1         0 := 1 * log base 2.0 of (0+1.0), pop