BrailleNote QT GPS V3.3 Command Summary

Description and QT Command

Address nearby: A

Advanced Find POI (distance and category): CTRL with F

Altitude: Z

Announce Current Waypoint: K

Announce Destination position: D

Announce Next Turn: T

City/feature/state/zip: C

Complete location details: CTRL with C

Contextual GPS Help: HELP

Distance between GPS and Virtual positions: I

Exit GPS and prompt to save: ESC

Find nearby POI : F

GPS Status: G

GPS Options Menu: O

GPS Virtual mode toggle: V

Heading and distance to Nearest WP: N

Heading in compass and degrees: H

Identify, GPS or Virtual mode: CTRL with I

Interrupt Speech: READ with SPACE

Intersection Description: CTRL with X

Jump to Turning Point: CTRL with T

Keyboard lock: =

Lat/Lon announce: Y

Look-around Menu: CTRL with M

Main Menu: MENU

Move backward in WP list: CTRL with W or DOWN ARROW

Move Forward in WP list: W or UP ARROW

Nearest and Next Intersection: X

Pedometer accumulated distance: B

POI Description: CTRL with P

POI Name: P

Record POI: CTRL with R

Record Waypoint: CTRL with K

Reissue last command : Spacebar

Repeat last announcement: Repeat Key

Reset Pedometer: CTRL with B

Set Address: CTRL with S

Set as Destination: CTRL with D

Set Lat/Lon: CTRL with Y

Set nearest WP near GPS position: CTRL with N

Set BrailleNote to GPS date/time: CTRL with U

Speed: S

Spell names: Read with K

Time (in GMT): U

Virtual Mode Commands

Move forward an intersection: CTRL with UP ARROW

Move backward an intersection: CTRL with DOWN ARROW

Turn Left at an intersection: CTRL with LEFT ARROW

Turn Right at an intersection: CTRL with RIGHT ARROW

Set Virtual position to GPS position: CTRL with G

Set POI or waypoint as your Virtual: CTRL with V

List Navigation Commands

Back 1 list item: Up-Arrow

Back 10 list items: Left-Arrow or Pg Up

Bottom of list: Read with B or End

Forward 1 list item: Down-Arrow

Forward 10 list items: Right-Arrow or PG Dwn

Top of list: Read with T or Home

Edit Commands

Copy point: E then C

Erase point: E then E

Paste point: E then V

Rename point: E then R

Make WP current WP: E then N

Make WP a POI: E then P

Make WP a destination: E then CTRL D

Multiple Announcement Commands

Address: M then A

Altitude: M then Z

City/Feature/State/Zip: M then C

Current Waypoint: M then K

Destination: M then D

GPS status: M then G

Heading: M then H

Intersection: M then X

Intersection description: M then I

Lat/Lon: M then Y

POI: M then P

Speed: M then S

Turn: M then T

Distance between GPS and Virtual: M then V

Route Commands

Auto Increment WP On/Off: R then A

Close Current route: R then C

Create Pedestrian Route: R then P

Create Vehicle Route: R then V

Distance from Current GPS position to End: R then E

Distance from current GPS to beginning WP: R then B

Follow route sequentially, no skipping Waypoints: R then F

Open/Create a route: R then O

Record POI as WP: R then K

Reverse Route: R then R

Route Information: R then I

Save route: R then S

Toggle Route Turns or Waypoint detail mode: R then M

Total route distance: R then T