BrailleNote and VoiceNote GPS, Version 3.3 ReadMe, August 29, 2005

Copyright 2002 2005, Sendero Group, LLC

This Read Me contains late breaking information for BrailleNote GPS Version 3.3 for the Keysoft 6 BrailleNotes, mPower and PK as well as important information you should read before using the products.  Documents listing changes between versions are in the Documents folder. A listing of which states are on which Compact Discs, is also contained in this document. See the respective User manuals for BrailleNote or GPS questions and instructions. Listen to the Audio Tutorial on the Compact Flash card or CD for a play-by-play of basic GPS functions. Optional GPS training courseware is available for purchase in Braille or print as is a Braille GPS manual.

For Keysoft versions 3, 4 and 5, GPS 3.0 is the latest version. You must have Keysoft 6 or higher to run GPS 3.1 or higher.

If you purchased the GPS at the same time as your VoiceNote or BrailleNote, the software should already be installed. If you purchased the GPS separately, or if you are installing an upgrade, see your BrailleNote or VoiceNote user manual for instructions on installing software. Make sure you have Keysoft 6.1 or later installed. The latest version of Keysoft is highly recommended.

Make sure you press and release the Reset with the 3 keys held down, dots 123 on the Braille keyboard or the SDF keys held together on the QT keyboard.  You will hear a beep after releasing the Reset. Wait for the second chime before you release the 3 keys. You may receive a message saying that the existing files are newer than the version you are installing. Ignore this and press Enter to overwrite the former files with the new ones. The PK has different reset keys.

When installing the GPS version 3.3 upgrade from Version 1 or 2, Make sure to delete all Version 2 maps and commercial points of interest files from your card and toss the CDs so you don’t mix them up with the new files. If you have a User.poi file with personal points, do not delete that file. It will work with all versions. Version 3.3 POIs are new and will not work with previous GPS versions. Copy the new maps if necessary and POIs into their respective directories.

Important Settings

The GPS Receiver Option defaults to Serial GPS for Keysoft 6.1 BrailleNotes. This works for the Delorme Earthmate and most other brands. If you have any of the Magellan models, especially the 310 or Sportrak, select the Magellan receiver option. The PK and mPower GPS will default to a Bluetooth receiver.

Note, in version 3.3, the hot keys for Serial GPS and Magellan are incorrectly switched. Press G for Magellan and M for Serial in the GPS Options GPS Receiver Type Menu.

The commercial points of interest for your state are located on the Compact Flash card supplied with your GPS package in a POI folder and important documents are in the Documents folder. The map files are in the Maps folder. If maps or POIs do not appear when you load the program, go to the GPS Options menu to point to the relevant Map and POI folders. Make sure the Points of Interest file with a .BPI extension for the region you desire is in the folder you designate and not in a sub folder.

The BrailleNote GPS software automatically looks for a folder called POI on your Storage or Compact Flash cards. Make sure your region is in that POI folder.

In order to hear the street names of some types of roads in the UK map data, it is necessary to set the GPS Options Street Names item to “Long.” The default is “short” street names. This may also be necessary in other countries as well. Note the Chord C command gives both the short and long street names no matter what you have the Street Name set to.

Known Issues

As stated elsewhere in the Read Me and manual, it is recommended to load only the POI files you need. Once you load 110 or so POI files in the POI directory, you may receive an Exception error. If you have multiple map folders, you may wish to put the associated state POI files in the same folder as the maps for those states.

If you are using a serial GPS receiver with the mPower, it will be necessary to turn the receiver off before a reset on the mPower can be performed successfully.

The BrailleNote “go to page” command does not work from within the GPS Menu User Guide. It will work if you open the GPS manual from within the BrailleNote Book Reader application.

Corrected Missouri and Florida City index files are located on the Sendero download site for CDs shipped in 2004. If cities are not coming up in  and address match in these two states, you may need this file.

If you load Missouri maps, please install this file over the file of the same name when you copy maps to your Compact Flash card Maps folder. If the Missouri maps were already installed for you, this file should already be installed.

If you are experiencing slow response to commands, switch the Look-Around mode off while performing these other functions. It will remain in the status you choose until you reset, at which time it will revert to its default status. Look-Around scans for nearby POIs and intersections every 12 seconds and this takes some of the BrailleNote’s processing power. Besides turning Look-Around off, you can also make adjustments to look for POIs or Intersections only. See the manual for details. This is normally not an issue on the PK or Keysoft 6 or higher systems.

In the case of a shopping mall, there could be dozens of POIs, which have different names but the same lat/lon based on a single geocoded address for that shopping center. The Look-Around mode will trigger on the first of these multiple points at the same lat/lon but it won’t automatically announce the rest of the points at that lat/lon. If you wish to hear all the POI names, you must press F or Chord F to generate a list.

The indexing of some towns in the map data may require a round about way of finding an address match. It is best to assume a town is in the database and to use the suggestions in the manual to find a means of pulling that town up if you don’t find it using a normal Address Look Up. Medium to large cities may also be in the Points of Interest database and you can place your virtual position in the vicinity of a city by doing a POI search in this way. See the list of which states are on which CDs at the end of this ReadMe.

There is no distance limit placed on route calculations, however, there is a limit of 2500 route waypoints placed on the BrailleNote and 5000 on the PK. The User.poi file is the last to be searched. Therefore, if you happen to have a search return more than the limit of points, the User points will not be displayed, even if they are close by.

Should the system ever stop responding or if you need to abort a long route calculation, press reset. Avoid excessive searches like all points within 1000 miles. Reset on the PK is C3 and C4 pressed together.

If you ask for a POI search and immediately afterwards the automatic “look-around” function triggers a nearby POI, the current point announced by your POI search may be replaced by the Look-Around POI.

When searching for a city or street, which contains an accented character, it is necessary to enter that character exactly in order to find the match. You can enter the first part of a city name and find matches containing that string at the beginning of the city name.

When using Grade II, the “\” character will be spelled out as Backslash when a Map folder or POI folder name is displayed. Also, if you are entering a city name during an address match, be aware of single letter contractions. For example, if you only put the letter “c” it will be interpreted as the contracted word, “can.” Use a letter sign or additional letters to avoid this confusion.

Some of the duplicate commands on the Braille style keyboard do not work in Grade 1 or Computer Braille modes.

With Computer Braille the dot 4 Find command does not work nor do the dot 3 or dot 2 for recording a waypoint turningpoint.

With Grade 1 Braille the keyboard lock 123456 command, dot 5 Heading and Dot 2 Current Waypoint commands do not function.

If you turn on to a U-shaped street in Explore mode and press the Forward command, you will probably hear that you have looped back to the intersection where you started. If you continue along the street that spans the mouth of the U, it is likely that the next intersection ahead will be the other end of the U. There is no way to travel on the U street itself unless it has intersecting segments.

If you have the maximum of 12 maps loaded on the BrailleNote or 18 on the PK and mPower, and perform major POI searches or route calculations, the GPS program may become sluggish. Pressing reset may speed things up again. You will soon learn how long normal operations take like loading maps so you will know if things slow down and it is necessary to reset. If you find that your unit locks up when you have multiple maps loaded, it may be necessary to temporarily turn Look-Around mode off or to reduce the number of maps loaded at one time.

There are occasional streets, which split off in multiple directions with the same name. This may happen at a Y in a road or at a roundabout or intersection with an unusual island. The result might be the announcement of a street being, ahead, behind and to the left or right, all at the same time.

When doing an address match, it is best, but not always necessary, to enter the street name without a prefix like Northwest or a suffix like Avenue. You will receive a list of options once the matches are found. In Portland Oregon, it is necessary to enter the prefix before numbered streets, for example, SE 12th. If you just put in 12th, no match will be made. Add the two-letter abbreviation before 12th, and it works. This is the exception to the rule however.

When inserting cards in the Compact Flash or Storage slots, wait 5 seconds or so after they are recognized before loading the GPS application. If you hear the message that no maps were loaded, go back to the main menu and press G to start the GPS application again.

Map Data

See the GPS manual for complete instructions on adding and subtracting maps from your Compact Flash or storage cards. The map files must be in a folder on the Storage or Compact Flash card. Make sure all files for each state are in the folder and not in a sub folder for that state. States with multiple segments will have one extra file with the entire state city list. Do not transfer maps from the Storage card to the Compact Flash card on the BrailleNote as this can take a very long time and there is the potential for a crash.

There is a Multiple City directory under the Documents folder. If you wish to have for example only 1 of the four California maps loaded and need to find out which map your California city is in, do a search on that city in the California Multiple City document. The relevant map, like CA1 will be listed after the city name.

Note: You must have the California.cty file included in the map folder for one or more of the California map segments to work properly. This means however that any city in the state can be matched even if it is not in the individual map segment loaded. To avoid confusion, we recommend against loading individual map segments.

When entering a state name in the Address State field like Alabama, you will need a letter-sign before the letters AL on the BT keyboard to distinguish it from the contraction for “also.” The same goes for TN for Tennessee.

Points of Interest:

To learn about the available POI categories, press Chord F on your BrailleNote and enter the search distances. The categories will then be presented and you can arrow through them. They are not in alphabetical order. Once you know what the category options are, it is best to select a category by pressing its first letter. A POI list is also available within the Look Around mode, Chord M.

Note: Washington DC is within the Maryland points of interest file.

Should you wish to change the way a category name is pronounced, you can use the BrailleNote’s pronunciation dictionary. For example, rename Gas to be pronounced as Fuel or Petrol.

In GPS 3.3, the US POI CDs are structured with sub folders for each state. One CD contains states from A to M and the other from N to Z. There are multiple POI files for each state. Copy the files you wish into the POI folder on the Compact Flash card.

GPS V3 Maps Regionally or Alphabetically

By default, the GPS program first looks for map files in the Maps folder and Points of Interest in the POI folder on the Storage card and second on the Compact Flash card if it doesn’t find them on the Storage card. If you wish to use other folders, you must set the desired map folder under the GPS Options menu.  Change the associated POI folder also under the GPS Options menu. See the Map section in the User’s manual for details about copying and using maps. We recommend transferring the maps directly from a Compact Flash card on the PC to the BrailleNote rather than using Active Sync or by transferring from the BrailleNote’s Storage card.

If you wish to arrange maps regionally, here is a recommended arrangement. First find the suggested map folder name followed by the number of maps in that folder. Note the numeral before some of the states indicating multiple map segments for that state. Set your home region with the folder name “Maps” so it will load by default.

Maps1 West: 7 maps

4California, Nevada, Oregon, Washington

Maps2 Southwest: 7 maps

Arizona, Colorado, Idaho, Montana, New Mexico, Utah, Wyoming

Maps3 Midwest: 7 maps

Iowa, Kansas, 2Missouri, Nebraska, North Dakota, South Dakota

Maps4 South-central: 8 maps

Arkansas, Louisiana, Mississippi, Oklahoma, 4Texas

Maps5 Central: 8 Maps

2Illinois, Indiana, Kentucky, 2Michigan, Wisconsin, Minnesota

Maps6 Southeast: 8 Maps

Alabama, 2Florida, 2Georgia, North Carolina, South Carolina, Tennessee

Maps7 East: 8 Maps

Delaware, Maryland/DC, 2Ohio, 2Pennsylvania, Virginia, West Virginia

Maps8 East Coast: 9 Maps

Connecticut, Massachusetts, Maine, New Hampshire, New Jersey, 2New York, Rhode Island, Vermont

Maps9 Offshore: 4 maps

Alaska, Guam, Hawaii, Puerto Rico

Maps Organized Alphabetically

The maps are organized on the 8 U.S. CDs. As follows. Ten of the states have a number indicating how many maps segments are in that state.

Note that Washington DC is in the Maryland map. Enter Maryland as the State and Washington as the city when setting an address.

Disc 1: AL-CA

Alabama, Arizona, Arkansas, 4California

Disc 2: CO-ID

Colorado, Connecticut, Delaware, 2Florida, 2Georgia, Guam, Hawaii, Idaho

Disc 3: IL-ME

2Illinois, Indiana, Iowa, Kansas, Kentucky, Louisiana, Maine

Disc 4: MD-MT

Maryland, Massachusetts, 2Michigan, Minnesota, 2Missouri, Montana

Disc 5, NE-NC

Nebraska, Nevada, New Hampshire, New Jersey, New Mexico, 2New York, North Carolina

Disc 6: ND-RI

North Dakota, 2Ohio, Oklahoma, Oregon, Pennsylvania, Puerto Rico, Rhode Island

Disc 7: SC-TX

South Carolina, South Dakota, Tennessee, 4Texas

Disc 8: UT-WY and AK

Alaska, Utah, Vermont, Virginia, Washington, West Virginia, Wisconsin, Wyoming

For technical support, contact the dealer from whom you purchased your GPS. Direct US Sendero customers call, 1-888-757-6810.

Copyright, Sendero Group, 2005

Please read other documents for a complete understanding of the BrailleNote GPS software including the manual, the audio tutorial and the optional GPS CourseWare.