Sendero’s Holux Getting Started Guide

Copyright 2005-2006

By Sendero Group

See the Holux GPSlim 236 User’s Guide for details about receiver specifications.

Holux Package

a. The first layer of the Holux box contains the battery wrapped in plastic on the left and the receiver on the right. If the battery is not on the left, then it is inside the receiver.

b. The body of the box contains the receiver soft case, a car power adapter and the AC charger.  The separate power cable can be used with either the AC or 12-volt plugs. This cable can also be used to connect the receiver to a computer USB port or to the mPower as a temporary source of power should the battery run out. It is necessary for the mPower to be turned on in order to supply power to the receiver via the USB port.

c. On the bottom of the box beneath the inner carton, you will find a print Holux manual, warranty card and CD with computer connectivity software not relevant for connection to the BrailleNote.

Holux Description

Situate the receiver with the rubber feet on the table and the three vertical ridges near the bottom edge closest to you. Those ridges are the status lights, orange, blue and green/red.

The on/off slide switch is on the left edge toward the bottom. Up is On, down is Off.

At the top left edge, there is a small hole for an optional external antenna.

On the bottom edge facing you, is a rectangular slot for the power/interface cable. The connector slides in only one way. You may be able to feel that the edge of the plug, which is supposed to face up, is slightly narrower than the lower edge. The sleeve of the connector is also smoother on the top than the bottom. This is a friction fit connection.

Flip the unit over to access the battery compartment. The entire back comes off by prying it from the top edge. This is at the opposite end from the charging jack. Once the compartment is open, the battery contacts are in the right corner nearest you. Find the matching contacts on the battery. The battery will fit easily if it is oriented correctly. Replace the battery compartment cover and make sure it is firmly snapped in place and put the receiver in the soft case.

Important Points about the Holux Receiver

This is one of the only GPS receivers with a physical On/Off switch.

The receiver uses the high sensitivity SirfStar III GPS chip set. Most manufacturers including Holux do not enable WAAS. A custom version of the Holux receiver has been made for Sendero with WAAS enabled. WAAS can increase accuracy to 10 feet. The BrailleNote GPS indicates when the WAAS satellite is engaged.

Because of higher satellite sensitivity, the Holux may pick up signals even inside buildings although the heading information may not be consistent.

The receiver may work fine inside a pocket or purse, however, a clear view of the sky is always recommended for best results.

The battery life should be around 10 hours and it takes about 2 hours to fully charge. Coincidentally, the battery is the same type as the battery used in many Nokia phones such as the 6620 and 6682. The Holux battery is 3.7 volts and 1000 MAH.

The Sendero Holux case offers two methods for attachment. It has a slide-on clip as well as Velcro wings which can be looped around the shoulder strap for extra security.

The strap and case:

A custom shoulder strap is provided for the PK. It has a belt loop to accommodate the clip on the back of the receiver case. The receiver is best situated on top of the shoulder so it can also be clipped on the sliding shoulder pad on either the PK or mPower straps. You can remove a portion of the PK strap to convert it into a PK handle by disconnecting the buckles located a couple inches from each end of the strap and reconnecting the shorter portions as a long handle. The mPower comes with the appropriate shoulder strap.

Configuring the BrailleNote PK or mPower for a Bluetooth GPS receiver with KS7 or higher

Summary of connection sequence:

* Options plus C then B gets you to the Bluetooth Menu. (BrailleNote Options followed by C for Connectivity and B for Bluetooth)

* Select the receiver and Activate Activesync.

Installing the Sendero GPS software:

When you first receive a PK, it may be necessary to press C1 and C4 together to take the unit out of deep sleep mode. Install the GPS software on the PK or mPower in the normal way, holding down dots 123 with a press and release of the C3 and C4 buttons. Use the normal Reset button on the mPower. See the respective manual for details on installing programs.

Holux Connection Details:

- Charge the Holux for 2 hours and then turn it on with the on/off switch on the left edge. Up is On. See the receiver layout description below.

- From the BrailleNote Main Menu, Press Chord O for Options, C for Connectivity Menu, and B for Bluetooth, Y for Yes (Bluetooth On), and Y for Search for Bluetooth devices. Beeps will indicate searching.

- Press Enter on the Holux selection. Space through the list of services and press Enter on Activesync. Select Yes for Pairing. Make sure all other Activesync connections are closed before trying to use the GPS Bluetooth.

- Enter 0000 for the authentication code when asked and press enter. Use a number sign before the zeros for Grade 1 or 2 Braille.

- Space through the list until you see Activesync and press Enter and A to Activate. Make sure you see “Activesync Active.” If you don’t see the word, “Active”, repeat the steps to activate Activesync.

- Go to the BrailleNote Main menu, wait 10 seconds and then select the GPS Navigation application. There is a pause followed by an announcement of the GPS Menu. If you hear Bluetooth not detected, go back to the main menu, wait a bit and try again. Otherwise, return to the Connectivity Menu and check to see that Activesync is indeed Active.

Once the GPS program loads, the Holux will start to acquire satellites. When you are in the GPS program, you can press G to find out the receiver and satellite status.

Pairing with the Bluetooth Receiver

It is best to be familiar with Bluetooth pairing as the GPS receiver may become unpaired for several reasons like resetting or perhaps when pairing with other Bluetooth devices. It is not hard to pair once you become familiar with the drill.

If you open the GPS application without first turning on the Holux, you will receive an error message indicating you should turn it on and/or reactivate the Bluetooth connection in the BrailleNote Options menu. Unless you have reset the BrailleNote, you should return to the Main Menu, turn on the Holux and load GPS again. If you turn the BrailleNote off while in the midst of the GPS application, you will have to turn it back on, go to the Main Menu and launch the GPS application. If all the Connectivity settings are correct, try waiting 10 or 15 seconds at the Main Menu before loading the GPS application.,

The Holux may stay in contact with the BrailleNote 30 or 40 feet away. If the Holux and BrailleNote get too far apart and the connection is lost, it will usually be necessary to go to the Main Menu and reload the GPS again after the units are within range of each other.

If you connect the BrailleNote to another Bluetooth device like a cell phone, you may first have to deactivate the Holux or reset the BrailleNote. It would then be necessary when you want to run the Holux GPS again to reactivate the PK or mPower for the Activesync GPS connection.

When you reset, the Bluetooth connection must be reestablished as follows:

- From the Main Menu, press Space O for Options, C for Connectivity Menu and B for Bluetooth and Y for Yes to Turn Bluetooth on if it isn’t already. Say No to Searching for devices and space through the list until you see Holux GPS and press Enter.

- Space through the list and press Enter on Activesync.

- You will get the message, “remove pairing with Holux GPS and Activesync status.” You will need to activate Activesync, press Reset and then A to activate the Holux. Make sure it says Activesync Active.

- Return to the Main Menu, wait 10 seconds and open the GPS application.

Note: If you have to do a hard reset, it will be necessary to search for devices and pair again with the Holux receiver.

For Technical Support on the BrailleNote and GPS, contact the dealer from whom you purchased your product. Sendero Group direct customers call 1-530-757-6800. HumanWare customers call 1-925-680-7100.