Product Specifications

i.Trek

TM

Personal Navigation System

User Manual

Semsons & Co., Inc.

4269 E. Live Oak Ave. Arcadia, CA 91006

Copyright 2004,

Tel: 626.574.0507 Fax: 626.574.0072

www.semsons.com

Hardware Features

•

Sat / Bat LED serves dual purposes. It indicates battery

level and satellite status.

Flashing Green – The battery is low and requires charge.

Orange – The battery is charging.

Red – The GPS is searching for satellites.

Flashing Red – The GPS position has fixed.

•

Press Power Switch for 2 seconds to turn on i.Trek BT

GPS. The LED1 will flash in red and green, and

Bluetooth LED will flash in blue.

•

Bluetooth LED will flash rapidly when searching for

Bluetooth signals. Bluetooth LED will flash slowly at

once per second when transferring data.

Package Contents

•

Bluetooth GPS receiver x 1

•

Battery x 1

•

AC Charger x 1

•

Car Charger x 1

•

User Manual x 1

•

Quick Start Guide x 1

•

External Active Antenna x 1 (Optional)

FAQ and Trouble Shooting

•

Does BT GPS require driver?

No, BT GPS does not require driver to run. As long as the

host device supports Bluetooth and serial profile, it will

recognize BT GPS through discovery mode.

•

The BT GPS is successfully paired through Bluetooth

Manager. However, the GPS mapping software can not

recognize the BT GPS.

Only one program can access the GPS at one time. Please

make sure all GPS programs are turned off except for one

you intend to use.

•

The BT GPS has received satellite signals. However, it

can not acquire a fix.

GPS stores the satellite data in the internal memory. If you

have not used GPS for a while or have traveled a long

distance, please remove the battery for 5 min. and let the

GPS reset.

•

The BT GPS takes long time to get a fix.

When you first use the GPS, please make sure that you are

outside with good view of the sky and stay stationary. It may

take about 2 minutes to locate the satellites for the first

time. However, it will locate much faster during the

subsequent uses.

•

The BT GPS can not be paired after the battery is

completely drained.

When the battery is completed drained, the unique paring ID

is lost. Please delete the BT GPS short cut and the paired

device from the Bluetooth manager, restart the host device,

and re-pair the BT GPS again.

•

Where to find more support information?

More information is available at www.semsons.com Also, you

may send questions to semsons@pacbell.net or call for

technical support at (626)574-0507.

•

How long is the warranty and where to exchange the

defective product?

The warranty is one year from the date of purchase.

Replacement will be issued directly by your authorized

dealer.

Adobe PDF Document End