Getting Started with SporTrak GPS

Note that there are multiple models of the SporTrak. This document refers to the basic Sportrak, not the Pro or other models.

Overall Description:

A form-fitting leather case covers all but the top inch of the SporTrak. A clear plastic window covers the receiver’s screen with buttons on the lower half of the receiver’s face. The lower right edge of the receiver leather case has a snap-button covering the battery compartment. The back of the leather case has a belt-clip and a cable connected just above it.

Description of buttons::

Note, all buttons beep when the unit is turned on

The screen is on the top half and the keypad is on the bottom half of the receiver’s surface.  There are eight small buttons arranged in a semi-circle around a large round multi-directional button.

List of Buttons along the Left:

Top, Cancel/Quit

* Middle, Navigation

Bottom, Mark

* The large round button acts as a multidirectional arrow for menu navigation.  It recognizes: up, down, left, right.

List of Buttons along the Right side:

* Top right corner, Enter

Second Down, Goto

* Third Down, Menu

* Bottom right corner, Power.  (Red in color and smaller than the rest)

The lone button below the center round button is the light control.

Connections:  The serial cable should already be connected to the back of the receiver.  The other end of the cable plugs into the serial port on the back of your BrailleNote or VoiceNote per the instructions in the User Manual.  The connector is longer on the top edge than the bottom.  Make sure the connector is up the right way.  Gently tighten the thumbscrews.

If you need to connect or disconnect the cable from the back of the SporTrak, find the screw, which secures the connector, just to the right of the cable on the housing of the connector. Flip the small metal ring up to turn the screw. Do not twist the housing of the connector itself. When you attach this connector, push the screw forward to line up with the matching hole between the contacts on the receiver. Be careful with this connection and do not over tighten the screw.

Power:  Hold the SporTrak vertically facing you. Press and release the small, lower right red Power button and you will hear a beep. Wait one second then press the upper right Enter button. It will also beep when you press it. If you don’t press the Enter button, the unit will power off automatically in 5 seconds. This keeps it from accidentally turning on.

To verify that the SporTrak is on, you may press the lower center button (just under the large circular button).  This is the light button.  When the light is on, there is a faint hum that can be heard from the receiver. Press the light button once for medium, again for high and a third time to turn it back off.

To turn the power off, press the small lower right button.  There is a 5 second delay while the receiver powers down.  If any key is pressed during this time, the power will stay on.

Batteries:  Two AA batteries are already installed in your unit.  To change batteries, first lay the receiver face down on a table and unfasten the buttoned case-strap at the bottom right side of the receiver.  Under the strap is a ring handle. Unscrew the battery-tray using the ring handle.  After approximately 20-30 half turns of the handle, you will be able to pull the battery tray from the receiver using the handle for leverage.

The orientation of the batteries is unconventional. Face the ring-handle end of the tray toward you. There are two springs on that end. The battery on the left should have the positive pointed end facing the spring. The battery on the right has the flat negative side on the spring in the normal fashion.

The tray is form-fitted so that it will only fit one way back into the unit.  While placing a small amount of pressure on the tray with your thumb, use the ring handle to screw it back into place.  The tray should be flush with the unit.  Remember to fasten the buttoned strap back in place.

You will normally get 15 hours or so of continuous use on a full charge or new set of batteries.  When you remove the batteries, you have at least 12 hours to install new batteries before memory is lost. A string of beeps occur when battery power is about to run out. You may have 10 to 20 minutes before power is actually lost. There is a visual battery meter at the bottom of the screen just above the keypad.

Acquiring a Position:  Place the receiver in clear view of the sky or near a window to begin acquiring GPS satellites.  When you travel to a new area or if the unit has been off for several days, it will take longer to determine the GPS position.  You do not have to have the BrailleNote turned on to begin acquiring satellites with the receiver.

To see if you are tracking, turn on your VoiceNote or BrailleNote and select the GPS application in the Main menu and then press Enter on Start. When the application first opens, you will either hear “GPS not detected” or GPS quality and number of satellites. Press G at any point to check GPS status. When you turn the receiver off, the “not detected” message will trigger on the BrailleNote to confirm that the power has been switched off. Remember that after you press the receiver’s power button, you must wait 5 seconds before it powers down.

HOW TO RE-INITIALIZE THE SPORTRAK

It will not normally be necessary to initialize the receiver.  The receiver is shipped from the factory with batteries and is preconfigured.  If the batteries are out of the unit for more than 12 hours, it may be necessary to initialize.  If the batteries are just low, it will retain its previous initialization for days or weeks.

If your SporTrak needs to be initialized, it is recommended that you have sighted assistance.  However, if you follow the instructions below, you may be able to get the unit to communicate on your own. See the layout of the buttons as described at the top of this document and do the following.

To re-initialize for the U.S.:

With the unit powered on, press Menu, arrow down twice, Enter nine times, and then press the Navigation key.

Note, this will reset your unit to the last state used. If you wish to initialize for a specific state, see the detailed initialization below. It is not essential to initialize to the state, however, It will speed up the GPS acquisition time.

To initialize for the U.K.:

With the unit powered on, press Menu, arrow down twice, press Enter 2 times,  arrow down 3 times, press Enter once, Up arrow twice, press Enter 5 times, and then press the Navigation key.

Detailed Re-Initialization Settings

With the unit powered on including pressing Enter to accept the copyright information, press Menu, (the second button up from the bottom right).  You will be at the menu.

Press Down twice to highlight Setup.

Press Enter once to choose Setup.

Press Enter to choose Initialize.

You will be at an item saying Coordinates.

To select US, press down once, and then press Enter.

To select Europe, press down three times, then press Enter.

To select Australia and New Zealand, press down six times, then press Enter.

The complete menu region list is:

USA, Americas, Europe, Asia East, Asia West, Australia, Africa, Pacific, Atlantic, Manual.

Once you make the region or Country selection, there is a list of countries or states as appropriate to choose from.  For the US, the first item is Alabama and the states are listed downward in alphabetical order.  Press Enter once you highlight the state you wish. For example, arrow down 4 times to highlight California.

Note, It is not necessary to make the individual state selections. It can help the receiver however acquire a position faster if the individual country or state is selected.

The next Item is elevation. Press enter

The next item depends upon whether you are changing to a state in a different time zone. If so, the Time item will pop up. If not, you will be presented with the Marine/Land option. Press Enter to accept Land and you should be back at the Navigation screen, ready to go.

This is the end of the menu.  Press Navigation (the second button down on the left) to return to main functionality.

Again, it is not necessary to re-initialize unless the receiver will not track. If you change countries, it is not necessary to change the other set-up options, just the country selection.

Note, when you initialize a brand new receiver for the first time, there will be other options, which do not appear when re-initializing. An important item to set during this first-time initialization is NMEA. This must be set to V2.1 GSA. Instead of picking Initialize, arrow down 12 times, press Enter, arrow down 3 times and press Enter again.

Be sure to read the BrailleNote or VoiceNote GPS User Manual for more information.  For technical support, refer to the full SporTrak manual also available in the Documents folder of your Storage card.

Thales Magellan Corporation technical support can be reached at 1-909-394-5000 for support on the receiver only.

Sendero Group, 1-530-757-6800, GPS@SenderoGroup.com