The following information on the Earthmate has been compiled by Sendero Group from multiple Delorme documents.

Caution: Make sure to depress the release on the receiver and on connectors before disconnecting. If  not, you can inadvertently pull the connector into pieces.

Delorme Earthmate

Features and Specifications

To Verify the Earthmate GPS is Acquiring Satellite Information

The Earthmate has a LED that indicates the type of satellite information it is obtaining. When the LED is:

•	Red, the Earthmate is acquiring satellite information.

•	Yellow, the Earthmate has a 2-D fix (navigating with fewer than four satellites).

•	Green, the Earthmate is receiving sufficient satellite information to determine a full 3-D fix.

Note: To help conserve power when attempting to acquire satellite data, the Earthmate shuts off automatically after 5 minutes of no GPS fix. (The LED shuts off after 10 seconds). Then, every 30 minutes, the Earthmate powers on for up to 5 minutes to try to acquire a 2-D or 3-D fix.

Using the Earthmate PowerPack

The Earthmate PowerPack lets you use the Earthmate GPS on your handheld computer by providing battery power to the Earthmate and a cabled connection to the handheld computer.

What the Power Pack Requires

4 AAA batteries or a rechargeable Li-ion battery.

To Place the USB Earthmate GPS Receiver On Top of the PowerPack

To place the USB Earthmate GPS on top of the PowerPack, place the USB Earthmate GPS receiver on the cradle just above the 12-pin connector. Then, slide the receiver so that the device is connected to the PowerPack. Do not slide the device down the PowerPack from the top of the PowerPack.

To Release the USB Earthmate GPS Receiver from the PowerPack

To release the USB Earthmate GPS from the PowerPack, press the release button below the USB Earthmate GPS cradle.

What the Lights on the PowerPack Indicate

Both the Earthmate PowerPack and the Earthmate Bluetooth PowerPack have an upper LED and a lower LED.

•	The lower LED displays the same for both the Earthmate PowerPack and the Earthmate Bluetooth PowerPack. The lower LED displays green while the battery is charging, turns off once the battery is fully charged, and displays red when the battery is low.

•	The upper LED on the Earthmate PowerPack flashes yellow when there is no GPS connection and displays solid yellow when a connection is established.

Note: An optional USB cable can be obtained for direct connection from the Earthmate receiver to a computer with a USB port and the receiver will be powered from this USB port. An optional Bluetooth PowerPack is available for wireless connection to computers with Bluetooth capabilities.

Helpful Information: www.delorme.com/support provides help with installation or other issues you may have and includes all the latest downloads to enhance your software.

Installation Support: Visit  www.delorme.com/support or email  support@delorme.com.

Customer Service: For general questions about our products, returns, order status, or problems with shipments, phone 800-511-2459 (Monday thru Friday).

Direct Sales: To place additional orders, phone 800-561-5105 or visit www.delorme.com.

Specifications

NEW Earthmate® GPS Receiver

Smaller...Faster...More Accurate...PC Compatible

Now get the improved accuracy of WAAS-enabled GPS positioning when you connect the new Earthmate GPS directly to yourVoiceNote or BrailleNote. See

your position updated in real-time as you track using BrailleNote GPS.

At the heart of the new Earthmate is the SiRFstar IIe low-power chipset, with cold-start times of less than 45 seconds and special technology providing

improved GPS tracking in weak signal conditions. This chip set requires 75% less power to operate than comparable chips.

Many customers have reported start-up times under 5 seconds! Now you can access the type of premium performance GPS tracking found in high-end systems at

new low price points.

Earthmate GPS includes a built-in LED that transitions through the various satellite acquisition modes: red for searching, yellow for two satellites in

view, and green for at least three satellites in view. Earthmate GPS is a 12-channel NMEA-compliant receiver.

Never Get Lost Again

When was the last time you wasted valuable travel time because you took a wrong turn in an unfamiliar location? Now there's an affordable and reliable solution–the

new Earthmate GPS receiver. The small and flexible Earthmate GPS receiver is a high-performance, easy-to-use link to the satellite navigation technology

of the Global Positioning System (GPS). Accessing these satellites is available for no additional charge. Earthmate, connected to aBrailleNote GPS shows

your real-time position on detailed street maps anywhere in the U.S. You will always know where you are and where you need to go.

GPS Specifications

• NMEA-compliant 12-channel receiver

• WAAS-enabled

• SiRF starIIe/LP low-power GPS chip set

• Cold start: <45 seconds; warm start: <38 seconds

• Hot start: < 8 seconds

• Power: full power: 175mW; Trickle Power: <60mW

• Maximum Velocity: 1000 knots

• SiRF SingleSat™ updates for improved reduced visibility GPS

• Superior urban canyon performance

• SiRF FoliageLock™ for weak-signal tracking

Environmental Characteristics

• Operating temperature range -40 ºC to +85 ºC (UART output), 0 ºC to +70 ºC (USB output)

• Storage temperature range -55 ºC to +100 ºC

• USB IC of Cypress is approved at 0 ºC to +70 ºC temperature

SiRF GSW2 software will be fully compliant with, and will conform to, the Rockwell International Corporation, Navstar GPS Segment, Navigation User Interface,

ICD-GPS-200, Rev C, specification, dated 10/13/93.

Altitude accuracy is < +/- 35m Vertical in term of 95% and open sky.

Battery Life

The PowerPack will last 11-12 hours with the Li-ion battery pack.

Warranty

60 days