BrailleNote with Garmin GPS 10

Getting Started Guide

Copyright 2005 By Sendero Group

This document pertains to the BrailleNote PK and Mpower with Bluetooth wireless capabilities. We will use the abbreviation BN when referring to both units.

Configuring the BrailleNote for a Bluetooth GPS receiver

A replacement shoulder strap is provided for the PK. It has a belt loop to accommodate the clip on the back of the receiver case. You can also remove a portion of the strap to convert it into a handle by disconnecting the buckles located a couple inches from each end of the strap and reconnecting the shorter portion as a long handle. The Mpower comes with a regular shoulder strap with loop for the receiver.

Install the GPS software on the BN in the normal way. See the BN manual for details on installing programs. You will also find that the Compact Flash card containing the maps and GPS program is called “Storage Card” in the PK drive list.

- First charge and then turn on the GPS10 by holding down the Power button. See the description of the receiver and components below.

- On the BN, press Space O for Options, C for Connectivity Menu, and B for Bluetooth, Y for Yes (Bluetooth On), and Y for Search for Bluetooth devices. Beeps will indicate searching.

- Press Enter on the Garmin GPS 10 selection. Space through the list of services and press Enter on Activesync. Select Yes for Pairing. Make sure all other Activesync connections are closed before trying to use the GPS Bluetooth.

- Enter 1234 for the authentication code when asked and press enter. Use a number sign for Grade 1 or 2 Braille.

- On the mPower, not the PK, press Reset. Then, go back to the Bluetooth connection and again Activate Activesync.

- Back out to the Main menu and select the GPS application. Once the GPS program loads, the GPS10 will start to acquire satellites and the flashing blue light will slow from once a second to once every two seconds. There is no audible equivalent of the status lights. However once you are in the program; you can press G to find out the receiver and satellite status. If the Bluetooth receiver is not detected, you will receive an indication right after the map-loading message.

Pairing with the Bluetooth Receiver

It is best to become very familiar with Bluetooth pairing as the GPS receiver may become unpaired for several reasons like resetting or perhaps when pairing with other Bluetooth devices. It is not hard to pair once you become familiar with the drill.

If you open the GPS application without first turning on the GPS10, you will receive a “Bluetooth receiver not detected” message. You should back out to the Main Menu, turn the receiver on and/or reactivate the Bluetooth connection in the BN Options menu. Unless you have reset the BN, you should return to the Main Menu, turn on the GPS10 and load GPS again. If you turn the BN off while in the midst of the GPS application, you will have to turn it back on, go to the Main Menu and reload the GPS application.

If the GPS10 and BN get more than 30 feet or so apart, and the connection is broken, it will be necessary to go back to the Main Menu and reload the GPS again after the units are within range of each other.

If you connect the BN to another Bluetooth device like a cell phone, you may first have to deactivate the GPS10 or reset the BN. It would then be necessary when you want to run the GPS10 GPS again to reactivate it.

If you reset, the Bluetooth connection must be reestablished as follows:

- From the Main Menu, press Space O for Options, C for Connectivity Menu, and B for Bluetooth and Y for Yes to Turn Bluetooth on if it isn’t on already.

- Say No to Searching for devices and space through the list until you see Garmin GPS 10 and press Enter.

- Press Enter on Activesync.

- Look for the message that says, “Remove pairing with GPS10 GPS or Deactivate.” Press D to Deactivate it. This is only necessary on the PK.

- Press Enter on Activesync and A to Activate it. It should say “Active Sync Active.”

- After you have reactivated the GPS10, return to the Main Menu and open the GPS application.

Note: If you have to do a hard reset, it will be necessary to search for devices and pair again with the GPS 10.

Other Nuances about the BN and GPS

Active use of The BrailleNote BN’s Bluetooth link will reduce BN battery life.

The Media Player can play audio while the GPS application is running.

GPS10 Receiver Description

The only button on the face of the GPS 10 is for Power and it is located approximately one half inch from the bottom beveled edge. The arrow-shaped point of a rubber surface texture terminates at the power button. You should be able to feel two small LED lights on the beveled edge. The left is a yellow charger indicator and the right is the blue GPS indicator.

To turn the GPS 10 on, press the button for about one second. To turn it off, press and hold for about 2 seconds or more.

When you plug in the GPS 10 to be charged, the Power is automatically turned on. This is true with both the AC charger and the car adapter. Therefore, if you wish to just charge the unit and not use it at the same time, press and hold the Power button after you plug it in to charge in order to switch off the unit.

Be careful when carrying the GPS 10 in a bag that it is not inadvertently switched on. It is easy for the Power button to be pressed and then the power will be drained. If you put the GPS 10 behind something like a window shade in an airplane, the Poer button can be pressed against the window and hence switched off. Because the front of the GPS 10 is slightly curved, try to position the unit so the Power button is not being pressed against anything.

The power jack is located on the right side in the bottom corner and it will accommodate either the 12-volt cigarette lighter adapter or AC charger.

The actual antenna is not visible but it is located toward the upper part of the front surface. It is about one inch square.

The entire back of the GPS 10 is magnetic so it can stick to a metal surface. Keep that magnetic surface away from media like tapes or compact flash cards.

The Sendero case for the GPS 10 has a belt clip on the back and a flap with snap for the opening at the top. The GPS 10 Power button falls at the very bottom of the hole in the face of the case when you have it oriented right side up. Neither the lights nor the charging jack are available in this configuration. If you put the GPS 10 in the case up side down, the Power button and lights can be accessed when the flap is open and the charging jack will also be accessible. Experiment with these two configurations to see which is best for you.

See the Set UP document for details on the specifications and operation of the GPS 10.

For Technical Support on the BrailleNote BN and GPS, contact the dealer from whom you purchased your product. Sendero Group direct customers call 1-888-757-6810, extension 111. HumanWare customers call 1-925-680-7100.