Delorme Earthmate

Getting Started

Copyright 2004 by Sendero Group, LLC

Caution: Make sure to press the release before disconnecting the Earthmate receiver from the battery pack or when disconnecting cables. Otherwise, the connectors can be pulled apart.

Earthmate Description:

The receiver is roughly one and a half inches square and has a slot on the bottom edge for the extension cable or to snap directly into the battery cradle. This cradle contains a battery compartment on the back. The bottom edge of this battery cradle has a jack for the small Earthmate end of the serial cable with a slide power-switch on one side of the jack and the hole for the power adapter on the other side.

To open the battery compartment, gently pry the back door open at the top back of the unit. This may require pressing a thumb nail or thin object in and down. Note the two plastic feet are near the edge so you know which way the compartment door goes when you replace it.

Your Earthmate comes with a rechargeable battery pack. This pack has a small contact area near the back bottom edge. If you take it out, orient the pack with this small contact area near the bottom edge facing the inside of the unit when you replace it.

The battery pack has a print label, which covers about three quarters of the upper surface. It nearly reaches the upper edge of the pack and about an inch from the lower edge. Orient the top end of the pack at the top of the battery compartment. The metal contact area is on the opposite side and end.

You can also use 4 regular triple-A batteries in the receiver in case you forget to charge the pack. These regular batteries are oriented at 90 degrees to the rechargeable pack so there is no chance of accidentally charging the standard batteries. They install in the normal fashion with the flat side against the spring.

Connecting the Earthmate to the Cradle and to the BrailleNote

The unit should already be connected when you receive it. Unbundle the receiver cable so you can fasten it to the shoulder strap or place it near a vehicle window. Keep the cradle and excess cable zipped inside the accessory pouch. Avoid crimping or putting pressure on cable connectors.

Should you need to assemble the receiver components, connect the 8-inch serial cable to the BrailleNote and the opposite end into the base of the battery cradle. The connector on the cradle end should have the arrow facing the back of the cradle and a small release button on top.

Next, plug the female end of the USB cable on to the cradle and the opposite connector into the base of the Earthmate receiver. The backside of this male connector is smooth and the release button is on top. Zip the cradle and excess cable inside the accessory pouch.

Situating the Earthmate Receiver:

The Earthmate receiver should already be installed in the form-fitting case with belt clip. Slide the clip over the belt loop on the shoulder strap. Alternatively, you may put it on the strap’s sliding shoulder pad. Tuck the cable into the Velcro channel and place any excess cable into the accessory pouch. Try to position the receiver as high on your shoulder as possible.

The easiest way to put the receiver on the dash is to unclip both ends of the shoulder strap from the BrailleNote and lay it and the receiver on the dash. When you are ready to get out of the car, you can quickly reconnect the clips and be on your way.

A suction cup is provided to assist in securing the Earthmate on or near a vehicle window. There is a small slit in the base of this cup. Spread it apart and push the cord inside. Slide it up near the small connector near the base of the Earthmate. The plastic cup won’t typically stick to a dash but you can press it against the glass of the window to prevent the receiver from sliding around. It can also be used to assist in keeping the receiver against the glass of a bus or train window. Experiment with configurations that suit the situation. You can even place the Earthmate up side down on the ledge of the window with the suction cup above. Don’t count on this being terribly secure but it will work if not severely knocked or shaken.

Connecting the Earthmate receiver directly to the battery/serial cradle:

For most purposes, this option is not preferable. The small end of the serial cable slides into the bottom edge of the Earthmate cradle. It will only go in one direction. A bump in the middle of the leading edge of the plug should be facing up with a tactile arrow toward the back. Squeeze the cable connector slightly to release the catch when you wish to remove the cable. Plug the larger DB9 serial connector into your BrailleNote. There is a belt loop near the sliding shoulder pad on the BrailleNote strap. If you are using the combined unit, slide the belt clip of the receiver into this loop making sure the clip is firmly fastened to the receiver.

The back of the battery pack accommodates a belt clip, which is mounted over the door to the battery compartment. Should you need to open the battery compartment, it is necessary to take the belt clip off first by sliding the release catch toward the bottom of the clip. The release catch is indicated by a rough surface at the base of the clip. Push that down and the clip should come loose. To reattach the clip, place the 3 plastic pegs in the holes at the bottom back of the unit between the bottom feet and slide the clip up into place until you hear or feel a click. Use the clip if you are combining the receiver and battery module and mounting it on the shoulder strap.

Should you ever choose to use the alternate configuration and snap the receiver directly into the battery cradle, there is a release to separate the receiver from the pack. Be very certain to depress the release on the battery pack before separating it from the receiver. See the Earthmate manual for more details. Again, this combined arrangement is not recommended.

Firing Up the Earthmate:

GPS version 3 defaults to the Serial GPS receiver type for the Earthmate receiver. No need to change any settings if you are running version 3.

Use the Earthmate in the “Garmin” GPS receiver mode in GPS version 2.0. Go to the GPS Options sub menu and select GPS Receiver Type. Press G for Garmin to switch from the default Magellan receiver mode to the Garmin receiver type.

The power switch of the Earthmate is located on the bottom edge of the unit. Slide it toward the outside edge to turn it on and toward the cable connector to turn it off. If your BrailleNote GPS application is loaded, you will hear that the receiver is detected as well as the GPS status. Press G to recheck status from within the GPS application. The Earthmate will not start acquiring satellites until the GPS application says, “Maps are loaded.” When you are in other BrailleNote applications or in the Main Menu, the receiver is not acquiring.

The Earthmate manual says that it will time out if signals are not acquired within 5 minutes. This does not seem to be the case when used with the BrailleNote. If you haven’t acquired satellites for 20 minutes or more and you are in reasonably good view of the sky, go to the Main Menu, turn the Earthmate off for 10 seconds, turn it back on and reload the GPS application. Under most circumstances, you should acquire satellites at least within 15 minutes and normally within a couple minutes.

The most effective strategy is to turn the receiver on only when you are in view of satellites. If you are among tall buildings, find an open place to acquire a position. Once you start tracking, the receiver is more likely to hang on to a position fix even among tall buildings. It does have a hard time acquiring when mostly obstructed so find that open place first.

Note: Once you are tracking on the Earthmate receiver, the BrailleNote GPS may sometimes indicate 0, 1, or 2 satellites in addition to the normally expected 3-12 satellites. A minimum of 3 satellites is needed to establish a position but you may get some position information with less than 3 satellites. Very Good quality indicates that the additional WAAS satellites are being picked up for accuracy as good as 10 feet. On version 2.0, at elevations near or less than sea level, you will hear a bogus number like 214,958 feet. If you lose and then reacquire a fix, the altitude reading may be initially incorrect. This does not happen in Version 3.

Because the Earthmate uses a patch antenna, it is somewhat sensitive to its orientation to the satellites. Try tilting it slightly if you feel you should be getting better reception.

Charging the Earthmate:

Plug the AC power cube into an outlet and the round end into the Earthmate. This power jack is located on the bottom left edge. The AC adapter automatically switches between 110V and 220V. It has a U.S. style plug. A green light will appear when the unit is charging on both the power cube and on the receiver. The light on the Earthmate will disappear when the unit is charged at approximately 1 hour and 40 minutes. There is no audible battery alarm on the Earthmate. A 12-volt cigarette lighter adapter is included.

Battery life:

Assume approximately 10 hours of battery life for a new battery pack. This may diminish over time. Regular triple A batteries may last more or less than that depending upon their quality and amperage. It is much more convenient to use the rechargeable battery pack.

Technical Support

Contact the dealer from whom you purchased your BrailleNote GPS for support.

Direct Customers of Sendero Group may call 530-757-6800 or email GPS@SenderoGroup.com.

Delorme can be contacted at Delorme.com or at 800 561-5105.

The manufacturer’s warranty on the Earthmate is 90 days.