i.Trek BT GPS Quick Start Quick Guide

PC Connection

•

From Desktop, open My Bluetooth Places by clicking on

My Bluetooth Places icon.

•

Click Search for devices in range to search all

available Bluetooth devices.

•

BT GPS should be displayed with question mark

icon. Please enter "0000" for BT passkey.

•

Right click and select Connect SPP Slave. The

connection between computer and BT GPS will be

established. The BTGPS icon will become green

Copyright, Semsons.com 2004

i.Trek BT GPS Quick Start Guide

MS Street & Trips

•

Open MS Street & Trips and configure GPS setting by

select Tools -> GPS -> Configure GPS Receiver… and

a GPS configuration dialog box will open.

•

Select the COM port that virtual serial port is assigned

to. It is typically COM 3. You can verify the information

in system manager.

•

Display GPS information by Select Tools -> GPS ->

Track Position and GPS sensor dialog box will display.

•

When BT GPS has fixed the position, a GPS icon

will be displayed on the map indicating current location.

Copyright, Semsons.com 2004

i.Trek BT GPS Quick Start Guide

Pocket PC Connection

•

Make sure Bluetooth is enabled on the Pocket PC. The

Bluetooth icon should be solid blue.

•

Open Bluetooth Manager by click on Bluetooth

Manger icon.

•

Select New -> Connect! to start Connection

Wizard.

•

Highlight Explore a Bluetooth device and click Next.

•

Select a device and click Next.

•

Bluetooth wizard will search for available Bluetooth

devices and display them on the screen.

•

Click on BT GPS icon and available service will

display. Please enter "0000" as BT passkey.

•

Select SPP slave and click Next.

•

Select Finish.

•

Go back to Bluetooth Manager again and BT GPS

should be available as short cut.

•

Tap and hold and choose connect to establish

connection.

Copyright, Semsons.com 2004

i.Trek BT GPS Quick Start Guide

Mapopolis

•

Start Mapopolis application.

•

Select Tools -> GPS to open GPS screen.

•

Select Settings to open GPS option settings page.

•

Uncheck Auto-Detect COM port and set the setting

to COM 6 (COM port may be different) and Baud Rate

to 4800.

•

Click on Start to connect to GPSmart and to acquire

satellites.

•

When the BT GPS has fixed the position, a GPS icon

will be displayed on the map indicating current location

Copyright, Semsons.com 2004

Adobe PDF Document End