Sendero GPS, Version 4.2 ReadMe, December 27, 2007

Copyright 2002 - 2007, Sendero Group, LLC

This Read Me contains last minute information for Sendero GPS Version 4.2 for the KeySoft 7 BrailleNotes, VoiceNotes and PK as well as important information you should read before using the products.

Documents listing changes between all versions are in a file posted on the Sendero Download site as well as on the media you receive with your product. See the respective User Manuals for BrailleNote or GPS questions and instructions. Listen to the Audio Tutorial on the Compact Flash card or CD for a play-by-play of basic GPS functions. Optional GPS training courseware is available for purchase in Braille or print as is a Braille GPS manual. A document called GPS Accuracy is in the Documents folder. See the Getting Started document in the Receiver folder for specifics on the GPS receiver you are using.

KeySoft GPS compatibility list:

GPS version 4.x is compatible with mPower and PK units running KeySoft 7x. Version 4.x does not run on the Classic BrailleNotes.  GPS 3.51 is the last version to run under KeySoft 6.x.

Note: The only changes in version 3.51 over version 3.5 have to do with operation in the Australia latitude/longitude area.

KeySoft 5.1 or below runs GPS 1.0 to GPS 3.0

KeySoft 6 or greater runs GPS 3.1 to GPS 3.5 with a few exceptions. Speeds of GPS functions under KS6.1 will be slower than with KS7.

KeySoft 7 with GPS 3.1 or 3.3 will work but is not recommended.

Software Installation:

If you purchased the GPS at the same time as your VoiceNote or BrailleNote, the software may already be installed. If you purchased the GPS separately, or if you are installing an upgrade, see the Sendero GPS User Manual for instructions on installing software.

1. Important GPS 4.2 things to know:

** Maps and POIs

* Map and POI data has not changed from 4.0 to 4.2. Version 3.5 commercial data files will not work with 4.x. Your User.poi file will work with any version. Put all map and POI files in the same folder. If you put the old files in the folder, you will hear an error message, “unable to load maps or POIs.”

* The commercial Points of Interest for your state are located on the 1 GB or 8 GB Compact Flash card supplied with your new GPS package in the Maps folder and important documents are in their respective folders. The commercial POI file names begin with the numbers 40. Extra POI files, which can be found on Sendero’s free download page, start with the numbers 400. The map files are in the Maps folder beginning with the state name. If maps or POIs do not appear when you load the program, go to the GPS Options menu to point to the relevant Map/POI folder.

* For the 10 states that have more than one section, like California with 4 sections or Florida with 2, you must load the file with the .CTY file extension along with the 3 files for the state section you wish to use. If for some reason you wish to only have Northern California loaded, you would need to put the 3 files for California4 plus the .CTY file in the maps folder plus any POI files you need like 40California4.bpi.

* Washington DC is in its own file as of version 4 maps.

* Maps can be downloaded with a high speed connection from MySendero.com. Otherwise, the Map files are available on DVDs with new full systems or for purchase with upgrades.

* When the GPS program loads, it opens every POI file in the active folder. By having an image of each POI file in memory, searches and Look-Around functions are greatly increased. Although you can load up to 75 POI files from one folder and 75 maps, be aware that this is using up a lot of memory. We recommend that you limit the number of POI files to those you actively need. There are over 120 POI files for the U.S. If you wish to calculate a route across the U.S., only load the Core POI files so you have less than 75 POI files. We recommend splitting the total number of U.S. maps into 2 or 3 folders.

* There is a Multiple City folder containing listings for the ten states with multiple sections. If you wish to have, for example, only 1 of the four California maps loaded and need to find out which map your California city is in, do a search on that city in the California Multiple City document. The relevant map, like CA1 will be listed after the city name.

Note: You must have the California.cty file included in the map folder for one or more of the California map segments to work properly. This means however that any city in the state can be matched even if it is not in the individual map segment loaded. To avoid confusion, we recommend against loading individual map segments.

* When entering the first few letters of a state name in the Address State field like AL for Alabama, you will need a letter-sign (dots 5 6) before the letters AL on the BT keyboard to distinguish it from the contraction for “also.”

* The entire U.S. maps and POIs total just over 7 GB. This means you can easily put them on two 4GB cards or on a single 8 GB card. As stated elsewhere, you should make it a practice to load only the maps you regularly use in a folder. However, for purposes of driving or flying across country, you could divide the U.S. into a few folders depending upon the card size available to you. If you purchase the 8 GB option from Sendero, the maps are already installed in regional folders.

* Collective User POI files are available as free downloads from the Sendero site. These files have a .BPI extension just like the commercial POI files. These files with user points must have the word “user” in the file name. Otherwise, when you select the User category for a POI search, these points will not be found.

** Routes

* Users may wish to open routes they created with 3.5 and recalculate them under 4.x to take advantage of the new maps and routing engine.

When you arrive at your destination the route will automatically be closed. If you wish to save your route, it is recommended to save after the route is created.

** GPS

* If you get a “Bluetooth receiver not detected” message, first make sure the receiver is turned on and activated in the Bluetooth Connectivity menu. See the Getting Started document in the Receivers folder. It may be necessary to simply wait 10 seconds and press the reconnect hot key within the GPS application.

*The first time you Install GPS version 4, the GPS Receiver Option defaults to “no receiver.” Press O for Options, G for set GPS receiver and B for Bluetooth if that is what you are using. Then pick the receiver you have activated under the KeySoft Options Connectivity Menu. Serial works for the Delorme Earthmate and most other brands that use the serial port. If you have any of the Magellan models, especially the 310 or Sportrak, select the Magellan receiver option. A Bluetooth receiver is recommended for improved accuracy and time to first fix.

* The Sendero GPS software indicates when it receives the extra WAAS signal in the U.S. and Canada. The European version of WAAS is called EGNOS, but this system is not fully implemented as of this writing. So, if you get the WAAS indicator while in Europe, New Zealand or Australia, this does not mean you are picking up corrected signals nor is your GPS accuracy improved.

** Virtual Mode

* You should close a virtual route before setting a new virtual position. When following a route in virtual mode and you set your virtual position to some other location before closing your route and then move forward, your virtual position will move to the next waypoint along the route.  You can return to the location you previously entered by closing your route and using the undo command SPACE with dots 2 3 5 6 on the BT or CONTROL with Down Arrow on the QT.  You will be returned to your previous virtual position.

** KeySoft

* Make use of the BrailleNote pronunciation dictionary to change things like NB to Northbound, SB to Southbound, etc. Any frequently heard words which bother you in terms of pronunciation can be tweaked in this way. The dictionary does not allow you to mix punctuation, numbers and letters. For example, something like CA-113 cannot be changed to California 113. It will be pronounced as CA Minus 113.

* When inserting cards in the Compact Flash or Storage slots, wait 5 seconds or so after they are recognized before loading the GPS application. If you hear the message that no maps were loaded, go back to the main menu and restart the GPS application.

* It is highly recommended that you remove memory cards when your BrailleNote is turned off or before doing a reset. This is particularly important with 8 GB compact flash cards. They may be damaged if they are not removed. If you have a 4 GB or larger compact flash card and you are using a PK, please remove it from the unit before using the 1 2 3 reset for an installation. Then reinsert the card to complete the installation. If you don’t, you may get a KeySoft strings error.

* Starting with KS7, the Sendero GPS application is under the Navigation Menu. The hot key from the Main Menu is N. In KeySoft 6 and below, the hot key was G.

* Under KeySoft 7 with GPS version 4.x, there is a slightly different behavior after one presses a command. You will not hear the title of the command announced. You will hear the information generated by that command. This “key-echo” functionality was available under KeySoft 6 and has returned in 7.2 or later.

** Miscellaneous

* You cannot set the Multiple Repeat interval from 1 to 4 seconds. You can set it to 0 seconds for stationary mode or you can set it 5 seconds or greater. If you enter 4 seconds, it will simply default to 5.

* When data from the GPS program is appended to the clipboard and pasted into a BrailleNote document, be aware that computer Braille is being used. You would want to open this in a text document and not a Grade II Braille document.

* The number of lanes indicated by the detailed city command, which is Space with C on the BT and Read with C on the QT, is less evident in version 4.2 maps than in the previous maps.

* In the UK and Australia you no longer need to have street name descriptions set to long in order to hear the common name of the street you are on, and you can use the default "Short" street names. Note: the detailed city command gives both the short and long street names no matter what you have the Street Name set to.

* A feature that was introduced in Version 3.5 was to alert the user as to which side of the street the final destination will be on, but in Version 4.0 the accuracy of the indication of the side of the street is a lot more reliable. With 4.0 it is even possible to get the side of the street information without an address being included with the detail of a POI if the destination is more than 50 feet from the street. Note, if the destination is less than 50 feet and no address is associated with it, no side of street information will be presented.

2. Known Issues

** Maps and POIs

* There are two POI files for most states and provinces. One has the word Core in it and the other has the word Bonus. The file called bonus has points which have not yet been categorized. This means that POIs like restaurants may be in the Bonus file and not categorized in the core file. Using the “All” category option will give access to the restaurants in this situation. Remember that these maps and POIs will not work with versions prior to 4.0.

* In the case of a shopping mall, there could be dozens of POIs, which have different names but the same lat/lon based on a single geocoded address for that shopping center. The Look-Around mode will trigger on the first of these multiple points at the same lat/lon but it won’t automatically announce the rest of the points at that lat/lon. If you wish to hear all the POI names, you must press F to hear nearby points or Space with F on the BT or Read with F on the QT to conduct a detailed search to generate a list.

* The indexing of some towns in the map data may require a round about way of finding an address match. It is best to assume a town is in the database and to use the suggestions in the manual to find a means of pulling that town up if you don’t find it using a normal Address Look Up. In London for example, some addresses must be matched by putting in the council name rather than London as the city name. Medium to large cities may also be in the Points of Interest database and you can place your virtual position in the vicinity of a city by doing a POI search in this way.

* There may be some issues finding an address match or a cross street for a freeway that switches names. For example, US 131 may change to US-131 or US/131, even though it is the same street.  This may cause trouble with finding cross streets.

* When searching for a city or street which contains an accented character, it is necessary to enter that character exactly in order to find the match.

* There are occasional streets which split off in multiple directions with the same name. This may happen at a Y in a road or at a roundabout or intersection with an unusual island. The result might be the announcement of a street being ahead, behind and to the left or right, all at the same time.

* When doing an address match, it is best, but not always necessary, to enter the street name without a prefix like Northwest or a suffix like Avenue. You will receive a list of options once the matches are found. In Portland Oregon, it is necessary to enter the prefix before numbered streets, for example, SE 12th. If you just put in 12th, no match will be made. Add the two-letter abbreviation before 12th, and it works. This is the exception to the rule however.

** Routes

* The speech synthesizer Eloquence sometimes studders when a replay and a route are loaded.  To minimize the studdering, go into the Lookaround menu, which is Space with M on the BT or Read with M on the QT and turn off street changes, city changes, and state/province changes.

* When creating pedestrian routes the BrailleNote can run out of memory. It is recommended to create short routes.  On the PK limit your routes to 10 Miles, on the mPower limit your routes to 25 MI.

.

* When two quick turns occur, you may hear the “continue straight” message between those turns, particularly when traveling in a vehicle. That message is simply lagging behind your vehicular travel. By the time the Continue Straight message is spoken, it is time to make the second turn. Be prepared when given two quick turns in a row to overlook the Continue Straight message.

* If you are operating in a route and change to a new destination, strange things may happen to the original route. It is recommended that you calculate a new route if you establish a new destination.

* If you are unable to create a route, either your starting location or destination could be on a non-navigable Terminal Street. Try finding a street nearby which is non-terminal.  To do this, set your virtual position to a nearby street, then use the detail city command to check the road classification.

* There is no distance limit placed on route calculations. However, there is a limit of 10,000 route waypoints.

* Should the system ever stop responding or if you need to abort a long route calculation, press the reset button. Avoid excessive searches, like all points within 1000 miles.

** GPS

* If you use multiple receivers, it is recommended that you exit the GPS application before attempting to change receivers from the KeySoft connectivity menu.

* After conducting a 4 5 6 or J K L reset, you will need to repair and activate your Bluetooth receiver.  After you have successfully paired and activated your receiver, go into the GPS option menu and select set GPS receiver type. Select B for Bluetooth, then press enter to select the GPS receiver that you are using.

* It is recommended to exit GPS before turning off your Bluetooth receiver. If you turn off your receiver then try and exit, you will notice a delay before returning to the main menu.

* If you are using a serial GPS receiver with the mPower, it will be necessary to turn the receiver off before a reset on the mPower can be performed successfully.

** Virtual Mode

* If you turn on to a U-shaped street in Explore mode and press the forward command, you may hear that you have looped back to the intersection where you started. If you continue along the street that spans the mouth of the U, it is likely that the next intersection ahead will be the other end of the U. There is no way to travel on the U street itself unless it has intersecting segments.

** Miscellaneous

* If you are using KeySoft 7.5 with Unified English Braille (UEB) you will need to use ENTER with dots 2 3 5 6 to move forward waypoint by waypoint or intersection by intersection. You will get an error beep if you just press dots 2 3 5 6.

* If you are reading the Sendero documentation and task switch to another application you will be asked to exit GPS.  Sendero recommends exiting the documentation prior to switching to another task.

* If you are experiencing slow response to commands with older versions of KeySoft, switch the Look-Around mode off while performing these other functions. Look-Around scans for nearby POIs and intersections every 12 seconds, which takes some of the BrailleNote’s processing power. Besides turning Look-Around off, you can also make adjustments to look for POIs or Intersections only. See the manual for details.

* When using the GPS Replay feature, it is not recommended to have the replay file on a Micro drive.  This may add unnecessary wear and tear to the drive as it will access the disk once a second.

* When using Grade II, the “\” character will be spelled out as Backslash when a Map folder or POI folder name is displayed. Also, if you are entering a city name during an address match, be aware of single letter contractions. For example, if you only put the letter “c” it will be interpreted as the contracted word, “can.” Use a letter sign or additional letters to avoid this confusion.

Thank you for purchasing Sendero GPS and for reading this document. Happy travels.

For technical support, contact the dealer from whom you purchased your GPS. Direct US or Canada Sendero customers call, 1-888-757-6810.

Please read other documents for a complete understanding of the Sendero GPS software including the manual, the audio tutorial and the optional GPS CourseWare.